3GPP TSG-RAN WG4 Meeting #108-bis	R4-2315256

Xiamen, China, Oct 9 – Oct 13, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Simulation results UE Demodulation for non-colocated FR1 intra-band EN-DC/NR-CA

Agenda item:	5.11.4

Document for:	Discussion

Introduction

In RAN4#108 it was agreed to base requirements following the framework of previous CA requirements, i.e., the KPI will be based on SNR numbers [1].

Test setup and parameters Agreement: - Reuse the existing PDSCH CA demodulation requirements (e.g., TS 38.101-4 5.2A.2.1)

Additionally, we agreed on many simulation parameters, with only the MCS values and #Tx being left open:

Other parameter configurations Agreement: - Reusing test set-up from existing CA demodulation requirements for TRS and SSB configuration. - Configure 33us received time difference (RTD) between PCell and SCell. Type 2 UE NR-CA demodulation requirements. Received power difference; 25 dB. Received time difference; 33 µs. Channel model; [AWGN]. Maximum number of HARQ transmission; [1]. Signal/Noise setting; Set both Es and Noc to set SNR test points. Throughput measurement procedure; Measure both carriers. Tx antenna configuration and rank; Option1: 2Tx, rank2 for both carriers Option 2: 1Tx with rank1 for carrier with lower power 2Tx with rank2 for another carrier Option 3: rank 1 for both carriers with 1Tx. MCS table and MCS index; FFS. Test metric; 70% of the maximum throughput. MCS pair and SNR test point pair Way forward: Interested companies are encouraged to bring the simulation results in RANM4#108bis to decide the MCS index pair and corresponding SNR test point pair according to the agreed test setup and test metrics. Note the required SNR difference between the carriers with lower power and higher power should not exceed 25dB. Tx antenna and rank; MCS table; Test metric. Carrier with lower power; Option 1: 2Tx, Rank 2 Option 2: 1Tx, Rank 1; Option 1: MCS index 1 Option 2: MCS index 2; 70% of the maximum throughput. Carrier with higher power; Option 1: 2Tx, Rank 2 Option 2: 1Tx, Rank 1; Option 1: MCS index 1 Option 2: MCS index 2; 70% of the maximum throughput.

As such we present our simulation results on the above options in this paper, with the main goal of helping MCS pair selection.

Simulations

Setup

All other non-listed configuration are as usual for CA requirements:

|  | Type 2 UE NR-CA demodulation requirements |  |
| --- | --- | --- |
| Received power difference | 25 dB |  |
| Received time difference | 33 µs |  |
| Channel model | [AWGN] |  |
| Maximum number of HARQ transmission | [1] |  |
| Signal/Noise setting | Set both Es and Noc to set SNR test points |  |
| Throughput measurement procedure | Measure both carriers |  |
| Tx antenna configuration and rank | Option1: 2Tx, rank2 for both carriers Option 2: 1Tx with rank1 for carrier with lower power 2Tx with rank2 for another carrier Option 3: rank 1 for both carriers with 1Tx |  |
| MCS table and MCS index | FFS |  |
| Test metric | 70% of the maximum throughput |  |
| Tx antenna and rank | Carrier with lower power | Option 1: 2Tx, Rank 2 Option 2: 1Tx, Rank 1 |
|  | Carrier with higher power | Option 1: 2Tx, Rank 2 Option 2: 1Tx, Rank 1 |

Table 1: Simulation setup

Results

AWGN 1Tx/1Layer

We have checked 3 MCS pairs (higher MCS for high power cell, lower MCS for lower power cell).

The SNR values to achieve 70% TPUT are as follows:

| Pair 1 | Table 1 – MCS 2 | Table 2 – MCS 24 |
| --- | --- | --- |
| Pair 1: 70%TPUT | -5.1 dB | 19.6 dB |
| Pair 2 | Table 2 – MCS 2 | Table 2 – MCS 26 |
| Pair 2: 70%TPUT | -3.2 dB | 21.9 dB |
| Pair 3 | Table 3 – MCS 4 | table 2 – MCS 20 |
| Pair 3: 70%TPUT | -9.6 dB | 16 dB |

Table 1: 70%TPUT SNR values AWGN 1Tx/1layer for 3MCS pairs

The full curves are as follows:

Figure 1: Relative TPUT curves AWGN 1TX/1layer for 3 MCS pairs

Conclusion

In this contribution we have shown our initial simulation results for NonCol CA based requirements, and in particular the possible MCS choices.

We will discuss the results in our companion discussion document.

- References

- R4-2313876 “WF for NonCol_intraB_ENDC_NR_CA_Demod”, Ericsson, RAN4#108-e, Toulouse.