3GPP TSG-RAN4 Meeting #108-bis		R4-2315287

Xiamen, China, Oct 9 – Oct 13, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev |  | Current version: | 17.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to Rel-17 TS 38.133 on monitoring of paging occasions for CG-SDT with HD-FDD Redcap UEs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap-Core |  |  |  |  |  | Date: |  |  | 2023-09-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Provide correction on the UE behaviour when paging occasion overlaps with CG-SDT for RedCap UE in HD-FFD mode based on the received LS from RAN2 (R2-2304562). Also, according to RAN4#108 agreement: RAN4 will further update requirements for the case of partial collisions of POs with CG-SDT occasions for HD-FDD RedCap UE within the SI modification period based on RAN2 LS. There are no existing RRM requirements for the case when all available POs are colliding with CG-SDT occasions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The changes are: Change 1: - Clarifying that for the case of partial collisions of POs with CG-SDT occasions within the SI modification period it is up to UE implementation whether to monitor paging or perform CG-SDT transmission. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will be misalignment between RAN2 understanding and RAN4. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1B.2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------START OF CHANGE 1----------------------------

#### 5.1B.2.6	Maximum interruption in paging reception

The requirements in clause 4.2B.2.6 shall apply for RedCap UEs.

For RedCap UE in HD-FDD mode, if some of paging occasions overlap with CG-SDT occasions within the SI modification period then it is up to UE implementation whether to monitor the paging during the paging occasions or perform CG-SDT transmissions.

----------------------END OF CHANGES 1----------------------------