3GPP TSG-RAN WG4 Meeting #108bis	R4-2315300
Xiamen, China, 9 – 13 Oct, 2023

Title: 	n109 REFSENS requirements to define the safe region

Source: 	Huawei, HiSilicon

Agenda Item:	4.32.3

Document for:	Approval

# 1	Introduction

The WF of the last meeting [1], has the following issue as an open point:

Issue 2-1-1: REFSENS Agreement: The new operating band n109 REFSENS requirements are specified as in the table below. Operating band / SCS / Channel bandwidth / REFSENS. Operating band; SCS kHz; Channel bandwidth (MHz); REFSENS (dBm)8; Duplex Mode. n109; 15; 5,10,15,20,25,30,40,50; -100 + 10log10(NRB/25) X; FDD. 30; 10,15,20,25,30,40,50; -97.1 + 10log10(NRB/24) X. NOTE X: Applies for DL channels for which channels edges are > A MHz away from 2xFUL at 15 kHz SCS and > B MHz away from 2xFUL at 30 kHz SCS. In case of UL second harmonic direct hit, the value is modified to -71.9 dBm for all channel bandwidths.. Way forward: Further check to be done regarding the values A and B in NOTE X on the above table whether we need more offset than 10 MHz for 15k SCS and 20 MHz for 30k SCS.

In this brief document, the values of A and B will be discussed

# 2	n109 Rx Safe Region

Based on another agreement on the last WF about REFSENS UL configuration, a CBW of 5MHz is considered for MSD evaluations for 15 KHz SCS. Figure 1 depicts the worst case scenario where a 5MHz signal is placed at the upper limit of the n109 UL. Its 2nd harmonic occupies 10 MHz ([1456, 1466]). On the other hand, the out of band leakage of the 2nd harmonic (in light blue) will be reduced by 30 dB due to ACLR1 requirements in PC3 UEs. Hence if A is larger than BW of the ACLR1 region for the 2nd harmonic, then region beyond A can be considered as a safe region.

Figure 1 n109 UL 2nd harmonic direct hit region

In a similar analysis, for the case of 30 KHz SCS, the CBW of the UL equals to 10 MHz and its 2nd harmonic would be over 20 MHz, hence B has to be larger than the ACLR region of the 2nd harmonic, which means B=20MHz

Proposal 1: A=10 MHz, B=20MHz to keep the 2nd harmonic adjacent channel leakages out of the safe region of n109 Rx.

# 3	Conclusion

Proposal 1: A=10 MHz, B=20MHz to keep the 2nd harmonic adjacent channel leakages out of the safe region of n109 Rx.

# 4	Reference

[1]	“R4-2314927, WF on the requirements for n109, Vodafone.”