3GPP TSG-RAN WG4 Meeting#108-bis	R4-2315350

Xiamen, China 9th Oct – 13th Oct, 2023

Source: 	Qualcomm Incorporated

Title: 	Views on PDSCH Performance Requirements for FR2 Multi-Rx

Agenda item:	5.7.4.2

Document for:	Discussion

# 1. Introduction

After the conclusion of RAN4#108, there were a few open issues that need to be further discussed [1]. In this paper, we share our views on these remaining open issues for PDSCH demodulation requirements.

2. Demodulation Requirements for PDSCH

- Proposals

  - Option 1: One PTRS port per TRP for sDCI schemes

  - Option 2: One PTRS port for sDCI scheme

PTRS Port for sDCI Schemes

Due to correlated nature of channels in FR2 multi-Rx scenarios and depending on the cross-talk level, having one PTRS port may prove to be difficult for common phase estimation for the TRP that is not configured with PTRS and can severely degrade performance of the sDCI scheme. Therefore, we support defining requirements with one PTRS port per TRP configuration.

Proposal 1: One PTRS port per TRP for sDCI schemes (Option 1).

- Proposals

  - Option 1: Only use TDLA30-75 for 100 MHz/120 kHz

  - Option 2: Keep TDLA30-300 as FFS.

Channel model

In our view, 300Hz Doppler may not be practical for FR2 multi-Rx application. Therefore, our preference is to define requirements with TDLA30-75 only.

Proposal 2: Only use TDLA30-75 for 100 MHz/120 kHz (option 1).

3. Conclusions

In this paper, we share our views on the remaining open issues for PDSCH demodulation requirements under the context of FR2 multi-Rx. The following have been proposed.

Proposal 1: One PTRS port per TRP for sDCI schemes (Option 1).

Proposal 2: Only use TDLA30-75 for 100 MHz/120 kHz (option 1).

4. References

[1] R4-2313977, “WF on UE demodulation and CSI requirements for FR2 multi-Rx DL reception,” 3GPP TSG RAN WG4 Meeting #108, Toulouse, FR, Aug 21 – 25, 2023.