3GPP RAN WG4 Meeting #108bis		R4-2315381

Xiamen, China, October 9-13, 2023

Agenda item:	5.15.2.1

Source:	Apple

Title:	TP to TR38.870 on MIMO radiated output power metric

Work Item:	NR_FR1_TRP_TRS_enh-Core

Document for:	Approval

# 1	Introduction

The Rel-18 work item on the enhancement of TRP/TRS methodologies and requirements for FR1 includes the following objectives related to TRP of 2Tx UEs [1]:

- Enhancements of TRP TRS test methodology - Specify necessary enhancement of the anechoic-chamber based test methodology (i.e. reference test methodology) to support (test methodology defined in TR 38.834 is the baseline): - UE with NR 2Tx configuration - Case 1: TxD (i.e., TxD capability supported) - Case 2: single layer UL-MIMO (i.e., codebook-based capability supported) - Study proper configuration from UE implementation and test system feasibility perspective - Define test case applicability for case 1 and case 2

RAN4 has devoted a considerable amount of time to discuss the radiated output power test methodology for UL MIMO capable devices.  RAN4 #105 made preliminary agreements in [2].  During RAN4 #106 the contributions in [3-7] were discussed and resulted in the agreement in [8].  During the RAN4 #106bis meeting contributions [9-14] were discussed, with the agreement captured in the WF [15].  During the RAN4 #107 meeting contributions [16 - 24] were discussed, with the agreement in [25].  During the RAN4 #108 a text proposal with the metric definition was discussed (but not approved) [26], with an overall compromise on the handling of TRP for UL MIMO captured in the approved WF [27].  Based on the discussion so far, this contribution provides a text proposal to TR38.870 to capture the definition of the radiated output power metric for UL MIMO.

# 2	Text proposal

<< start of change >>

# 5	Performance metrics

## 5.1	Definition of the Total Radiated Power (TRP)

### 5.1.1	Definition of the Total Radiated Power (TRP) for AC

Transmitter power measurements shall be performed using the Total Radiated Power (TRP) as the measurement metric. This clause defines the definition of TRP value of NR FR1 DUT for Anechoic Chamber (AC) method.

The TRP with Anechoic Chamber method is defined as:

TRP= 14πθ=0πϕ=02πEIRPθθ,ϕ+EIRPϕθ,ϕsinθdϕdθ		(5.1)

Where the effective isotropic radiated power (EIRP) is defined as

EIRPθ,ϕ=PTGTθ,ϕ              	  (5.2)

Where PTGT is the product of the power delivered to the antenna and the antenna’s power gain.

Where EIRPθ and EIRPϕ are the EIRP in the corresponding θ and ϕ polarizations.

The summation form based on the sin weights of TRP with Anechoic Chamber method is defined as:

TRP≈ π2NMn=0N-1m=0M-1EIRPθθn,ϕm+EIRPϕθn,ϕmsinθn      	      (5.3)

Where N and M are the number of sampling intervals for θ and ϕ. θn and ϕm are the measurement angles.

The summation form based on the Clenshaw-Curtis quadrature integral approximation of TRP with Anechoic Chamber method is defined as:

TRP≈ 12Mn=0Nm=0M-1EIRPθθn,ϕm+EIRPϕθn,ϕmWθn            	(5.4)

Where the value of W(θn) can be calculated as follows:

W(θn)=ciN1-j=1int(N2)bj4j2-1cos⁡(2jθn)	(5.5)

with

bj=    1,  2j=N       2,  otherwise

and

ci=   1,  i=0 or N   2,  otherwise

The applicability of TRP quadratures, frequency ranges, and measurement grids is tabulated in Table 5.1.1-1.

Table 5.1.1-1: Applicability for TRP measurement grids

| Frequency Range | Quadrature | [°] | N | M | Min. Number of Grid Points |
| --- | --- | --- | --- | --- | --- |
| < 3GHz | sin() | 15 | 12 | 24 | 266 |
|  |  | 30 | 6 | 12 | 62 |
|  | Clenshaw-Curtis | 15 | 12 | 24 | 266 |
|  |  | 30 | 6 | 12 | 62 |
| > 3GHz | sin() | 15 | 12 | 24 | 266 |
|  |  | 30 | 6 | 12 | 62 |
|  | Clenshaw-Curtis | 15 | 12 | 24 | 266 |
|  |  | 30 | 6 | 12 | 62 |

#### 5.1.1.2	TRP for UL MIMO devices

##### 5.1.1.2.1	Non-coherent UL MIMO

For devices capable of non-coherent UL MIMO, the definition of TRP used to verify the Rel-18 requirement is enhanced, based on the definition provided in 5.1.1, as follows.

The total radiated power for UL MIMO devices is defined as:

TRP= 14πθ=0πϕ=02πEIRPθ,TPMI2θ,ϕ+EIRPϕ,TPMI2θ,ϕsinθdϕdθ		(5.x1)

Where the effective isotropic radiated power (EIRP) is measured by configuring the UE with the transmit precoding matrix indecex (TPMI) set to 2:

EIRPTPMI2θ,ϕ=PTGT,TPMI2θ,ϕ              	  (5.x2)

By substituting equation 5.1 in Clause 5.1.1 for 5.x1 in this clause, all of the remaining definitions in Clause 5.1.1 apply for UL MIMO TRP.

As an alternative metric, which is captured in this Technical Report for informational purposes, the definition of TRP provided in 5.1.1 is enhanced as follows.

The total radiated power for UL MIMO devices is defined as:

TRP= 14πθ=0πϕ=02πEIRPθ,maxθ,ϕ+EIRPϕ,maxθ,ϕsinθdϕdθ		(5.x3)

Where the effective isotropic radiated power (EIRP) is defined as the outcome of selecting the best EIRP from a sweep of all EIRP measured by configuring the UE with the transmit precoding matrix indeces (TPMI) which are applicable according to the UE reported UL MIMO capability:

EIRPmaxθ,ϕ=maxPTGT,TPMI_indθ,ϕ              	  (5.x4)

By substituting equation 5.1 in Clause 5.1.1 for 5.x3 in this clause, all of the remaining definitions in Clause 5.1.1 apply for UL MIMO TRP.

##### 5.1.1.2.1	Coherent UL MIMO

For devices capable of coherent UL MIMO, the definition of TRP provided in 5.1.1 is enhanced as follows.

The total radiated power for UL MIMO devices is defined as:

TRP= 14πθ=0πϕ=02πEIRPθ,maxθ,ϕ+EIRPϕ,maxθ,ϕsinθdϕdθ		(5.x5)

Where the effective isotropic radiated power (EIRP) is defined as the outcome of selecting the best EIRP from a sweep of all EIRP measured by configuring the UE with the transmit precoding matrix indeces (TPMI) which are applicable according to the UE reported UL MIMO capability:

EIRPmaxθ,ϕ=maxPTGT,TPMI_indθ,ϕ              	  (5.x6)

By substituting equation 5.1 in Clause 5.1.1 for 5.x5 in this clause, all of the remaining definitions in Clause 5.1.1 apply for UL MIMO TRP.

### 5.1.2	Definition of the Total Radiated Power (TRP) for RC method

TRP in the reverberation chamber is a measurement of transmitter performance in an isotropic Rayleigh fading environment that is based on sampling the radiated power of the UE/MS for a discrete number of field combinations. The average value of these statistically distributed samples is proportional to the TRP and by calibrating the average power transfer function, an absolute value of the TRP can be obtained.  The TRP with Reverberation Chamber method is defined as:

TRP=  PavgC1-R Pref

Where Pref is the reference power transfer function for the fixed measurement antenna, R is the reflection coefficient for the fixed measurement antenna and C is the path loss in the cables connecting the measurement receiver to fixed measurement antenna. These parameters are calculated from the calibration measurement and are further discussed in calibration section. Pavg is the average power measured by the fixed measurement antenna and can be calculated using the following expression:

Pavg=  m=1MS21,.m2M

Where S21,m is sample number m of the complex transfer function measured with the fixed measurement antenna and M is the total number of samples measured.

## 5.2	Definition of Total Radiated Sensitivity (TRS)

### 5.2.1	Definition of the Total Radiated Sensitivity (TRS) for AC

Receiver sensitivity measurements shall be performed using data throughput as the measurement metric. The DUT’s receiver sensitivity corresponds to the minimum downlink signal power required to provide a data throughput rate greater than or equal to 95% of the maximum throughput of the reference measurement channel (RMC).

This definition will be used to calculate the Total Radiated Sensitivity (TRS) value of NR FR1 DUT.

The TRS with Anechoic Chamber method is defined as:

TRS=4πθ=0πϕ=02π1EISθθ,ϕ+1EISϕθ,ϕsinθdϕdθ     	   (5.6)

Where the effective isotropic sensitivity (EIS) is defined as the minimum power level at which the throughput exceeds or equal to 95% of the maximum throughput of the specified RMC, at each sampling point.

Where EISθ and EISϕ are the EIS in the corresponding θ and ϕ polarizations.

The summation form based on the sin weights of TRS with Anechoic Chamber method defined as:

TRS≈2NMπn=0N-1m=0M-11EISθθn,ϕm+1EISϕθn,ϕmsinθn                 	 (5.7)

Where N and M are the number of sampling intervals for θ and ϕ. θn and ϕm are the measurement angles.

The summation form based on the Clenshaw-Curtis quadrature integral approximation of TRS with Anechoic Chamber method is defined as:

TRS≈2Mn=0Nm=0M-11EISθθn,ϕm+1EISϕθn,ϕmWθn                  	(5.8)

Where the value of W(θn) follows Equation 5.5.

The applicability of TRP quadratures, frequency ranges, and measurement grids is tabulated in Table 5.2.1-1.

Table 5.2.1-1: Applicability for TRS measurement grids

| Frequency Range | Quadrature | [°] | N | M | Min. Number of Grid Points |
| --- | --- | --- | --- | --- | --- |
| < 3GHz | sin() | 30 | 6 | 12 | 62 |
|  | Clenshaw-Curtis | 30 | 6 | 12 | 62 |
|  |  | 45 | 4 | 8 | 26 |
| > 3GHz | sin() | 30 | 6 | 12 | 62 |
|  | Clenshaw-Curtis | 30 | 6 | 12 | 62 |
|  |  | 45 | 4 | 8 | 26 (Note 1) |
| Note 1: When the back pole at = 180° cannot be measured due to obstruction and/or blocking, extrapolation is used to estimate EIS at = 180° for measurement grids with =45° by either a) using at least two points within 15° of the pole or b) averaging the last cut (i.e. = 135°) |  |  |  |  |  |

### 5.2.2	Definition of the Total Radiated Sensitivity (TRS) for RC method

The calculation of TRS is based on searching for the lowest power received by the UE for a discrete number of field combinations in the chamber. The downlink power received by the UE at each discrete field combination that provides a BER (or BLER) which is better than the specified target BER/BLER level shall be averaged with other such measurements using different field combinations. By calibrating the average power transfer function, an absolute value of the TRS can be obtained when the linear values of all downlink power levels have been averaged.

The TRS with Reverberation Chamber method is defined as:

TRS=(Pref 1-R C)1M m=1M1PBSSm-1

Where Pref is the reference power transfer function for the fixed measurement antenna, R is the reflection coefficient for the fixed measurement antenna and C is the path loss in the cables connecting the measurement receiver to fixed measurement antenna. The parameters are calculated from the calibration measurement and are further discussed in calibration section. PBSSm is the output power from the base station emulator when it is adjusted to give the specified digital error rate or throughput from the DUT for mode-stirring sample m.

<< end of change >>

# References

RP-223112, “Enhancement of UE TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements and test methodologies for FR1 (NR SA and EN-DC),” vivo, 3GPP RAN #98, December 2022

R4-2220610, “WF for Rel-18 TRP TRS WI,” vivo, 3GPP RAN4 #105, November 2022

R4-2300349, “Test methodology for radiated power of UL MIMO capable devices,” Apple, 3GPP RAN4 #106, Februarry 2023

R4-2300141, “Proposals for OTA tests under UL MIMO and CA,” Huawei, 3GPP RAN4 #106, Februarry 2023

R4-2300996, “Discussion on 2Tx test method in Rel-18 TRP TRS WI,” Samsung, 3GPP RAN4 #106, February 2023

R4-2301459, “Consideration on proper TPMI for UL-MIMO TRP test,” OPPO, 3GPP RAN4 #106, February 2023

R4-2302754, “Testability aspects for 2Tx TRP,” Rohde & Schwarz, 3GPP RAN4 #106, February 2023

R4-2302917, “WF for Rel-18 TRP/TRS requirements,” vivo, 3GPP RAN4 #106, February 2023

R4-2304343, “On TRP TRS test methodology,” Apple, 3GPP RAN4 #106bis, April 2023

R4-2305701, “Further discussion on FR1 1 Layer UL-MIMO 2Tx SISO OTA TRP,” MediaTek, vivo, CAICT, 3GPP RAN4 #106bis, April 2023

R4-2304028, “on TRP TRS test aspects,” Huawei, 3GPP RAN4 #106bis, April 2023

R4-2304828, “Discussion on TRP concept and test method for multi-antenna UE,” Samsung, 3GPP RAN4 #106bis, April 2023

R4-2305607, “Discussion on enhancement of test method for TRP and TRS,” Qualcomm, 3GPP RAN4 #106bis, April 2023

R4-2305610, “Discussion on concept and definition of TRP,” OPPO, 3GPP RAN4 #106bis, April 2023

R4-2305904, “WF for Rel-18 TRP/TRS requirements,” vivo, 3GPP RAN4 #106bis, April 2023

R4-2309055, “UL MIMO radiated output power metric and test methodology,” Apple, 3GPP RAN4 #107, May 2023

R4-2307245	“on TRP TRS tests under TxD, UL MIMO and CA, Huawei, HiSilicon,” 3GPP RAN4 #107, May 2023

R4-2307936 “Discussion on TRP concept and test method for UL MIMO and TxD,” Samsung, 3GPP RAN4 #107, May 2023

R4-2308252	“Discussions on AC test method and NR bandwidth,” vivo, 3GPP RAN4 #107, May 2023

R4-2308366	“2Tx SISO OTA TRP and antenna correlation, MediaTek Inc,” 3GPP RAN4 #107, May 2023

R4-2308977	“Discussion on TPMI configuration for one-layer UL MIMO TRP test,” OPPO, 3GPP RAN4 #107, May 2023

R4-2309013	“Futher discussion on 2Tx configuration,” Xiaomi, 3GPP RAN4 #107, May 2023

R4-2309243	“Views on FR1 TRP and TRS test method enhancement,” Qualcomm Incorporated, 3GPP RAN4 #107, May 2023

R4-2309354	“Discussion on 2TX test methodology,” CAICT, 3GPP RAN4 #107, May 2023

R4-2309815, “WF for Rel-18 TRP/TRS WI,” vivo, 3GPP RAN4 #107, May 2023

R4-2311230, “TP to TR38.870 on MIMO radiated output power metric,” Apple, 3GPP RAN4 #108, August 2023

R4-2313891, “WF for Rel-18 FR1 TRP TRS,” vivo, 3GPP RAN4 #108, August 2023