3GPP RAN WG4 Meeting #108bis		R4-2315389

Xiamen, China, October 9th – 13th, 2023

Agenda item:	4.30.1.5

Source:	Apple

Title:	MSD analysis for UL CA_n26(2A)

WI/SI:	NR_700800900_combo_enh-Core

Release:	Rel-18

Document for:	Approval

# 1	Introduction

CA_n26(2A) with UL configuration same as in DL is the latest addition to the “enhancement on 700/800/900 band combinations” work item which was approved in RAN #101 meeting [1]. The combination is expected to be subject to odd order UL intermodulation (IMD3, IMD5, IMD7, ...) self-interference to both DL carriers where MSD from IMD3 and IMD5 direct hit could potentially be relatively high. In this contribution, we provide our MSD analysis for IMD3 and IMD5 direct hit to DL carriers based on numerical simulations and link analysis.

# Discussion

Our MSD analysis is carried out for the carrier configuration where UL IMD3 and IMD5 have a direct hit on DL PCC and SCC or vice versa which will always take place when the carrier spacing is half of the duplex distance at 22.5 MHz as exemplified in Figure 2-1.

Figure 2-1 UL/DL CA_n26(2A) example configuration with IMD3 and IMD5 direct hit to DL carriers

Observation 1: UL/DL CA_n26(2A) has 2UL IMD3 and IMD5 direct hit to DL PCC and SCC or vice versa when the carrier spacing is half of the duplex distance at 22.5 MHz.

The Tx interference noise power to DL PCC and SCC frequency ranges at the PA output was evaluated based on numerical simulations where the simulation assumptions are summarized in Table 2-1.

| Parameter | Value | Comment |
| --- | --- | --- |
| Tx and PA model | Same as used for MPR/A-MPR simulations | CIM3, CIM5, IRR, etc. |
| Total Tx power | 23 dBm | At antenna port |
| FE insertion loss | 4 dB |  |
| Channel BW | PCC UL/DL: 5/5; SCC UL/DL: 5/5 | MHz |

Table 2-1 Simulation assumptions for CA_n26(2A) Tx interference noise power to DL PCC and SCC

Figure 2-2 shows the simulated spectral profile at n26 PA output (single PA) where the ranges highlighted in light green are the DL carrier allocations. The IMD3 and IMD5 direct hit to DL carriers can be observed via the first and second noise humps next to the right UL carrier. Notice that the Tx noise into DL range is not only contributed by the 2UL intermodulation products but also the nonlinear self-distortion (ACLR) from each UL carrier respectively.

Observation 2: The UL CA_n26(2A) Tx noise into DL range is not only contributed by the 2UL intermodulation products but also the nonlinear self-distortion (ACLR) from each UL carrier respectively.

The simulated Tx noise power in DL PCC and SCC ranges was then incorporated in link analysis for MSD calculations where further assumed parameters are summarized in Table 2-2.

| Parameter | Value | Comment |
| --- | --- | --- |
| FE insertion loss | 4 dB |  |
| Duplexer isolation | 50 dB | Tx to Rx |
|  | 42 dB | Tx to ANT |
| Antenna isolation | 10 dB | Main to diversity |
| MRC |  | Uncorrelated |

Table 2-2 Assumed parameters for CA_n26(2A) MSD link analysis

In the MSD link analysis, apart from the Tx noise generated from PA nonlinearity, other RF impairments such as passive components IMD and LNA IMD were also considered which however have much less noise contribution than PA. In the case of 2Tx implementation with one Tx for each UL carrier, the MSD would be dominated by the PA reversed IMD.

Observation 3: Among all the RF impairments contributing to MSD, Tx noise generated from PA nonlinearity dominates.

Observation 4: In the case of 2Tx implementation with one Tx for each UL carrier, the MSD would be dominated by the PA reversed IMD.

Table 2-3 summarizes the calculated MSD results for the CA configuration as shown in Figure 2-1.

| UE Architecture | PCC MSD (IMD3) | SCC MSD (IMD5) |
| --- | --- | --- |
| 1Tx | 47.5 dB | 30.5 dB |
| 2Tx | 46.5 dB | 11.8 dB |

Table 2-3 Calculated MSD results for CA_n26(2A) configuration as shown in Figure 2-1

It can be seen that the MSD caused by IMD3 is quite substantial which may render the combination not being so useful. On the other hand, though the 2Tx architecture can help improve MSD due to IMD5, it does not benefit much to MSD due to IMD3.

Observation 5: For UL/DL CA_n26(2A), the MSD caused by IMD3 is quite substantial which may render the combination not being so useful.

Observation 6: The 2Tx architecture for UL CA_n26(2A) does not benefit much to MSD due to IMD3, though it helps improve MSD due to IMD5.

Based on the above analysis, we propose RAN4 to reconsider whether there is sufficient technical justification to support UL CA_n26(2A) as the DL carriers are susceptible to relatively high MSD.

Proposal 1: RAN4 to reconsider whether there is sufficient technical justification to support UL CA_n26(2A) as the DL carriers are susceptible to relatively high MSD.

Proposal 2: If RAN4 would proceed to specify UL/DL CA_n26(2A), take the MSD values in Table 2-3 into consideration.

# 3	Conclusion

In this contribution, we provide our MSD analysis for UL/DL CA_n26(2A) with IMD3 and IMD5 direct hit to DL carriers based on numerical simulations and link analysis.

Observation 1: UL/DL CA_n26(2A) has 2UL IMD3 and IMD5 direct hit to DL PCC and SCC or vice versa when the carrier spacing is half of the duplex distance at 22.5 MHz.

Observation 2: The UL CA_n26(2A) Tx noise into DL range is not only contributed by the 2UL intermodulation products but also the nonlinear self-distortion (ACLR) from each UL carrier respectively.

Observation 3: Among all the RF impairments contributing to MSD, Tx noise generated from PA nonlinearity dominates.

Observation 4: In the case of 2Tx implementation with one Tx for each UL carrier, the MSD would be dominated by the PA reversed IMD.

Observation 5: For UL/DL CA_n26(2A), the MSD caused by IMD3 is quite substantial which may render the combination not being so useful.

Observation 6: The 2Tx architecture for UL CA_n26(2A) does not benefit much to MSD due to IMD3, though it helps improve MSD due to IMD5.

Proposal 1: RAN4 to reconsider whether there is sufficient technical justification to support UL CA_n26(2A) as the DL carriers are susceptible to relatively high MSD.

Proposal 2: If RAN4 would proceed to specify UL/DL CA_n26(2A), take the MSD values in the table below into consideration.

| CA Combination | UE Architecture | PCC MSD (IMD3) | SCC MSD (IMD5) |
| --- | --- | --- | --- |
| UL/DL CA_n26(2A) | 1Tx | 47.5 dB | 30.5 dB |
|  | 2Tx | 46.5 dB | 11.8 dB |

# 4	References

RP-232680 “Revised WID on enhancement for 700/800/900MHz band combinations”, CATT, 3GPP TSG-RAN Meeting #101, Bengaluru, India, September 11th – 15th, 2023