3GPP TSG-RAN WG4 Meeting # 108bis				                                                                                 R4-2315400

Xiamen, China, October 09 – October 13, 2023

Title: 	Discussion on Carrier Phase Positioning

Source: 	Xiaomi

Agenda item:	5.22.2.6

Document for:	Discussion

Introduction

In RAN#98-e, the WID “New WID on Expanded and Improved NR Positioning” [1] was approved.

One of objectives for the core part in RAN4 is to specify requirements for the carrier phase positioning measurements:

- Specify physical layer measurements and signalling to support NR DL and UL carrier phase positioning for UE-based. UE-assisted positioning, and NG-RAN node assisted [RAN1, RAN2, RAN3, RAN4]. - Existing DL PRS and UL SRS for positioning are used for NR carrier phase measurements. - Specify measurements that are limited to a single carrier/PFL. - Specify corresponding new core requirements, as well as identifying and specifying the impact on the existing RAN4 specification, including RRM measurements and procedures [RAN4].

And in the previous RAN4 meetings the high-level general aspects of this WI were agreed in [2].  In this contribution we will provide further considerations on RRM requirements for Rel18 carrier phase positioning on latest RAN1’s agreements [3].

NR DL carrier phase measurement

Core requirements

In the last RAN4 meeting the general principle to define CPP requirements was agreed as below.

Agreements: - RSCP/RSCPD is reported together with other positioning measurement(s) and the same measurement period requirement (i.e., Rx-Tx time difference/RSTD measurements) should apply. - FFS: whether the existing Rx-Tx time difference/RSTD measurement periods apply

Hereby we can continue to discuss the open issues left. For an example, whether the existing PRS measurement can be reused for RSCP/RSCPD.

In the last RAN1 meeting, the following agreements achieved.

R1#104 Agreement Each DL RSCP/RSCPD measurement instance is obtained with Nsample=1 sample only.

From RAN4 RRM perspective, since the existing requirements of Rx-Tx time difference/RSTD measurement periods  in TS38.133[] includes multiple samples and single sample, we can propose that:

Proposal 1: The requirements on DL RSCP/RSCPD (including measurement reporting delay and accuracy) shall be defined with single sample only.

Meanwhile, RAN1 also agreed that:

R1#114 Agreement When DL RSCPD/RSCP measurements are reported together with the DL RSTD/ UE Rx – Tx time difference measurements, the DL RSCPD/RSCP measurements are obtained from a single DL PFL only. Note: From RAN1’s perspective, the reporting of the carrier phase measurements from one DL PFL has no impact on the reporting of the DL RSTD and/or UE Rx – Tx time difference measurements from the same DL PFL or other DL PFLs.

In current in current 38.133, the requirements for other PRS measurement is for the multiple PFLs. e.g.

TRSTD,Total=i=1LTRSTD,i+ L-1*maxTeffect,i Where , i is the index of positioning frequency layer

Observation 1: The current requirement defined in TS38.133 already includes these when the total measurement positioning frequency layer number is 1 and >1.

Therefore, we can propose that:

Proposal 2:  RSCP/RSCDP measurement reporting period requirement can be defined as:

TRSTD,Total=i=1LTRSTD,i+ L-1*maxTeffect,i Where , i is the index of positioning frequency layer, L is total number of positioning frequency layers, and for RSCP/RSCDP requirements L is equal to 1.

Accuracy requirements

Additionally, to mitigate the multipath impacts, the multipath indication was also agreed in the last RAN1 meeting[3].

R1#112 Agreement[3] Rel-17 LOS/NLOS indication (when indicated) applies for the carrier phase measurement(s) in the same report.

Observation 2: In Rel17, there is obvious performance degradation in NLOS channel in comparison with these in LOS.

Thus, we can suggest that:

Proposal 3:  RAN4 can FFS on the different the accuracy requirements for the carrier phase measurements under the different LOS/NLOS channel conditions.

NR UL carrier phase measurement

In the previous RAN1 meeting, it was agreed to introduce NR UL carrier phase positioning measurement [3]:

R1#112 Agreement NR UL reference signal carrier phase (RSCP) (of i-th path) is defined as the phase of the channel response at the i-th path delay derived from the resource elements (REs) that carry the UL SRS signal for positioning purpose configured for the measurement. A UL RSCP is associated with a specific RF frequency. - FFS: the reference point of the UL RSCP - FFS: whether/how the measurement timing is defined - Note: the i-th path is used for the sake of definition, whether only the first path or additional paths will be supported is subject to further discussion - Note: The support of MIMO SRS for positioning is transparent to UE

For the NG-RAN node assisted NR carrier phase positioning, at least RAN4 needs define the performance requirements for gNB also. Thus, we can propose that:

Proposal 4:  The gNB accuracy measurement requirements of UL RSCP NG-RAN node assisted positioning shall be defined.

CPP Measurement report mapping

One of the fundamental issues for RAN4 is to define the measurement reporting mapping including the reporting granularity and range. And in the last meeting the following agreements were captured in [2, R4-2318].

- Proposals - Option 1: fixed granularity (CATT, Huawei) - Alt 1: 0.1 degree

However, in the last RAN1 meeting, the following agreements were also achieved.

Agreement Support UE/TRP to report the phase quality indication for the RSCP/RSCPD measurements. The phase quality indication includes the following fields: - phase quality index - phase quality resolution The values of the phase quality index and phase quality resolution are left for RAN4.

According to RAN1’s agreement, the phase quality will be indicated to NW. That is the best quality of measurement can be variable for the different UE/TRP. From the reporting resolution perspective, it can be also dynamically configured. This is a little contradicted with last RAN4’s agreements above.

From CPP positioning measurement itself, in order to support different using scenarios the dynamic range and resolution of timing measurements may have large difference for different application scenarios. Therefore, the reporting granularity for CPP could be configurable also.

Observation 3: If the phase quality was indicated to NW, there is beneficial to report the measurement results with different resolution based on the best quality UE can achieved. For an example, the total reporting overhead can be reduced without any additional cost from UE.

Proposal 5: The reporting graduality of RSCP/RSCPD can be configurable depending on UE’s phase quality.

Conclusion

This contribution provided further RRM impacts on the following aspects of Rel18 WI of “Expanded and Improved NR Positioning” given the current physical layer design.

NR DL carrier phase measurement

Proposal 1: The requirements on DL RSCP/RSCPD (including measurement reporting delay and accuracy) shall be defined with single sample only.

Observation 1: The current requirements defined in TS38.133 already include these when the total measurement positioning frequency layer number is 1 and >1.

Proposal 2:  RSCP/RSCDP measurement reporting period requirement can be defined as:

TRSTD,Total=i=1LTRSTD,i+ L-1*maxTeffect,i Where , i is the index of positioning frequency layer, L is total number of positioning frequency layers, and for RSCP/RSCDP requirements L is equal to 1.

Observation 2: In Rel17, there is obvious performance degradation in NLOS channel in comparison with these in LOS.

Proposal 3:  RAN4 can FFS on the different the accuracy requirements for the carrier phase measurements under the different LOS/NLOS channel conditions.

NR UL carrier phase measurement

Proposal 4:  The gNB accuracy measurement requirements of UL RSCP NG-RAN node assisted positioning shall be defined.

CPP measurement report mapping

Observation 3: If the phase quality was indicated to NW, there is beneficial to report the measurement results with different resolution based on the best quality UE can achieved. For an example, the total reporting overhead can be reduced without any additional cost from UE.

Proposal 5: The reporting graduality of RSCP/RSCPD can be configurable depending on UE’s phase quality.

References

- RP-223532, New WID on Expanded and Improved NR Positioning, Intel, CATT, Ericsson

- R4-2310163

- R1-2308328

- TS38.133 v17.8.0