3GPP TSG-RAN WG4 Meeting #108bis                              R4-2315442

xiamen, China, October 09 – October 13, 2023

Source: 	Murata Manufacturing Co Ltd.

Title: 	n8 DL 30MHz REFSENS and RSD

Agenda Item:	4.26.2.1 Adding new channel bandwidth(s) support to existing NR bands [NR_bands_R18_BWs-Core]

Document for:	Approval

Introduction

In the RAN meeting #101, n8 DL 30MHz was added to Basket WID [1]. In this paper, we proposed the REFSENS for PC3 and the RSD for PC2 for n8 DL 30MHz.

REFSENS for PC3 and RSD for PC2

FE architecture

The two kinds of FE architecture shown in the Figure 1 are used to analyse REFSENS and RSD. For 2Tx/2Rx architecture, the mutual influence between primary Tx and diversity Tx was considered.

Figure 1. FE architecture

Assumptions for REFSENS and RSD estimation

The assumptions are shown in Table 1.

Table 1. Calculation conditions

| Parameter | Value | Comment |
| --- | --- | --- |
| Tx power | 23 dBm | For 1Tx PC3, at ANT1 |
|  | 26 dBm | For 1Tx PC2, at ANT1 |
|  | 23 dBm | For 2Tx PC2, at ANT1&ANT2 |
| Post PA loss | 4 dB |  |
| ANT_ Iso. | 10 dB | ANT1⇔ANT2 |
| Duplexer rejection | 55 | Tx to Rx, at Tx band |
|  | 55 | Tx to Rx, at Rx band |
|  | 40 | Tx to ANT, at Rx band |
| BPF | 40 | ANT to Rx, at Tx band |
| MRC | ANT1&ANT2 | Correlated |

Results

The estimated results are shown in Table 2.

UL BW is restricted to 20MHz same as n8 DL 25MHz and 35MHz.

Table 2. REFSENS for PC3 and RSD for PC2

|  | Parameter | Unit | UL/DL = 20/30MHz |
| --- | --- | --- | --- |
| PC3 | SCS | kHz | 15 |
|  | Scaling REFSENS | dBm | -88.9 |
|  | De-sense | dB | 7.7 |
|  | MRC REFSENS | dBm | -81.3 |
| PC2 | RSD 1Tx | dB | 2.9 |
|  | RSD 2Tx | dB | 6.6 |

Proposal:

The REFSENS of PC3 and RSD of PC2 for n8 DL 30MHz are proposed in bold in Table 2.

Conclusion

In this contribution, we proposed the REFSENS for PC3 and RSD for PC2 for n8 DL 30MHz.

Proposal:

The REFSENS of PC3 and RSD of PC2 for n8 DL 30MHz are proposed in bold in Table 2.

Table 2. REFSENS for PC3 and RSD for PC2

|  | Parameter | Unit | UL/DL = 20/30MHz |
| --- | --- | --- | --- |
| PC3 | SCS | kHz | 15 |
|  | Scaling REFSENS | dBm | -88.9 |
|  | De-sense | dB | 7.7 |
|  | MRC REFSENS | dBm | -81.3 |
| PC2 | RSD 1Tx | dB | 2.9 |
|  | RSD 2Tx | dB | 6.6 |

References

[1] RP-231706 “Basket WID: adding new channel bandwidth(s) support to existing NR bands”, Ericsson, 3GPP TSG RAN Meeting #101, Bangalore, India, September 11 – 15, 2023