3GPP TSG-RAN WG4 Meeting # 108bis	R4-2315475

# Xiamen, China, October 09 – October 13, 2023

Agenda item:	5.7.4.2

Source:	Apple

Title:	Simulation results for PDSCH with multi-RX in FR2

Release:	Rel-18

Document for:	Information

# 1. Introduction

In RAN4#108 demodulation requirements for multi-RX in FR2 were discussed and way forward [1] was agreed.  In this contribution we present simulation results for PDSCH performance evaluation with multi-RX reception on the DL in FR2.

# 2. Simulation Results

We present initial performance evaluation results with different crosstalk levels for sDCI and mDCI transmission schemes agreed. The purpose of this evaluation is to understand the feasible test configurations for introducing requirements for multi-RX in FR2.  For initial evaluation of performance of PDSCH the following simulation parameters were used based on agreements in [1].

- Single DCI SDM:

  - MCS/Layers

    - 1+1 – MCS 17,

    - 2+2 – MCS 13

  - UE processing

    - Joint processing

    - Separate processing

  - FO/TO for TRP2

    - 0 Hz, 0.25us

    - 600 Hz, -0.0625us

- Multi DCI Non overlapping:

  - MCS/Layers: 2+2 – MCS 17

  - UE processing: separate processing

  - FO/TO for TRP2: 600 Hz, -0.0625us

- Multi DCI Fully overlapping

  - MCS/Layers

    - 1+1 – MCS 17

    - 2+2 – MCS 13

  - UE processing:

    - Joint processing

    - Separate processing

  - FO/TO for TRP2

    - 0 Hz, 0.25us

    - 600 Hz, -0.0625us

- Channel Model: TDLA30-75

- Cross talk values

  - ρ  =  -15, -12, -9, -6 dB

- SCS/CBW: 120KHz/100MHz

- Receiver assumption: MMSE-IRC

## Multi-DCI Transmission Scheme

Separate processing

Observations:

- With separate processing, non-overlapping PDSCH performance is not impacted with different levels of crosstalk.

- For overlapping PDSCH, 1 layer per TRP is not severely degraded with crosstalk.

- For overlapping PDSCH with 2 layers per TRP, performance is severely degraded.

- Performance is better with smaller TO than with larger TO for overlapping PDSCH.

- Multi-DCI with non-overlapping PDSCH is not impacted by cross talk.

- Multi-DCI with 1 layer per TRP overlapping PDSCH is not severely impacted by cross talk with separate processing.

- Multi-DCI with 2 layers per TRP overlapping PDSCH is severely impacted by cross talk with separate processing.

- Multi-DCI with overlapping PDSCH has better performance with smaller TO (-0.0625us)

Joint processing

Observations:

- For overlapping PDSCH joint processing is more robust to crosstalk.

- Performance is better with smaller TO than with larger TO for overlapping PDSCH.

- For multi-DCI with 1 or 2 layers per TRP overlapping PDSCH, joint processing is more robust to cross talk.

- Multi-DCI with overlapping PDSCH has better performance with smaller TO (-0.0625us)

## Single-DCI SDM Transmission Scheme

Joint processing

Observations:

- For overlapping PDSCH with single DCI SDM, joint processing is robust to crosstalk.

- Performance is better with smaller TO than with larger TO

- For single-DCI SDM with 1 or 2 layers per TRP, joint processing is robust to cross talk.

- Single-DCI with overlapping PDSCH has better performance with smaller TO (-0.0625us)

# 3. Conclusion

In this paper, we provide initial performance evaluation results for PDSCH demodulation with multi-RX reception on the DL in FR2. Our observations are captured below:

- Multi-DCI with non-overlapping PDSCH is not impacted by cross talk.

- Multi-DCI with 1 layer per TRP overlapping PDSCH is not severely impacted by cross talk with separate processing.

- Multi-DCI with 2 layers per TRP overlapping PDSCH is severely impacted by cross talk with separate processing.

- Multi-DCI with overlapping PDSCH has better performance with smaller TO (-0.0625us)

- For multi-DCI with 1 or 2 layers per TRP overlapping PDSCH, joint processing is more robust to cross talk.

- Multi-DCI with overlapping PDSCH has better performance with smaller TO (-0.0625us)

- For single-DCI SDM with 1 or 2 layers per TRP, joint processing is robust to cross talk.

- Single-DCI with overlapping PDSCH has better performance with smaller TO (-0.0625us)

# Reference

- R4-2313977, “WF for Rel-18 Multi-Rx performance requirements”, Qualcomm.