3GPP TSG-RAN WG4 Meeting # 108bis	R4-2315482

# Xiamen, China, October 09 – October 13, 2023

Agenda item:	5.21.3

Source:	Apple

Title:	On testability with AI/ML in air interface

Release:	Rel-18

Document for:	Discussion

# 1. Introduction

In RAN4#108 RAN4 started the discussion on RAN4 impact with AI/ML in the air interface and way forward [1] was noted.  In this contribution we present our views on testability with AI/ML in air interface.

# 2. Discussion

In RAN4#107 the following agreements were made for interoperability and testing:

Issue 3-3: Encoder/decoder for 2-sided model Agreement: - Down select option 6.

On the testing framework 2-sided the following options are on the table and we need to analyze them in the SI and capture them in TR 38.843:

Issue 3-3: Encoder for 2-sided model

  - Option 1: reference decoder is provided by the vendor of the encoder under test so that the encoder and decoder are jointly designed and trained

  - Option 2: reference decoder is provided by the vendor of the decoder(infra-vendors) so that the encoder and decoder are jointly designed and trained

  - Option 3: The reference decoder(s) are fully specified and captured in RAN4 spec to ensure identical implementation across equipment vendors without additional training procedure needed.

  - Option 4: The reference decoder(s) are partially specified and captured in RAN4 spec.

- For Option 3 - Fully specified model means the model and weights are captured in RAN4 specification.

- Confirm that for option 3 model and weights are fully specified in RAN4 specification.

- For Option 4 – It is not clear what partially specified means. Is only the architecture of the model or is the model is specified, but training is left to TE.

- Further clarify what option 4 – partially specified model means. If only architecture of model is specified, or model is specified with training left to TE.

In RAN4#108, it was agreed to update the terminology to use test encoder/decoder. We propose to update the terminology in options to use test decoder/encoder.

- Update the wording in options 1-4 to use test decoder/encoder as - 
For UE under test - 
Option 1: Test decoder is provided by the vendor of the encoder under test so that the encoder and decoder are jointly designed and trained

Option 2: Test decoder is provided by the vendor of the decoder(infra-vendors) so that the encoder and decoder are jointly designed and trained

Option 3: The test decoder(s) are fully specified and captured in RAN4 spec to ensure identical implementation across equipment vendors without additional training procedure needed.

Option 4: The test decoder(s) are partially specified and captured in RAN4 spec.

For gNB under test - 
Option 1: Test encoder is provided by the vendor of the decoder under test so that the encoder and decoder are jointly designed and trained

Option 2: Test encoder is provided by the vendor of the encoder(UE) so that the encoder and decoder are jointly designed and trained

Option 3: The test encoders(s) are fully specified and captured in RAN4 spec to ensure identical implementation across equipment vendors without additional training procedure needed.

Option 4: The test encoders(s) are partially specified and captured in RAN4 spec.

In RAN4#108 there was a table proposed to collect company views on the options. We use the table to provide our response to the different options for encoder testing and test decoder in table 1 and for test encoder in table 2.

Table 1: Options analysis for test decoder

|  | Option 1 | Option 2 | Option 3 | Option 4 |
| --- | --- | --- | --- | --- |
| Clarification of options |  |  |  |  |
| Source of the test decoder | UE vendor | NW vendor | RAN4 spec | RAN4 spec and TE vendor |
| Source of decoder training data | No additional training is needed at TE since UE vendor gives the model after training | No additional training is needed at TE since NW vendor gives the model after training | If model is fully specified, no additional training is needed | UE vendor provides training data to TE for training. |
| DUT vendor knowledge of the test decoder | Complete | No knowledge | Complete | partial based on what is specified |
| Supported training collaboration type (source of training data should be consistent with the collaboration type) | Type 1 with UE side training Type 3 training with UE first | Type 1 with NW side training Type 3 with NW first training | This is possibly the parallel training under Type 3 which was not further discussed in RAN1 | Unsure as it is not clear what is partially specified |
| Test decoder verification procedure at TE and/or DUT | Up to the UE to guarantee the decoder works with the encoder. Tested at DUT | The reference encoder used by NW should be made available to verify the decoder provided by NW | Should be guaranteed when specified, no need to test | Unsure |
| Feasibility of test decoder verification procedure | Feasible at DUT | Feasible at TE with reference encoder from NW | During specification effort | Unsure |
| Pros/Cons analysis |  |  |  |  |
| Reflection on the real deployment (knowledge of model, training type, etc.) | It could potentially be the same model used in testing and deployment with Type 1 UE side training | It could potentially be the same model used in testing and deployment with Type 1 NW side training | It could limit implementation of encoder model to work with the test decoder, or potentially lead to mismatch in model used for testing and in deployment | With no knowledge of the full decoder, there might be a mismatch in encoder-decoder pair, as test decoder is only partially specified. It could limit implementation of encoder model to work with the test decoder, or potentially lead to mismatch in model |
| TE requirements to deploy the decoder (e.g. training, complexity, interoperability) | UE vendor provides trained model to TE | NW vendor provides trained model to TE | Assume no training needed, fully specified. Complex to ensure inter-operability with different UE vendor encoder models | Train the model. Complex to ensure inter-operability with different UE vendor encoder models |
| Specification Effort (e.g. test decoder) | Need not be specified | Need not be specified | It would take a lot of time and effort to reach consensus on a test decoder model to be specified by RAN4 | It would take a lot of time and effort to reach consensus on a test decoder model to be specified by RAN4 |
| Confidentiality/IP issues | Disclose to TE | Disclose to TE | decoder already specified, and disclosed | decoder already specified, and disclosed |
| Applicability to different scenarios/conditions/ configurations | The model should be applicable for the scenarios/ configurations tested for in RAN4 | The model should be applicable for the scenarios/ configurations tested for in RAN4 | The model should be applicable for the scenarios/ configurations tested for in RAN4 | The model should be applicable for the scenarios/ configurations tested for in RAN4 |
| Complexity of actual testing procedure for the ecosystem | complex | complex | complex | complex |

Table 2: Options analysis for test encoder

|  | Option 1 | Option 2 | Option 3 | Option 4 |
| --- | --- | --- | --- | --- |
| Clarification of options |  |  |  |  |
| Source of the test encoder | NW vendor | UE vendor | RAN4 spec | RAN4 spec and TE vendor |
| Source of encoder training data | No additional training is needed at TE since NW vendor gives the model after training | No additional training is needed at TE since UE vendor gives the model after training | If model is fully specified, no additional training is needed | UE vendor provides training data to TE for training. |
| DUT vendor knowledge of the test encoder | Complete | No knowledge | Complete | partial based on what is specified |
| Supported training collaboration type (source of training data should be consistent with the collaboration type) | Type 1 with NW side training Type 3 training with NW first | Type 1 with UE side training Type 3 with UE first training | This is possibly the parallel training under Type 3 which was not further discussed in RAN1 | Unsure as it is not clear what is partially specified |
| Test encoder verification procedure at TE and/or DUT | Up to the NE to guarantee the decoder works with the encoder. Tested at DUT | The reference decoder used by UE should be made available to verify the encoder provided by UE | Should be guaranteed when specified, no need to test | Unsure |
| Feasibility of test encoder verification procedure | Feasible at DUT | Feasible at TE with reference decoder from UE | During specification effort | Unsure |
| Pros/Cons analysis |  |  |  |  |
| Reflection on the real deployment (knowledge of model, training type, etc.) | It could potentially be the same model used in testing and deployment with Type 1 NE side training | It could potentially be the same model used in testing and deployment with Type 1 UE side training | It could limit implementation of decoder model to work with the test encoder, or potentially lead to mismatch in model used for testing and in deployment | With no knowledge of the full decoder, there might be a mismatch in encoder-decoder pair, as test encoder is only partially specified. It could limit implementation of decoder model to work with the test encoder, or potentially lead to mismatch in model |
| TE requirements to deploy the encoder (e.g. training, complexity, interoperability) | UE vendor provides trained model to TE | NW vendor provides trained model to TE | Assume no training needed, fully specified. Complex to ensure inter-operability with different NW vendor encoder models | Train the model. Complex to ensure inter-operability with different NW vendor encoder models |
| Specification Effort (e.g. test encoder) | Need not be specified | Need not be specified | It would take a lot of time and effort to reach consensus on a test decoder model to be specified by RAN4 | It would take a lot of time and effort to reach consensus on a test decoder model to be specified by RAN4 |
| Confidentiality/IP issues | Disclose to TE | Disclose to TE | Encoder already specified, and disclosed | Encoder already specified, and disclosed |
| Applicability to different scenarios/conditions/ configurations | The model should be applicable for the scenarios/ configurations tested for in RAN4 | The model should be applicable for the scenarios/ configurations tested for in RAN4 | The model should be applicable for the scenarios/ configurations tested for in RAN4 | The model should be applicable for the scenarios/ configurations tested for in RAN4 |
| Complexity of actual testing procedure for the ecosystem | complex | complex | complex | complex |

# 3. Conclusion

In this paper, we provide our views on issues related to testability aspects for AI/ML. We also provide our views on the options for test decoder/encoder. Our observations and proposals are captured below:

- For Option 3 - Fully specified model means the model and weights are captured in RAN4 specification.

- Confirm that for option 3 model and weights are fully specified in RAN4 specification.

- For Option 4 – It is not clear what partially specified means. Is only the architecture of the model or is the model is specified, but training is left to TE.

- Further clarify what option 4 – partially specified model means. If only architecture of model is specified, or model is specified with training left to TE.

- Update the wording in options 1-4 to use test decoder/encoder as - 
For UE under test - 
Option 1: Test decoder is provided by the vendor of the encoder under test so that the encoder and decoder are jointly designed and trained

Option 2: Test decoder is provided by the vendor of the decoder(infra-vendors) so that the encoder and decoder are jointly designed and trained

Option 3: The test decoder(s) are fully specified and captured in RAN4 spec to ensure identical implementation across equipment vendors without additional training procedure needed.

Option 4: The test decoder(s) are partially specified and captured in RAN4 spec.

For gNB under test - 
Option 1: Test encoder is provided by the vendor of the decoder under test so that the encoder and decoder are jointly designed and trained

Option 2: Test encoder is provided by the vendor of the encoder(UE) so that the encoder and decoder are jointly designed and trained

Option 3: The test encoders(s) are fully specified and captured in RAN4 spec to ensure identical implementation across equipment vendors without additional training procedure needed.

Option 4: The test encoders(s) are partially specified and captured in RAN4 spec.

# Reference

- R4-2314740, “WF on FS_NR_AIML_air”, Qualcomm Incorporated.