3GPP TSG-RAN WG4 Meeting # 108bis	R4-2315484

# Xiamen, China, October 09 – October 13, 2023

Agenda item:	5.29.2.1

Source:	Apple

Title:	On RRM requirements impacts for NR MIMO evolution for downlink and uplink

Release:	Rel-18

Document for:	Discussion

# 1. Introduction

In RAN4#108 RRM impacts for R18 MIMO evolution were discussed and way forward [1] was agreed.  In this contribution we present our views on RRM requirements for NR MIMO evolution for downlink and uplink other than 2TA and unified TCI.

# 2. Discussion

For RRM impacts for R18 MIMO evolution the following agreements were reached in [1]:

Issue 1-1-1: Whether to define TDCP measurement delay requirements? Agreement: - Do not define TDCP measurement delay requirements Issue 1-1-2: Whether to define TDCP measurement accuracy requirements? Agreement: - Further identify feasibility of methodology and test setup to define TDCP accuracy requirements including at least ideal TDCP definition, channel models - Define TDCP accuracy requirements subject to conclusions of feasibility analysis

TDCP Reporting

RAN1 has agreed to introduce TRS based TDCP reporting as part of CSI enhancements under R18 MIMO evolution.

The correlation value between 2 symbols is reported as the TDCP report. The correlation value depends on the UE speed/ Doppler spread of the channel. For a slow fading channel/ low velocity the correlation value is high whereas for a fast-fading channel/ high velocity the correlation is low. It is hard to quantify the actual correlation value for a propagation condition. The genie value for correlation given a propagation model is hard to obtain. It is not like RSRP or SINR accuracy tests where they are determined based on test conditions.

- The genie correlation value for a given propagation model/condition is hard to quantify.

It is not possible to define measurement accuracy requirements without a genie or ideal measurement.

- It is not feasible to define measurement accuracy requirements without a genie or ideal measurement for correlation.

In RAN4#108 there were proposals to use CDL channel model for introducing TDCP accuracy requirements. We don’t use CDL channel models to define requirements, as they are not calibrated in RAN4 and implementable by TE.

- It is not feasible to use CDL channel model to define requirements.

Based on the above observations, we propose not to introduce any TDCP measurement accuracy requirements.

Do not introduce TDCP measurement accuracy requirements.

SRS Enhancements

In [1] the following were captured:

Issue 1-2-1: Whether to specify RRM requirements for Rel-18 SRS enhancement for 8TX UL?

Way forward:

  - Option1: (MediaTek)

    - If S=8 (subsets factor) is agreed in RAN1, then RAN4 should specify RRM requirement for SRS enhancement in this WI.

  - Option 2: (Samsung, Huawei, vivo)

    - Not to specify new SRS switching RRM requirements for 8TX UL. Legacy requirements of Interruptions at NR SRS antenna port switching can be reused.

We support option 2, and this is also in line with previous agreements.

For Option 1, there are no agreements in RAN1 to support TDM factor s=8 and only s=2 is supported. Hence, we don’t need to specify any requirements for that case.

- TDM factor s=2 is supported in RAN1. No agreement to support s=8.

Do not specify new SRS switching RRM requirements for 8TX UL. Legacy interruption requirements of NR SRS antenna port switching can be reused.

# 3. Conclusion

In this paper, we provide our views on open issues on RRM requirements for NR MIMO evolution for downlink and uplink other than 2TA and unified TCI. Our observations and proposals are captured below:

- The genie correlation value for a given propagation model/condition is hard to quantify.

- It is not feasible to define measurement accuracy requirements without a genie or ideal measurement for correlation.

- It is not feasible to use CDL channel model to define requirements.

- Do not introduce TDCP measurement accuracy requirements.

- TDM factor s=2 is supported in RAN1. No agreement to support s=8.

- Do not specify new SRS switching RRM requirements for 8TX UL. Legacy interruption requirements of NR SRS antenna port switching can be reused.

# Reference

- R4-2314347, “WF on R18 NR MIMO RRM requirement”, Samsung