3GPP TSG-RAN WG4 Meeting #108bis	R4-2315490

Xiamen, China, 9th ~13th October 2023

Source: 	Apple

Title: 	RRM requirements applicability for interBandMRDC-WithOverlapDL-Bands-r16

Agenda Item:	7.3.5

Document for:	Discussion

# Introduction

To RAN4:

ACTION: 	RAN2 respectfully requests RAN4 to provide answers to the questions in order to clarify the ambiguities described above.

Question 1: For UE supporting and not supporting interBandMRDC-WithOverlapDL-Bands-r16, what are the requirements in NE-DC operation respectively?

Question 2: For FDD-FDD inter-band EN-DC with overlapping frequency, if UE does not report interBandMRDC-WithOverlapDL-Bands-r16, what are the MRTD requirements for asynchronous operation? And are there any differences on MRTD requirements in asynchronous operation for UE(s) supporting and not supporting interBandMRDC-WithOverlapDL-Bands-r16?

Question 3: For FDD-FDD inter-band EN-DC with overlapping frequency, if UE does not report interBandMRDC-WithOverlapDL-Bands-r16, what are the MTTD requirements for both synchronous and asynchronous operations? And for asynchronous operation, are there any differences on MTTD requirements for UE(s) supporting and not supporting interBandMRDC-WithOverlapDL-Bands-r16?

In the incoming LS reply [1], the following questions are raised by RAN2 on RAN4 requirement applicability related to UE capability interBandMRDC-WithOverlapDL-Bands-r16. This contribution will present our view on the questions raised by RAN2.

# Discussion

Question 1: For UE supporting and not supporting interBandMRDC-WithOverlapDL-Bands-r16, what are the requirements in NE-DC operation respectively?

The UE capability interBandMRDC-WithOverlapDL-Bands-r16 was introduced in Rel-16 related to the FDD example EN-DC band combination DC_20-n28. NE-DC is also added to the signaling description although NE-DC was firstly discussed in Rel-17. In 38.307, the NE-DC requirement is applied for UE from R17 onwards.

For TDD case, the concerning band combos are DC_42_n77/n78 for UE indicating support of interBandMRDC-WithOverlapDL-Bands-r16. According to the Band combination definition in 38.101-3, only single uplink is supported for these band combinations and dual UL is not supported (E.g. N/A). And there are no NE-DC combinations specified up to now for these concerning bands.

Table 2-1: TDD-TDD EN-DC combinations for overlapping inter-band non-collocated scenario

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed | DL interruption allowed (Note 14) |
| --- | --- | --- | --- |
| DC_42A_n77A3,4,9,11 DC_42A_n77C3,4,9,11 DC_42C_n77A3,4,9,11 DC_42C_n77C3,4,9,11 DC_42D_n77A3,4,9,11 DC_42D_n77C DC_42E_n77A3,4,9,11 DC_42E_n77C | N/A | N/A |  |
| DC_42A_n77(2A)3,4,9,11 DC_42C_n77(2A)3,4,9,11 | N/A | N/A |  |
| DC_42A_n78A3,4,9,11 DC_42A_n78C3,4,9,11 DC_42C_n78A3,4,9,11 DC_42C_n78C3,4,9,11 DC_42D_n78A3,4,9,11 DC_42D_n78C3,4,9,11 DC_42E_n78A3,4,9,11 DC_42E_n78C3,4,9,11 | N/A | N/A |  |

Observation 1: For TDD-TDD band combination related to interBandMRDC-WithOverlapDL-Bands-r16, no MTTD requirement is needed since only single uplink is supported for EN-DC and no NE-DC combos are introduced. While the current MTTD requirement is specific to TDD combinations for UE indicating interBandMRDC-WithOverlapDL-Bands-r16.

Proposal 1: It is proposed to update the MTTD requirement in section 7.5.2.1. e.g. it can be specified in generic way or specific to FDD inter-band EN-DC.

For FDD EN-DC case, for UE indicating support of interBandMRDC-WithOverlapDL-Bands-r16, the MRTD requirement in 7.6.2.1 is applicable. Otherwise, the MRTD requirement in 7.6.3 is applicable. For deployment and implementation perspective, NE-DC and EN-DC will share same MRTD and MTTD requirements for the same set of concerning component bands. While in current 38.133, 7.6.3 is specified for EN-DC case only. There is no corresponding section for NE-DC case.

Observation 2: From deployment perspective, the MRTD requirement for NE-DC can be the same as that for EN-DC regarding the same set of component bands.

Observation 3: There is no description about how to apply MRTD/MTTD requirement for NE-DC for UE indicating/not indicating support of interBandMRDC-WithOverlapDL-Bands-r16 in 38.133.

Proposal 2: It is proposed to reuse inter-band synchronous NE-DC MRTD/MTTD requirement in 7.6.5.1/7.5.5.1 for UE indicating capable of interBandMRDC-WithOverlapDL-Bands-r16, and same requirement as in 7.6.3/7.5.3 for UE not indicating capable of interBandMRDC-WithOverlapDL-Bands-r16.

Question 2: For FDD-FDD inter-band EN-DC with overlapping frequency, if UE does not report interBandMRDC-WithOverlapDL-Bands-r16, what are the MRTD requirements for asynchronous operation? And are there any differences on MRTD requirements in asynchronous operation for UE(s) supporting and not supporting interBandMRDC-WithOverlapDL-Bands-r16?

From 38.133, the applicability of different MRTD requirements for different FDD-FDD EN-DC scenarios are interpretated as below:

- For FDD-FDD inter-band EN-DC without overlapping frequency, the current requirement for asynchronous EN-DC as specified in Table 7.6.2-1 is applicable as mandatory.

- For FDD-FDD inter-band EN-DC with overlapping downlink frequency, the requirement applicability depends on UE capability reporting.

  - If asyncIntraBandENDC is reported, the current requirement for asynchronous EN-DC as specified in Table 7.6.2-1 apply.

  - If asyncIntraBandENDC is not reported, our understanding is that,

    - The current requirement for synchronous inter-band EN-DC as specified in 7.6.2.1 (to be modified) should apply for UE reporting capable of interBandMRDC-WithOverlapDL-Bands-r16 (corresponding to non-collocated scenario). But it is not specified in 38.133 yet.

    - The current requirement for synchronous intra-band collocated EN-DC as specified in 7.6.3 should apply for UE not reporting capable of interBandMRDC-WithOverlapDL-Bands-r16 (corresponding to collocated scenario).

Proposal 3: It is proposed to provide answer for Q2 as following:

- For FDD-FDD inter-band EN-DC with overlapping downlink frequency, the requirement applicability depends on UE capability reporting.

  - If asyncIntraBandENDC is reported, the current requirement for asynchronous EN-DC as specified in Table 7.6.2-1 apply.

  - If asyncIntraBandENDC is not reported,

    - The current requirement for synchronous inter-band EN-DC as specified in 7.6.2.1 (to be modified) should apply for UE reporting capable of interBandMRDC-WithOverlapDL-Bands-r16 (corresponding to non-collocated scenario). But it is not specified in 38.133 yet.

    - The current requirement for synchronous intra-band collocated EN-DC as specified in 7.6.3 should apply for UE not reporting capable of interBandMRDC-WithOverlapDL-Bands-r16 (corresponding to collocated scenario).

Question 3: For FDD-FDD inter-band EN-DC with overlapping frequency, if UE does not report interBandMRDC-WithOverlapDL-Bands-r16, what are the MTTD requirements for both synchronous and asynchronous operations? And for asynchronous operation, are there any differences on MTTD requirements for UE(s) supporting and not supporting interBandMRDC-WithOverlapDL-Bands-r16?

Similar answer as that for MRTD requirement apply for MTTD. From 38.133, the applicability of different MTTD requirements for different FDD-FDD EN-DC scenarios are interpretated as below:

- For FDD-FDD inter-band EN-DC without overlapping frequency, the current requirement for asynchronous EN-DC as specified in Table 7.5.2-1 is applicable as mandatory.

- For FDD-FDD inter-band EN-DC with overlapping downlink frequency, the requirement applicability depends on UE capability reporting.

  - If asyncIntraBandENDC is reported, the current requirement for asynchronous EN-DC as specified in Table 7.5.2-1 apply.

  - If asyncIntraBandENDC is not reported, our understanding is that,

    - The current requirement for synchronous inter-band EN-DC as specified in 7.5.2.1 (to be modified for FDD) should apply for UE reporting capable of interBandMRDC-WithOverlapDL-Bands-r16 (corresponding to non-collocated scenario). But it is not specified in 38.133 yet.

    - The current requirement for synchronous intra-band collocated EN-DC as specified in 7.5.3 should apply for UE not reporting capable of interBandMRDC-WithOverlapDL-Bands-r16 (corresponding to collocated scenario).

Proposal 4: It is proposed to provide answer for Q3 as following:

- For FDD-FDD inter-band EN-DC without overlapping frequency, the current requirement for asynchronous EN-DC as specified in Table 7.5.2-1 is applicable as mandatory.

- For FDD-FDD inter-band EN-DC with overlapping downlink frequency, the requirement applicability depends on UE capability reporting.

  - If asyncIntraBandENDC is reported, the current requirement for asynchronous EN-DC as specified in Table 7.5.2-1 apply.

  - If asyncIntraBandENDC is not reported

    - The current requirement for synchronous inter-band EN-DC as specified in 7.5.2.1 (to be modified for FDD) should apply for UE reporting capable of interBandMRDC-WithOverlapDL-Bands-r16 (corresponding to non-collocated scenario). But it is not specified in 38.133 yet.

    - The current requirement for synchronous intra-band collocated EN-DC as specified in 7.5.3 should apply for UE not reporting capable of interBandMRDC-WithOverlapDL-Bands-r16 (corresponding to collocated scenario).

It is also noticed that the signaling asyncIntraBandENDC is appliable for intra-band EN-DC as well as inter-band EN-DC with overlapping frequency. While in the signaling description, it only says intra-band EN-DC requirement apply when asyncIntraBandENDC is not reported. For inter-band EN-DC with overlapping DL frequency, the synchronous EN-DC requirement in 7.6.2.1 should apply instead. But this is missing from the signaling description. Which makes it confusing for inter-band EN-DC with overlapping frequency.

Further, technically it seems reasonable that the applicability of asynchronous NE-DC requirement applicability follow the same logic as EN-DC. E.g. asynchronous MTTD and MRTD requirements for NE-DC are only applicable when indicated support by UE. But now there is no such dedicated signaling for NE-DC case. It would be preferable to check whether asyncIntraBandENDC can be updated to also cover NE-DC UE reporting.

It is proposed to send this observation to RAN2 and let RAN2 to consider updating the asyncIntraBandENDC signaling description.

Proposal 5: Send LS to RAN2 to update the asyncIntraBandENDC signaling description that

- the synchronous EN-DC requirement in 7.6.2.1 should apply for inter-band EN-DC with overlapping frequency if asyncIntraBandENDC is not reported.

- Check whether the signaling can also cover NE-DE UE reporting

# Summary

This contribution presented our view on RAN4 requirement applicability related to RAN2 questions. The following observations and proposals are concluded.

Observation 1: For TDD-TDD band combination related to interBandMRDC-WithOverlapDL-Bands-r16, no MTTD requirement is needed since only single uplink is supported for EN-DC and no NE-DC combos are introduced. While the current MTTD requirement is specific to TDD combinations for UE indicating interBandMRDC-WithOverlapDL-Bands-r16.

Proposal 1: It is proposed to update the MTTD requirement in section 7.5.2.1 for FDD inter-band non-collocated EN-DC.

Observation 2: From deployment perspective, the MRTD requirement for NE-DC can be the same as that for EN-DC regarding the same set of component bands.

Observation 3: There is no description about how to apply MRTD/MTTD requirement for NE-DC for UE indicating/not indicating support of interBandMRDC-WithOverlapDL-Bands-r16 in 38.133.

Proposal 2: It is proposed to reuse inter-band synchronous NE-DC MRTD/MTTD requirement in 7.6.5.1/7.5.5.1 for UE indicating capable of interBandMRDC-WithOverlapDL-Bands-r16, and same requirement as in 7.6.3/7.5.3 for UE not indicating capable of interBandMRDC-WithOverlapDL-Bands-r16.

Proposal 3: It is proposed to provide answer for Q2 as following:

- For FDD-FDD inter-band EN-DC with overlapping downlink frequency, the requirement applicability depends on UE capability reporting.

  - If asyncIntraBandENDC is reported, the current requirement for asynchronous EN-DC as specified in Table 7.6.2-1 apply.

  - If asyncIntraBandENDC is not reported,

    - The current requirement for synchronous inter-band EN-DC as specified in 7.6.2.1 (to be modified) should apply for UE reporting capable of interBandMRDC-WithOverlapDL-Bands-r16 (corresponding to non-collocated scenario). But it is not specified in 38.133 yet.

    - The current requirement for synchronous intra-band collocated EN-DC as specified in 7.6.3 should apply for UE not reporting capable of interBandMRDC-WithOverlapDL-Bands-r16 (corresponding to collocated scenario).

Proposal 4: It is proposed to provide answer for Q3 as following:

- For FDD-FDD inter-band EN-DC without overlapping frequency, the current requirement for asynchronous EN-DC as specified in Table 7.5.2-1 is applicable as mandatory.

- For FDD-FDD inter-band EN-DC with overlapping downlink frequency, the requirement applicability depends on UE capability reporting.

  - If asyncIntraBandENDC is reported, the current requirement for asynchronous EN-DC as specified in Table 7.5.2-1 apply.

  - If asyncIntraBandENDC is not reported,

    - The current requirement for synchronous inter-band EN-DC as specified in 7.5.2.1 (to be modified for FDD) should apply for UE reporting capable of interBandMRDC-WithOverlapDL-Bands-r16 (corresponding to non-collocated scenario). But it is not specified in 38.133 yet.

    - The current requirement for synchronous intra-band collocated EN-DC as specified in 7.5.3 should apply for UE not reporting capable of interBandMRDC-WithOverlapDL-Bands-r16 (corresponding to collocated scenario).

Proposal 5: Send LS to RAN2 to update the asyncIntraBandENDC signaling description that

- the synchronous EN-DC requirement in 7.6.2.1 should apply for inter-band EN-DC with overlapping frequency if asyncIntraBandENDC is not reported.

- Check whether the signaling can also cover NE-DE UE reporting

# References

[1] R2-2309218, Reply LS on update for “interBandMRDC-WithOverlapDL-Bands-r16” in 38.306.

[2] R4-1711964, WF on synchronous and asynchronous definitions for LTE-NR DC in Rel-15, Ericsson, MediaTek, RAN4#84