3GPP TSG-RAN WG4 Meeting #108bis		R4-2315503

Xiamen, China, Oct. 9 – 13, 2023

Agenda item:	5.7.1.1

Source:	Apple

Title:   	TP on NTC vs. ETC

Document for:	Approval

# 1	Introduction

As discussed in [1], there are two options of addressing the NTC vs. ETC issue with respect to the two-AoA spherical coverage requirement.

- Option 1: keep core requirement wording for the two AoA spherical coverage requirement consistent with that of the legacy single AoA requirement, i.e., with a note saying the EIS spherical coverage requirements are verified only under normal thermal conditions as defined in Annex E.2.1.

- Option 2: Do not use the note in the two-AoA spherical coverage requirement but ask RAN5 to verify it for NTC only. Note this option was driven by the argument that the use of note for legacy requirement is not preferred as core requirement should be specified independent of how it is verified.

At the last meeting, it seemed most companies are OK with Option 2. If RAN4 decides to take Option 2, we propose to document the rationale behind this decision and capture in TR 38.751 [2] that RAN4 agrees to verify the requirement under NTC only. In this contribution, a TP is provided for this purpose.

# 2	Text Proposal

### 6.3.7	NTC vs. ETC

During the Rel-15 discussion when the legacy single-AoA spherical coverage requirement was specified, the testing conditions were captured as part of the requirement in TS 38.101-2 as a note, i.e., Note 2 in Table 7.3.4.3-1.

When it comes to the two-AoA spherical coverage requirement, the same issue emerged whether the requirement shall be verified under normal temperature conditions (NTC) but not under extreme temperature conditions (ETC).

As the legacy spherical coverage requirement are required to be verified under NTC only, and the DL fixed power level for the two-AoA requirement is directly reused from the legacy spherical coverage requirement, it is reasonable to consider only the NTC condition for verifying the two-AoA requirement. To achieve this purpose in specification, there are two options:

- Option 1: keep core requirement wording for the two AoA spherical coverage requirement consistent with that of the legacy single AoA requirement, i.e., with a note saying the EIS spherical coverage requirements are verified only under normal thermal conditions as defined in Annex E.2.1.

- Option 2: Do not use the note in the two AoA spherical coverage requirement but ask RAN5 to verify it for NTC only. Note this option was driven by the argument that the use of note for legacy requirement is not preferred as core requirement should be specified independent of how it is verified.

The argument for Option 2 is the RAN4 convention of specifying core requirements should be made as much as independent of the testing conditions and such convention should be preserved. The way the legacy requirement is specified is some exception that should be avoided. Furthermore, RAN4 can ask RAN5 to verify the requirement only under NTC. In this way, the RAN4 convention is kept, and no additional work is required to derive the appropriate core requirement for ETC.

In the end, Option 2 is adopted.

# 4	References

R4-23xxxxx, "RF requirement for NR FR2 multi-Rx chain DL reception," Apple

TR 38.751, " User Equipment (UE) RF requirements for NR frequency range 2 (FR2) multi-Rx chain DL reception," v1.0.0