3GPP TSG-RAN WG4 Meeting #108bis	R4-2315523

Xiamen, China, 09 – 13 October 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	RRM performance aspects on network energy saving

Agenda item:	5.34.4

Document for:	Discussion

Introduction

According to the WID on network energy saving [1], the performance part will be discussed starting from RAN4#108bis meeting. In this paper, we will provide our views on RRM performance aspects.

4.2    Objective of Performance part WI

NOTE:    Leave empty if the WI proposal does not contain a RAN performance part.

- Specify corresponding RRM performance requirements and test cases for the network energy saving techniques [RAN4].

- Specify the necessary demodulation performance and CSI reporting requirements [RAN4].

- Specify the BS conformance tests, if necessary [RAN4].

Discussion

SSB-less operation for inter-band CA

For SSB-less operation, the performance requirements for SSB-based L3/L1 measurements do not apply as SSB is assumed not transmitted on the SCell. What matters the performance may be the CSI-RS based L3/L1 measurements if configured on SSB-less SCells. If the necessity of CSI-RS based L3/L1 measurement is concluded in core requirement, the accuracy requirements need to be defined accordingly.

Proposal 1: If the necessity of CSI-RS based L3/L1 measurement is concluded in core requirement, the accuracy requirements for the CSI-RS based L3/L1 measurement on SSB-less SCells need to be defined accordingly.

Cell DTX/DRX

According to RAN1, there is no agreement if the CSI-RS resources used for L3/L1/BFD/CBD will be transmitted during non-active periods of cell DTX/DRX hence the legacy behaviors will apply [2]. Since the CSI-RS transmission would not be impacted by cell DTX/DRX, the existing accuracy requirements for CSI-RS based L3/L1 measurements shall still apply.

Proposal 2: For CSI-RS based L3/L1 measurements, existing accuracy requirements shall apply irrespective of cell DTX/DRX.

As discussed in core part requirement [2-3], the SCell activation delay needs to be defined for activating a SSB-less operation pending on the side conditions. Besides, we further observe the impact on SCell activation delay due to cell DTX/DRX operation. Therefore, the test cases shall be defined at least to verify the SCell activation delay for both SSB-less operation and cell DTX/DRX operation.

Proposal 3: Test cases shall be defined at least to verify the SCell activation delay for both SSB-less operation and cell DTX/DRX operation.

Conclusion

In this paper we have made the following proposals and observations related to the RRM performance aspects:

Proposal 1: If the necessity of CSI-RS based L3/L1 measurement is concluded in core requirement, the accuracy requirements for the CSI-RS based L3/L1 measurement on SSB-less SCells need to be defined accordingly.

Proposal 2: For CSI-RS based L3/L1 measurements, existing accuracy requirements shall apply irrespective of cell DTX/DRX.

Proposal 3: Test cases shall be defined at least to verify the SCell activation delay for both SSB-less operation and cell DTX/DRX operation.

References

[1] RP-223540, New WID: Network energy savings for NR

[2] R1-2308949, Enhancement on cell DTX/DRX mechanism, Nokia, Nokia Shanghai Bell

[3] R4-2315520, SSB-less operation for FR1 inter-band co-located CA, Nokia, Nokia Shanghai Bell