3GPP TSG-RAN WG4 Meeting # 108bis	R4-2315526

# Xiamen, China, 09 – 13 October, 2023

Source:	LG Electronics

Title:	Discussion on NR sidelink CA operation

Agenda item:	5.30.3.1

Document for:	Discussion

Introduction

In this contribution, we provide our views on the RRM requirements for NR SL CA based on approved WF[1].

Discussion

For interruption location of the interruption to WAN due to SL carrier addition/release, there is no need to define the exact interruption location since mode 2 SL CA operation is only considered as follow.

Issue 1-1: Interruption location of the interruption to WAN at SL carrier addition/release - Agreements - For Mode 2 SL CA operation do not specify the exact interruptions location - FFS if constraints on interruptions during WAN paging or SIB reception shall be introduced

For FFS part, the interruption due to SL carrier addition/release could be consider not to apply while receving paing or receiving system information for mode 2 SL CA operation similar to existing interruption requirement for SL-DRX active to inactive state transition.

- Proposal 1: Add the constraints on interruptions during WAN paging or system information reception

For selection/reselection of sync reference source under SL CA operation, RAN4 agreed to reuse LTE requirements if  additional RAN1/2 agreements on SL CA operation does not impact RRM.

Issue 1-3: Requirements of Selection / Reselection of Synchronization Reference Source under NR SL CA operation - Agreements - Reuse LTE TS 36.133 clause 13.10 RRM requirements of V2X (re)selection of Synchronization reference source for NR SL CA operation. - Note: the agreement can be revisited in case of additional RAN1/2 agreements on NR SL CA operation

Based on RAN1/RAN2 agreements for SL CA operation, exsiting synchronization procedure is reused and no enhancements are observed. Therefore, selection/reselection of sync reference source for SL CA operation can be reused for NR SL CA operation.

- Proposal 2: Confirm to reuse LTE TS 36.133 clause 13.10 RRM requirements of V2X (re)selection of Synchronization reference source for NR SL CA operation

Conclusion

In this contribution, we provide views on NR sidelink CA operation, and, and propose

- Proposal 1: Add the constraints on interruptions during WAN paging or system information reception

- Proposal 2: Confirm to reuse LTE TS 36.133 clause 13.10 RRM requirements of V2X (re)selection of Synchronization reference source for NR SL CA operation

Reference

[1] R4-2314350, “WF on R18 NR SL RRM requirements (part 2),” OPPO.