3GPP TSG-RAN WG4 Meeting # 108bis			            	R4-2315528

Xiamen, China, Oct 9 – Oct 13, 2023

Source:	LG Electronics

Title:	UE RF requirements of con-current operation on Uu and sidelink

Agenda item:	5.30.2.2

Document for:	Discussion

Introduction

In this paper, we provide our views on the remaining UE RF requirements of con-current operation on Uu and sidelink based on the WF [1].

Discussion

Regarding the WF [1], the current status can be summarized in Table 2.1.

Table 2.1: Status of issues for con-current operation on Uu and sidelink

| Issue | Status |
| --- | --- |
| 1-1-1: Limited number of example band combination for Uu@Licensed and SL@Un-licensed | Agreed |
| 1-1-2: Example band combination for Uu@Licensed and SL@Un-licensed | Agreed |
| 1-2-1: Con-current operation of Uu@Un-licensed and SL@Un-licensed | Agreed |
| 1-2-1: Con-current operation of Uu@Un-licensed and SL@licensed | Agreed |
| 1-2-3: Con-current operation of intra-band SL@Un-licensed | Agreed |
| 1-3-1: Basis of SL-U for con-current operation | Agreed |
| 1-3-2: Applicable channel bandwidth for con-current operation of Uu@Licensed and SL@Un-licensed | Agreed |
| 1-3-3: RF architecture reference of Uu@Licensed and SL@Un-licensed | Agreed |
| 1-4-0: MOP for Uu@Licensed and SL@Un-licensed | FFS |
| 1-4-1: Targeted power class for Uu@Licensed and SL@Un-licensed | Agreed |
| 1-4-2: MPR/A-MPR for Uu@Licensed and SL@Un-licensed | Agreed |
| 1-4-3: Configured transmitted power for Uu@Licensed and SL@Un-licensed | Initial agreement |
| 1-4-4: UE coexistence requirements for Uu@Licensed and SL@Un-licensed | Agreed |
| 1-4-5: General spurious emission for Uu@Licensed and SL@Un-licensed | Agreed |
| 1-4-6: Additional spurious emission for Uu@Licensed and SL@Un-license | Agreed |
| 1-4-7: Other Tx requirements for Uu@Licensed and SL@Un-licensed | Agreed |
| 1-5-1: MSD for Uu@Licensed and SL@Un-licensed | Agreed |
| 1-5-2: Other Rx requirements for Uu@Licensed and SL@Un-licensed | Agreed |

The remained issues are as follows.

1-1-1: MOP for Uu@Licensed and SL@Un-licensed

  - Further discuss this issue with Issue 1-1-2 as a whole package in the next meeting

    - Option 1: Defines MOP per UE as the total transmitted power from each operating band

    - Option 2: Define MOP per each operating band in Rel-18

1-1-2: Configured transmitted power for Uu@Licensed and SL@Un-licensed

  - Further discuss in next meeting

    - Option 1: Apply the principle of the configured transmitted power of NR inter-band CA for the total UE configured maximum output power PCMAX (p,q) in a slot p of NR uplink carrier and a slot q of NR sidelink

      - PCMAX_L (p,q) =  MIN {10log10 [pCMAX_L,c,NR (p)+ pCMAX_L,c,SL(q)], PPowerClass_CA, PEMAX,CA}

      - PCMAX_H (p,q) = MIN {10 log10 [pCMAX_H,c,NR (p) + pCMAX_H,c,SL(q)], PPowerClass_CA, PEMAX,CA}

    - Option 2: Consider the maximum total transmit power (e.g., PEMAX,con-current)

    - Option 3: Follow the same approach as Rel-17 inter-band concurrent operation

For 1-1-1,

RAN4 agreed with n78@licensed band + n46@unlicensed band as the example band combination.

- Power class 3 (23dBm) for n78 + Power class 5 (20dBm) for n46

Total maximum output power is about 24.7dBm. It can make SAR issue. Because the use case is not limited to V2X. In V2X case, it was free from SAR.

It can be easily solved by defining MOP as total maximum output power, for example, PC3. It is not new way. In 3Tx case, MOP is defined as total maximum output power over all component carrier. For example, MOP = 26dBm for PC2@cc1 + PC3@cc2. Therefore, the same approach can be applied.

Proposal 1: Define MOP as the total transmitted power from each operating band of Uu and SL.

For 1-1-2,

Option 1 and Option 2 seem not be different option.  Option 2 can be covered with option1. Option 1 says only the principle. Based on the principle, the detail needs to be updated as follows.

- PCMAX_L (p,q) =  MIN {10log10 [pCMAX_L,c,NR (p) + pCMAX_L,c,SL(q)], PPowerClass,con-current, PEMAX,con-current}

- PCMAX_H (p,q) = MIN {10 log10 [pCMAX_H,c,NR (p) + pCMAX_H,c,SL(q)], PPowerClass,con-current, PEMAX,con-current}

Here,

- PPowerClass,con-current is the maximum UE power specified as MOP.

- PEMAX,con-current is the value indicated by p-NR-FR1 or by p-UE-FR1 whichever is the smallest if both are present

For option 3, same problem of SAR is not solved for other use case than V2X.

Proposal 2: Consider option 1 as principle of configured transmitted power and update the detail as follows.

- PCMAX_L (p,q) =  MIN {10log10 [pCMAX_L,c,NR (p) + pCMAX_L,c,SL(q)], PPowerClass,con-current, PEMAX,con-current}

- PCMAX_H (p,q) = MIN {10 log10 [pCMAX_H,c,NR (p) + pCMAX_H,c,SL(q)], PPowerClass,con-current, PEMAX,con-current}

Conclusion

In this contribution, we provided our view on the remaining UE RF requirements of con-current operation on Uu and SL-U. Based on those, the followings are proposed.

Proposal 1: Define MOP as the total transmitted power from each operating band of Uu and SL.

Proposal 2: Consider option 1 as principle of configured transmitted power and update the detail as follows.

- PCMAX_L (p,q) =  MIN {10log10 [pCMAX_L,c,NR (p) + pCMAX_L,c,SL(q)], PPowerClass,con-current, PEMAX,con-current}

- PCMAX_H (p,q) = MIN {10 log10 [pCMAX_H,c,NR (p) + pCMAX_H,c,SL(q)], PPowerClass,con-current, PEMAX,con-current}

- References

[1] R4-2314738, “WF on NR_SL_enh2_UERF_part2”, LG Electronics