3GPP TSG-RAN WG4 Meeting # 108bis					R4-2315530

Xiamen, China, Oct 9 – Oct 13, 2023

Source:	LG Electronics

Title:	TP to TR on con-current operation on Uu and sidelink

Agenda item:	5.30.2.2

Document for:	Approval

Background

This contribution provides text proposal on MOP for the con-current operation on Uu and sidelink.

Text Proposal

- <<<<<<<<<<< Start of changes in section 6.2.1 >>>>>>>>>>

6.2.1	Maximum output power for inter-band con-current operation

It has been agreed to target the following power classes.

  - Power class 3 Uu@Licensed + Power class 5 SL@Un-licensed

  - Power class 3 Uu@Licensed + Power class 3 SL@Un-licensed can be considered as 2nd priority if Power class 3 SL is agreed in a single carrier at unlicensed band

For Power class 2 for Uu@Licensed for inter-band concurrent operation, it is not considered in Rel-18.

For the inter-band con-current operation, UE maximum output power shall be measured over all component carriers from different bands. If each band has separate antenna connectors, maximum output power is defined as the sum of maximum output power from each UE antenna connector. The period of measurement shall be at least one sub frame (1 ms). The maximum output power is specified in Table 6.2.1-1.

Table 6.2.1-1: Power Class for inter-band con-current combination (two bands)

| Inter-band con-current operating band Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SL_n78A-n46A |  |  |  |  | 23 | ±2/-3 |  |  |

- <<<<<<<<<<< End of changes in section 6.2.1 >>>>>>>>>>

References

[1] R4-2315528, “UE RF requirements of con-current operation on Uu and sidelink”, LG Electronics

[2] TR 38.786 v1.0.0