3GPP TSG-RAN WG4 Meeting # 108bis                      				R4-2315532

# Xiamen, China, October 09 – October 13, 2023

Source:	LG Electronics

Title:	UE RF requirements of SL CA

Agenda item:	5.30.2.3

Document for:	Discussion

Introduction

In this paper, we provide our views on the remaining issues of UE RF requirements and SL CA MPR/A-MPR simulation results for intra-band SL CA in Band n47 based on the WF[1].

Discussion

The current status can be summarized in Table 2-1.

Table 2-1: Status of issues for intra-band SL CA in Band n47

| Issue | Status |
| --- | --- |
| 1-1: Spec suffix | Agreed |
| 1-2: Notation for NR SL CA | Agreed |
| 1-3: SCS for NR SL CA | Agreed |
| 2-1: Channel bandwidth class and aggregated channel BW for sidelink CA | Agreed |
| 2-2: The maximum aggregated CBW for sidelink CA | Agreed |
| 2-3: Configurations for sidelink CA | Agreed |
| 3-1: UE maximum output power for SL CA | Agreed |
| 3-2: MPR/A-MPR simulation assumptions for SL CA | Initial agreement |
| 3-3: Configured transmitted power for SL intra-band contiguous CA | Agreed |
| 3-4: Methodology on Tx requirements for SL intra-band contiguous CA | FFS |
| 4: Rx RF Requirements for SL CA | Agreed |

Issues to be clarified

Issue 2-1: Methodology on Tx requirements for SL intra-band contiguous CA

  - WF: FFS: Take the requirements for

    - LTE V2X intra-band contiguous CA or LTE intra-band contiguous CA

    - NR intra-band contiguous CA

as baseline.

  - Encourage companies to provide results based on the preferred methodology. Make decision for the suitable manner for SL CA in next meeting based on the inputs

It is preferred to take NR intra-band contiguous CA as methodology on Tx requirements for SL intra-band contiguous CA. It is better to align the methodology of NR intra-band contiguous CA, because there is no big difference between NR intra-band contiguous CA and SL intra-band contiguous CA. It is more easy for reader to understand the specification.

Proposal 1: Take NR intra-band contiguous CA as methodology on Tx requirements for SL intra-band contiguous CA.

Issue 2-3: Configured transmitted power for sidelink intra-band CA

  - WF: FFS the baseline for configured transmitted power for sidelink intra-band CA

    - LTE V2X intra-band contiguous CA

    - LTE intra-band contiguous CA

    - NR intra-band contiguous CA

It is preferred to take NR intra-band contiguous CA as baseline for configured transmitted power of SL intra-band contiguous CA.

For supporting the SL intra-band contiguous CA, a capability of power class per band combination needs to be defined as NR intra-band CA, for example, ‘powerClassSidelink-r18’. It can be indicated by power class 3 and power class 2. Power class 3 can be default. The corresponding terminology in RAN4 specification can be considered with ‘PPowerClass,SL-CA’.

Sidelink CA parameters in NR

Table 2.1-1: Capability of SL CA power class

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| powerClassSidelink-r18 This parameter indicates power class the UE supports when operating according to this band combination used for sidelink. If the field is absent, the UE supports the default power class. If this power class is higher than the power class that the UE supports on the individual bands of this band combination (ue-PowerClassSidelink-r16 in BandNR), the latter determines maximum TX power available in each band. The UE sets the power class parameter only in band combinations that are applicable as specified in TS 38.101-1 [2] and TS 38.101-3 [4]. | BC | No | N/A | FR1 only |

Another issue is for PEMAX,CA for SL CA which was agreed in RAN4# 106bis as follows.

- Specify PEMAX,CA for SL CA

  - PEMAX,CA is not applied to S-SSB

Based on the agreement, the following PCMAX,L and PCMAX,H for SL CA can be considered as one example.

-  PCMAX_L  = MIN{10 log10 ∑ pEMAX,c , PEMAX,CA, (PPowerClass,SL-CA– ΔPPowerClass,SL-CA) – MAX(MAX(MPR, A-MPR) + ΔTIB,c, P-MPR ), PRegulatory }

-  PCMAX_H  = MIN{10 log10 ∑ pEMAX,c , PEMAX, CA ,PPowerClass,SL-CA– ΔPPowerClass,SL-CA , PRegulatory }

However, so far, it is not clear what is the corresponding IE. The PEMAX,CA for SL CA should be differentiated from PEMAX,c . PEMAX,c  is the value given by  IE sl-maxTransPower, defined by TS 38.331.

  - IE sl-maxTransPower  : indicates the maximum value of the UE’s sidelink transmission power on this resource pool.

The corresponding IE to PEMAX,CA for SL CA should be defined by RAN2 specification.

Proposal 2: Consider NR intra-band contiguous CA as baseline for configured transmitted power for sidelink intra-band CA.

Proposal 3: Consider the terminology of  ’PPowerClass,SL-CA’ for power class of SL intra-band contiguous CA.

Proposal 4: Define a capability of power class per band combination for supporting SL intra-band contiguous CA as Table 2.1-1.

Proposal 5: Send LS to RAN2  on necessity of a capability of power class per band combination and a new IE which corresponds to PEMAX,CA for supporting SL intra-band contiguous CA.

MPR for SLCA PSSCH/PSCCH

For SL CA PSSCH/PSCCH MPR, the existing RB allocations which are defined in the NR contiguous CA MPR can be reused.

- Inner/Outer RB allocation in NR C-CA with contiguous RB allocations

- Inner/Outer1/Outer2 RB allocation in NR C-CA with non-contiguous RB allocations

Proposal 6: Reuse Inner/Outer RB allocation and Inner/Outer1/Outer2 RB allocation of NR C-CA for SL C-CA PSSCH/PSCCH MPR.

The following simulation assumption is considered in the WF[1].

Center frequency; 5.9GHz. Bandwidth; per CC: 10/20/30/40MHz aggregated CBW: 10+10MHz/20+30MHz/ 30+40MHz. Maximum output power for aggregated CBW; 23dBm. Numerology; 15 kHz/30kHz/60kHz. Modulation per CC; QPSK/16QAM/64QAM/256QAM. Waveform; CP-OFDM. Carrier leakage; 25dBc. IQ image; 34dBc. CIM3; 60dBc. PA calibration; PA calibrated to deliver 30dBc ACLR for a fully allocated RBs in 20MHz QPSK DFT- S-OFDM waveform at 1 dB MPR. This is based to share PA between LTE V2X and NR V2X at 5.9GHz as worst case.. In addition, 20+40MHz, 20+20MHz, and 10+30MHz are also considered.

For Contiguous RB allocation of SL contiguous CA

The following simulation scenarios are considered as Table 2.2-1 for Contiguous RB allocations.

Table 2.2-1: SLCA PSSCH/PSCCH MPR simulation scenarios (Contiguous RB allocation)

| Aggregated CBW | Scenario | CC1 | CC2 | Inner/Outer RB allocation | SCS |
| --- | --- | --- | --- | --- | --- |
| 10MHz + 10MHz | 1 | 10RB42 | 10RB0 | Inner | 15 |
|  | 2 | 10RB42 | 12RB0 | Inner | 15 |
|  | 3 | 10RB42 | 15RB0 | Inner | 15 |
|  | 4 | 10RB42 | 25RB0 | Inner | 15 |
|  | 5 | 10RB42 | 30RB0 | Inner | 15 |
|  | 6 | 10RB42 | 36RB0 | Outer | 15 |
|  | 7 | 10RB42 | 50RB0 | Outer | 15 |
|  | 8 | 12RB40 | 36RB0 | Outer | 15 |
|  | 9 | 25RB27 | 36RB0 | Outer | 15 |
|  | 10 | 40RB12 | 40RB0 | Outer | 15 |
|  | 11 | 50RB2 | 50RB0 | Outer | 15 |
| 20MHz + 30MHz | 12 | 10RB41 | 10RB0 | Inner | 30 |
|  | 13 | 10RB41 | 12RB0 | Inner | 30 |
|  | 14 | 10RB41 | 25RB0 | Inner | 30 |
|  | 15 | 10RB41 | 30RB0 | Inner | 30 |
|  | 16 | 10RB41 | 48RB0 | Inner | 30 |
|  | 17 | 10RB41 | 50RB0 | Outer | 30 |
|  | 18 | 25RB26 | 36RB0 | Outer | 30 |
|  | 19 | 36RB15 | 36RB0 | Outer | 30 |
|  | 20 | 40RB11 | 40RB0 | Outer | 30 |
|  | 21 | 50RB1 | 75RB0 | Outer | 30 |
| 20MHz + 40MHz | 22 | 10RB41 | 10RB0 | Inner | 30 |
|  | 23 | 10RB41 | 25RB0 | Inner | 30 |
|  | 24 | 10RB41 | 36RB0 | Inner | 30 |
|  | 25 | 10RB41 | 48RB0 | Inner | 30 |
|  | 26 | 10RB41 | 60RB0 | Inner | 30 |
|  | 27 | 10RB41 | 70RB0 | Outer | 30 |
|  | 28 | 25RB26 | 36RB0 | Outer | 30 |
|  | 29 | 36RB15 | 70RB0 | Outer | 30 |
|  | 30 | 40RB11 | 90RB0 | Outer | 30 |
|  | 31 | 50RB1 | 105RB0 | Outer | 30 |
| 30MHz + 40MHz | 32 | 10RB68 | 10RB0 | Inner | 30 |
|  | 33 | 10RB68 | 25RB0 | Inner | 30 |
|  | 34 | 10RB68 | 36RB0 | Inner | 30 |
|  | 35 | 10RB68 | 48RB0 | Inner | 30 |
|  | 36 | 10RB68 | 60RB0 | Inner | 30 |
|  | 37 | 10RB68 | 70RB0 | Outer | 30 |
|  | 38 | 10RB68 | 105RB0 | Outer | 30 |
|  | 39 | 25RB53 | 70RB0 | Outer | 30 |
|  | 40 | 36RB42 | 90RB0 | Outer | 30 |
|  | 41 | 40RB38 | 90RB0 | Outer | 30 |
|  | 42 | 75RB3 | 105RB0 | Outer | 30 |
| 20MHz + 20MHz | 43 | 10RB41 | 10RB0 | Inner | 30 |
|  | 44 | 10RB41 | 12RB0 | Inner | 30 |
|  | 45 | 10RB41 | 15RB0 | Inner | 30 |
|  | 45 | 10RB41 | 25RB0 | Inner | 30 |
|  | 46 | 10RB41 | 30RB0 | Inner | 30 |
|  | 47 | 10RB41 | 36RB0 | Outer | 30 |
|  | 48 | 10RB41 | 50RB0 | Outer | 30 |
|  | 49 | 12RB39 | 36RB0 | Outer | 30 |
|  | 50 | 25RB26 | 36RB0 | Outer | 30 |
|  | 51 | 40RB11 | 40RB0 | Outer | 30 |
|  | 52 | 50RB1 | 50RB0 | Outer | 30 |
| 10MHz + 30MHz | 53 | 10RB41 | 10RB0 | Inner | 30 |
|  | 54 | 10RB41 | 12RB0 | Inner | 30 |
|  | 55 | 10RB41 | 15RB0 | Inner | 30 |
|  | 56 | 10RB41 | 20RB0 | Inner | 30 |
|  | 57 | 10RB41 | 25RB0 | Outer | 30 |
|  | 58 | 10RB41 | 50RB0 | Outer | 30 |
|  | 59 | 10RB41 | 75RB0 | Outer | 30 |
|  | 60 | 12RB12 | 36RB0 | Outer | 30 |
|  | 61 | 15RB9 | 60RB0 | Outer | 30 |
|  | 62 | 20RB4 | 75RB0 | Outer |  |
|  | 63 | 24RB0 | 75RB0 | Outer | 30 |

Table 2.2-2 and Figure 2.2-1 show PSSCH/PSCCH MPR simulation results.

Table 2.2-2: PSSCH/PSCCH MPR simulation results for SL Contiguous CA with Contiguous RB allocations

| '10MHz+10MHz' | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 | #11 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 'QPSK' | 0.13 | 0.13 | 0.14 | 0.13 | 0.13 | 0.95 | 2.70 | 1.27 | 1.60 | 2.32 | 3.09 |  |  |  |  |  |  |  |
|  | '16QAM' | 1.27 | 0.96 | 1.27 | 0.95 | 0.95 | 0.96 | 2.69 | 1.27 | 1.60 | 2.32 | 3.09 |  |  |  |  |  |  |  |
|  | '64QAM' | 2.31 | 2.31 | 2.31 | 2.31 | 2.30 | 2.31 | 2.69 | 2.31 | 2.31 | 2.32 | 3.09 |  |  |  |  |  |  |  |
|  | '256QAM' | 4.36 | 3.92 | 3.93 | 3.92 | 3.92 | 3.92 | 3.92 | 3.92 | 3.92 | 3.92 | 3.92 |  |  |  |  |  |  |  |
| '20MHz+30MHz' | Scenario # | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 | #21 |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 0.37 | 0.65 | 1.26 | 0.96 | 0.67 | 0.95 | 0.96 | 1.95 | 2.31 | 3.09 |  |  |  |  |  |  |  |  |
|  | '16QAM' | 0.66 | 0.94 | 0.95 | 0.95 | 1.27 | 1.27 | 1.27 | 1.95 | 2.31 | 3.09 |  |  |  |  |  |  |  |  |
|  | '64QAM' | 1.93 | 1.93 | 2.31 | 2.31 | 2.31 | 2.31 | 2.31 | 2.31 | 2.31 | 3.09 |  |  |  |  |  |  |  |  |
|  | '256QAM' | 3.91 | 3.91 | 3.92 | 3.92 | 3.92 | 3.92 | 3.91 | 3.92 | 3.92 | 3.92 |  |  |  |  |  |  |  |  |
| '20MHz+40MHz' | Scenario # | #22 | #23 | #24 | #25 | #26 | #27 | #28 | #29 | #30 | #31 |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 0.00 | 0.00 | 0.00 | 0.13 | 0.13 | 0.39 | 0.66 | 1.95 | 2.31 | 3.09 |  |  |  |  |  |  |  |  |
|  | '16QAM' | 0.65 | 0.95 | 0.96 | 0.95 | 0.96 | 0.96 | 0.95 | 1.94 | 2.31 | 3.09 |  |  |  |  |  |  |  |  |
|  | '64QAM' | 1.93 | 1.94 | 2.30 | 2.31 | 2.31 | 2.31 | 2.30 | 2.31 | 2.31 | 3.09 |  |  |  |  |  |  |  |  |
|  | '256QAM' | 3.49 | 3.91 | 3.92 | 3.92 | 3.92 | 3.92 | 3.92 | 3.92 | 3.92 | 3.92 |  |  |  |  |  |  |  |  |
| '30MHz+40MHz' | Scenario # | #32 | #33 | #34 | #35 | #36 | #37 | #38 | #39 | #40 | #41 | #43 |  |  |  |  |  |  |  |
|  | 'QPSK' | 0.00 | 0.00 | 0.00 | 0.12 | 0.13 | 0.66 | 3.09 | 1.27 | 2.69 | 2.69 | 3.09 |  |  |  |  |  |  |  |
|  | '16QAM' | 0.64 | 0.95 | 0.95 | 0.95 | 1.27 | 1.27 | 3.09 | 1.27 | 2.69 | 2.69 | 3.09 |  |  |  |  |  |  |  |
|  | '64QAM' | 1.93 | 2.30 | 2.30 | 2.30 | 2.30 | 2.30 | 3.09 | 2.31 | 2.69 | 2.69 | 3.09 |  |  |  |  |  |  |  |
|  | '256QAM' | 3.49 | 3.91 | 3.91 | 3.91 | 4.35 | 3.92 | 4.36 | 3.92 | 4.36 | 3.92 | 4.35 |  |  |  |  |  |  |  |
| '20MHz+20MHz' | Scenario # | #44 | #45 | #46 | #47 | #48 | #49 | #50 | #51 | #52 | #53 | #54 |
|  | 'QPSK' | 0.00 | 0.00 | 0.00 | 0.00 | 0.13 | 1.26 | 3.09 | 1.59 | 1.94 | 2.31 | 3.09 |
|  | '16QAM' | 0.65 | 0.95 | 0.95 | 0.95 | 0.95 | 1.27 | 3.09 | 1.59 | 1.95 | 2.32 | 3.09 |
|  | '64QAM' | 1.93 | 1.93 | 1.94 | 2.30 | 2.31 | 2.31 | 3.08 | 2.31 | 2.31 | 2.31 | 3.09 |
|  | '256QAM' | 3.91 | 3.91 | 3.92 | 3.91 | 3.92 | 3.92 | 3.92 | 3.92 | 3.92 | 3.92 | 3.92 |
| '10MHz+30MHz' | Scenario # | #55 | #56 | #57 | #58 | #59 | #60 | #61 | #62 | #63 | #64 | #65 |
|  | 'QPSK' | 0.00 | 0.00 | 0.00 | 0.38 | 0.66 | 1.60 | 2.69 | 1.60 | 2.31 | 2.70 | 3.09 |
|  | '16QAM' | 0.65 | 0.94 | 0.65 | 0.94 | 0.95 | 1.60 | 2.69 | 1.60 | 2.31 | 2.70 | 3.09 |
|  | '64QAM' | 1.93 | 1.94 | 1.95 | 1.94 | 2.31 | 2.30 | 2.69 | 2.31 | 2.31 | 2.70 | 3.09 |
|  | '256QAM' | 3.92 | 3.91 | 3.91 | 3.92 | 3.92 | 3.92 | 4.36 | 3.92 | 3.92 | 3.92 | 3.92 |

- 10MHz + 10MHz

- 20MHz + 30MHz

- 20MHz + 40MHz

- 30MHz + 40MHz

- 20MHz + 20MHz

- 10MHz + 30MHz

Figure 2.2-1: PSSCH/PSCCH MPR simulation results for SL Contiguous CA with Contiguous RB allocations

Table 2.2-3 shows the maximum value of simulation results for SL Contiguous CA with Contiguous RB allocations considering Inner RB allocation and Outer RB allocation as NR uplink Contiguous CA.

Table 2.2-3: PSSCH/PSCCH MPR simulation results for SL Contiguous CA with Contiguous RB allocations

| Modulation |  | MPR for bandwidth class B(dB) |  |
| --- | --- | --- | --- |
|  |  | inner | outer |
| CP-OFDM | QPSK | 0.38 | 3.09 |
|  | 16QAM | 1.27 | 3.09 |
|  | 64QAM | 2.31 | 3.09 |
|  | 256QAM | 4.36 | 4.36 |

The MPR can be proposed as Table 2.2-4 based on the simulation results when considering implementation margin.

Table 2.2-4: PSSCH/PSCCH MPR simulation results for SL Contiguous CA with Contiguous RB allocations

| Modulation |  | MPR for bandwidth class B(dB) |  |
| --- | --- | --- | --- |
|  |  | inner | outer |
| CP-OFDM | QPSK | ≤ 3.0 | ≤ 5.0 |
|  | 16QAM | ≤ 3.0 | ≤ 5.0 |
|  | 64QAM | ≤ 4.5 | ≤ 5.0 |
|  | 256QAM | ≤ 7.0 | ≤ 7.0 |

Proposal 7: For SL intra-band C-CA PSSCH/PSCCH MPR, consider Table 2.2-4 for contiguous RB allocation.

For Non-contiguous RB allocation of SL contiguous CA

The following test scenarios are considered as Table 2.2-5 for Non-contiguous RB allocations.

Table 2.2-5: SL contiguous CA MPR test scenarios (Non-contiguous RB allocation)

| Aggregated CBW | Scenario | CC1 | CC2 | Inner/Outer1/Outer2 RB allocation | SCS |
| --- | --- | --- | --- | --- | --- |
| 10MHz + 10MHz | 1 | 10RB0 | 10RB42 | Outer2 | 15 |
|  | 2 | 10RB18 | 10RB22 | Outer2 | 15 |
|  | 3 | 10RB19 | 10RB22 | Outer2 | 15 |
|  | 4 | 10RB27 | 10RB14 | Outer2 | 15 |
|  | 5 | 10RB33 | 10RB7 | Outer1 | 15 |
|  | 6 | 10RB34 | 10RB7 | Outer1 | 15 |
|  | 7 | 10RB42 | 10RB1 | Inner | 15 |
|  | 8 | 25RB0 | 25RB27 | Outer2 | 15 |
|  | 9 | 25RB19 | 25RB8 | Outer2 | 15 |
|  | 10 | 25RB20 | 25RB8 | Outer2 | 15 |
|  | 11 | 25RB27 | 25RB1 | Outer1 | 15 |
|  | 12 | 30RB0 | 30RB22 | Outer2 | 15 |
|  | 13 | 30RB19 | 30RB3 | Outer2 | 15 |
|  | 14 | 30RB19 | 30RB2 | Outer2 | 15 |
|  | 15 | 30RB22 | 30RB1 | Outer1 | 15 |
|  | 16 | 50RB1 | 50RB0 | Outer2 | 15 |
| 20MHz + 30MHz | 17 | 10RB0 | 10RB68 | Outer2 | 30 |
|  | 18 | 10RB15 | 10RB32 | Outer2 | 30 |
|  | 19 | 10RB15 | 10RB31 | Outer2 | 30 |
|  | 20 | 10RB32 | 10RB10 | Outer1 | 30 |
|  | 21 | 10RB34 | 10RB9 | Outer1 | 30 |
|  | 22 | 10RB34 | 10RB8 | Outer1 | 30 |
|  | 23 | 10RB41 | 10RB1 | Inner | 30 |
|  | 24 | 25RB0 | 25RB53 | Outer2 | 30 |
|  | 25 | 25RB20 | 25RB25 | Outer2 | 30 |
|  | 26 | 25RB21 | 25RB25 | Outer2 | 30 |
|  | 27 | 25RB26 | 25RB1 | Outer1 | 30 |
|  | 28 | 30RB0 | 30RB48 | Outer2 | 30 |
|  | 29 | 30RB15 | 30RB12 | Outer2 | 30 |
|  | 30 | 30RB15 | 30RB11 | Outer2 | 30 |
|  | 31 | 30RB21 | 30RB1 | Outer1 | 30 |
|  | 32 | 50RB0 | 50RB28 | Outer2 | 30 |
|  | 33 | 50RB0 | 50RB0 | Outer2 | 30 |
|  | 34 | 50RB0 | 78RB0 | Outer2 | 30 |
| 20MHz + 40MHz | 35 | 10RB0 | 10RB96 | Outer2 | 30 |
|  | 36 | 10RB4 | 10RB30 | Outer2 | 30 |
|  | 37 | 10RB5 | 10RB30 | Outer2 | 30 |
|  | 38 | 10RB37 | 10RB15 | Outer1 | 30 |
|  | 39 | 10RB38 | 10RB15 | Inner | 30 |
|  | 40 | 10RB41 | 10RB1 | Inner | 30 |
|  | 41 | 25RB0 | 25RB81 | Outer2 | 30 |
|  | 42 | 25RB20 | 25RB38 | Outer2 | 30 |
|  | 43 | 25RB20 | 25RB37 | Outer2 | 30 |
|  | 44 | 25RB26 | 25RB1 | Outer1 | 30 |
|  | 45 | 30RB0 | 30RB76 | Outer2 | 30 |
|  | 46 | 30RB15 | 30RB26 | Outer2 | 30 |
|  | 47 | 30RB15 | 30RB25 | Outer2 | 30 |
|  | 48 | 30RB21 | 30RB1 | Outer1 | 30 |
|  | 49 | 50RB0 | 50RB56 | Outer2 | 30 |
|  | 50 | 50RB0 | 50RB0 | Outer2 | 30 |
|  | 51 | 50RB0 | 105RB0 | Outer2 | 30 |
| 30MHz + 40MHz | 52 | 10RB0 | 10RB96 | Outer2 | 30 |
|  | 53 | 10RB13 | 10RB30 | Outer2 | 30 |
|  | 54 | 10RB14 | 10RB30 | Outer1 | 30 |
|  | 55 | 10RB50 | 10RB15 | Outer1 | 30 |
|  | 56 | 10RB51 | 10RB15 | Inner | 30 |
|  | 57 | 10RB68 | 10RB1 | Inner | 30 |
|  | 58 | 25RB0 | 25RB81 | Outer2 | 30 |
|  | 59 | 25RB28 | 25RB37 | Outer2 | 30 |
|  | 60 | 25RB28 | 25RB36 | Outer1 | 30 |
|  | 61 | 25RB51 | 25RB1 | Outer1 | 30 |
|  | 62 | 25RB52 | 25RB1 | Inner | 30 |
|  | 63 | 30RB0 | 30RB76 | Outer2 | 30 |
|  | 64 | 30RB24 | 30RB26 | Outer2 | 30 |
|  | 65 | 30RB24 | 30RB25 | Outer1 | 30 |
|  | 66 | 30RB48 | 30RB1 | Outer1 | 30 |
|  | 67 | 75RB0 | 50RB56 | Outer2 | 30 |
|  | 68 | 75RB0 | 50RB0 | Outer2 | 30 |
|  | 69 | 75RB0 | 105RB0 | Outer2 | 30 |
| 20MHz + 20MHz | 70 | 10RB0 | 10RB41 | Outer2 | 30 |
|  | 71 | 10RB17 | 10RB22 | Outer2 | 30 |
|  | 72 | 10RB18 | 10RB22 | Outer1 | 30 |
|  | 73 | 10RB27 | 10RB14 | Outer1 | 30 |
|  | 74 | 10RB33 | 10RB7 | Outer1 | 30 |
|  | 75 | 10RB34 | 10RB7 | Inner | 30 |
|  | 76 | 10RB41 | 10RB1 | Inner | 30 |
|  | 77 | 25RB0 | 25RB26 | Outer2 | 30 |
|  | 78 | 25RB18 | 25RB8 | Outer2 | 30 |
|  | 79 | 25RB19 | 25RB8 | Outer1 | 30 |
|  | 80 | 25RB26 | 25RB1 | Outer1 | 30 |
|  | 81 | 30RB0 | 30RB21 | Outer2 | 30 |
|  | 82 | 30RB18 | 30RB3 | Outer1 | 30 |
|  | 83 | 30RB18 | 30RB2 | Outer1 | 30 |
|  | 84 | 30RB21 | 30RB1 | Outer2 | 30 |
|  | 85 | 50RB0 | 50RB0 | Outer2 | 30 |
| 10MHz + 30MHz | 86 | 10RB0 | 10RB68 | Outer2 | 30 |
|  | 87 | 10RB6 | 10RB32 | Outer2 | 30 |
|  | 88 | 10RB6 | 10RB31 | Outer1 | 30 |
|  | 89 | 10RB14 | 10RB10 | Outer1 | 30 |
|  | 90 | 10RB14 | 10RB1 | Outer1 | 30 |
|  | 91 | 10RB0 | 12RB53 | Outer2 | 30 |
|  | 92 | 10RB3 | 12RB25 | Outer2 | 30 |
|  | 93 | 25RB4 | 12RB25 | Outer1 | 30 |
|  | 94 | 25RB12 | 12RB1 | Outer1 | 30 |
|  | 95 | 25RB0 | 30RB48 | Outer2 | 30 |
|  | 96 | 25RB3 | 30RB7 | Outer2 | 30 |
|  | 97 | 30RB3 | 30RB6 | Outer1 | 30 |
|  | 98 | 30RB12 | 30RB1 | Outer1 | 30 |
|  | 99 | 30RB0 | 50RB28 | Outer2 | 30 |
|  | 100 | 30RB0 | 50RB1 | Outer2 | 30 |
|  | 101 | 50RB0 | 75RB1 | Outer2 | 30 |

Table 2.2-6 and Figure 2.2-2 show PSSCH/PSCCH MPR simulation results.

Table 2.2-6: PSSCH/PSCCH MPR simulation results for SL Contiguous CA with Non-contiguous RB allocations

| '10MHz+10MHz' | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 | #11 | #12 | #13 | #14 | #15 | #16 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 'QPSK' | 6.67 | 3.09 | 2.69 | 2.33 | 0.00 | 0.00 | 0.14 | 4.81 | 2.32 | 2.32 | 0.14 | 4.81 | 1.95 | 1.95 | 1.28 | 3.09 |  |  |
|  | '16QAM' | 6.67 | 2.70 | 3.10 | 2.70 | 0.95 | 0.97 | 0.96 | 5.26 | 2.32 | 2.32 | 0.96 | 4.81 | 1.95 | 1.95 | 1.28 | 3.09 |  |  |
|  | '64QAM' | 6.67 | 2.69 | 3.09 | 2.32 | 1.94 | 2.31 | 2.31 | 5.26 | 2.31 | 2.32 | 2.31 | 4.81 | 1.95 | 1.95 | 2.32 | 3.09 |  |  |
|  | '256QAM' | 7.15 | 3.92 | 3.93 | 3.93 | 3.92 | 3.92 | 3.93 | 4.81 | 3.92 | 3.92 | 3.92 | 4.81 | 3.92 | 3.92 | 3.92 | 3.92 |  |  |
| '20MHz+30MHz' | Scenario # | #17 | #18 | #19 | #20 | #21 | #22 | #23 | #24 | #25 | #26 | #27 | #28 | #29 | #30 | #31 | #32 | #33 | #34 |
|  | 'QPSK' | 5.27 | 2.69 | 2.69 | 0.00 | 0.00 | 0.00 | 0.00 | 3.50 | 2.69 | 2.69 | 0.38 | 3.10 | 2.69 | 2.69 | 1.27 | 3.10 | 3.09 | 3.09 |
|  | '16QAM' | 5.27 | 2.70 | 2.69 | 0.66 | 0.66 | 0.65 | 0.66 | 3.50 | 2.70 | 2.69 | 0.96 | 3.09 | 2.69 | 2.69 | 1.27 | 3.09 | 3.09 | 3.09 |
|  | '64QAM' | 5.27 | 2.69 | 2.70 | 1.93 | 1.94 | 1.93 | 1.93 | 3.51 | 2.69 | 2.69 | 2.31 | 3.09 | 2.69 | 2.69 | 2.31 | 3.09 | 3.09 | 3.09 |
|  | '256QAM' | 5.26 | 3.50 | 3.49 | 3.48 | 3.91 | 3.49 | 3.49 | 3.50 | 3.50 | 3.49 | 3.91 | 3.92 | 3.92 | 3.92 | 3.92 | 3.92 | 3.92 | 3.92 |
| '20MHz+40MHz' | Scenario # | #35 | #36 | #37 | #38 | #39 | #40 | #41 | #42 | #43 | #44 | #45 | #46 | #47 | #48 | #49 | #50 | #51 |  |
|  | 'QPSK' | 5.28 | 3.09 | 3.08 | 0.00 | 0.00 | 0.00 | 3.09 | 2.69 | 2.69 | 0.66 | 3.09 | 2.69 | 2.69 | 1.60 | 3.10 | 3.09 | 3.09 |  |
|  | '16QAM' | 5.26 | 3.08 | 3.08 | 0.65 | 0.65 | 0.65 | 3.09 | 2.69 | 2.69 | 0.95 | 3.10 | 2.69 | 2.69 | 1.60 | 3.09 | 3.09 | 3.09 |  |
|  | '64QAM' | 5.27 | 3.09 | 3.09 | 1.94 | 1.94 | 1.94 | 3.09 | 2.69 | 2.69 | 1.94 | 3.09 | 2.69 | 2.69 | 2.31 | 3.10 | 3.09 | 3.09 |  |
|  | '256QAM' | 5.27 | 3.49 | 3.49 | 3.49 | 3.48 | 3.49 | 3.50 | 3.49 | 3.49 | 3.91 | 3.92 | 3.50 | 3.92 | 3.92 | 3.92 | 3.92 | 3.92 |  |
| '30MHz+40MHz' | Scenario # | #52 | #53 | #54 | #55 | #56 | #57 | #58 | #59 | #60 | #61 | #62 | #63 | #64 | #65 | #66 | #67 | #68 | #69 |
|  | 'QPSK' | 5.27 | 2.69 | 2.69 | 0.00 | 0.00 | 0.00 | 3.09 | 2.69 | 2.69 | 0.00 | 0.00 | 3.09 | 2.69 | 2.69 | 0.00 | 3.09 | 3.09 | 3.09 |
|  | '16QAM' | 5.26 | 2.69 | 2.69 | 0.65 | 0.65 | 0.65 | 3.10 | 2.69 | 2.69 | 0.95 | 0.94 | 3.09 | 2.69 | 2.69 | 0.95 | 3.09 | 3.09 | 3.09 |
|  | '64QAM' | 5.26 | 2.69 | 2.68 | 1.93 | 1.93 | 1.93 | 3.09 | 2.69 | 2.69 | 1.94 | 1.94 | 3.09 | 2.69 | 2.69 | 2.30 | 3.09 | 3.09 | 3.09 |
|  | '256QAM' | 5.27 | 3.48 | 3.49 | 3.48 | 3.49 | 3.49 | 3.50 | 3.49 | 3.49 | 3.91 | 3.91 | 3.92 | 3.92 | 3.92 | 3.91 | 3.92 | 3.92 | 4.36 |
| '20MHz+20MHz' | Scenario # | #70 | #71 | #72 | #73 | #74 | #75 | #76 | #77 | #78 | #79 | #80 | #81 | #82 | #84 | #84 | #85 |
|  | 'QPSK' | 5.27 | 2.69 | 2.69 | 2.68 | 0.00 | 0.00 | 0.00 | 3.50 | 2.31 | 2.31 | 0.39 | 3.09 | 1.95 | 1.95 | 1.60 | 3.09 |
|  | '16QAM' | 5.27 | 2.69 | 2.69 | 2.69 | 0.65 | 0.65 | 0.65 | 3.50 | 2.31 | 2.31 | 0.95 | 3.09 | 1.95 | 1.95 | 1.60 | 3.10 |
|  | '64QAM' | 5.26 | 2.69 | 2.69 | 2.68 | 1.93 | 1.94 | 1.93 | 3.50 | 2.31 | 2.31 | 1.94 | 3.09 | 1.95 | 1.95 | 2.31 | 3.09 |
|  | '256QAM' | 5.27 | 3.49 | 3.49 | 3.49 | 3.49 | 3.49 | 3.49 | 3.50 | 3.92 | 3.92 | 3.92 | 3.92 | 3.92 | 3.92 | 3.92 | 3.92 |
| '10MHz+30MHz' | Scenario # | #86 | #87 | #88 | #89 | #90 | #91 | #92 | #93 | #94 | #95 | #96 | #97 | #98 | #99 | #100 | #101 |
|  | 'QPSK' | 5.27 | 2.69 | 2.69 | 1.94 | 0.00 | 4.81 | 3.08 | 3.09 | 0.66 | 3.50 | 2.69 | 2.69 | 1.59 | 3.09 | 3.09 | 3.09 |
|  | '16QAM' | 5.26 | 2.69 | 2.69 | 1.93 | 0.66 | 4.81 | 3.09 | 3.09 | 0.66 | 3.51 | 2.69 | 2.69 | 1.59 | 3.09 | 3.09 | 3.09 |
|  | '64QAM' | 5.27 | 3.09 | 2.68 | 1.94 | 1.93 | 4.81 | 3.08 | 3.08 | 1.94 | 3.51 | 2.69 | 2.69 | 2.30 | 3.09 | 3.09 | 3.09 |
|  | '256QAM' | 5.27 | 3.49 | 3.49 | 3.49 | 3.49 | 4.81 | 3.49 | 3.92 | 3.92 | 3.50 | 3.50 | 3.92 | 3.92 | 3.93 | 3.93 | 3.92 |

- 10MHz + 10MHz

- 20MHz + 30MHz

- 20MHz + 40MHz

- 30MHz + 40MHz

- 20MHz + 20MHz

- 10MHz + 30MHz

Figure 2.2-2: PSSCH/PSCCH MPR simulation results for SL Contiguous CA with Non-contiguous RB allocations

Table 2.2-7 shows the maximum value of simulation results for SL Contiguous CA with Non-contiguous RB allocations considering Inner RB allocation and Outer RB allocation as NR uplink Contiguous CA.

Table 2.2-7 : PSSCH/PSCCH MPR simulation results for SL Contiguous CA with Non-contiguous RB allocations

| Modulation |  | MPR for bandwidth class B(dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Inner | Outer1 | Outer2 |
| CP-OFDM | QPSK | 0.13 | 3.09 | 6.68 |
|  | 16QAM | 0.96 | 3.09 | 6.67 |
|  | 64QAM | 2.31 | 3.10 | 6.67 |
|  | 256QAM | 4.35 | 3.93 | 6.67 |

The MPR can be proposed as Table 2.2-8 based on the simulation results when considering implementation margin.

Table 2.2-8 PSSCH/PSCCH MPR simulation results for SL Contiguous CA with Non-contiguous RB allocations

| Modulation |  | MPR for bandwidth class B(dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Inner | Outer1 | Outer2 |
| CP-OFDM | QPSK | ≤ 3.0 | ≤ 5.0 | ≤ 9.5 |
|  | 16QAM | ≤ 3.0 | ≤ 5.0 | ≤ 9.5 |
|  | 64QAM | ≤ 4.5 | ≤ 5.0 | ≤ 9.5 |
|  | 256QAM | ≤ 7.0 | ≤ 7.0 | ≤ 9.5 |

Proposal 8: For SL intra-band C-CA PSSCH/PSCCH MPR, consider Table 2.2-8 for Non-contiguous RB allocation.

MPR for SLCA PSFCH

The following simulation scenarios are considered as Table 2.3-1 for SL contiguous CA.

Table 2.3-1: SL contiguous CA MPR test scenarios

| Aggregated CBW | Scenario | CC1 | CC2 | R = NGap/(NRB1+NRB2+ NGBchannel_CC1+ NGBchannel_CC2) | SCS |
| --- | --- | --- | --- | --- | --- |
| 10MHz + 10MHz | 1 | 1RB0 | - |  | 15 |
|  | 2 | 1RB51 | - |  | 15 |
|  | 3 | 1RB0 | 1RB0 | 0.5255 | 15 |
|  | 4 | 1RB0 | 1RB10 | 0.6185 | 15 |
|  | 5 | 1RB0 | 1RB20 | 0.7116 | 15 |
|  | 6 | 1RB0 | 1RB30 | 0.8046 | 15 |
|  | 7 | 1RB0 | 1RB40 | 0.8976 | 15 |
|  | 8 | 1RB0 | 1RB51 | 1.0 | 15 |
|  | 9 | 1RB10 | 1RB0 | 0.4324 | 15 |
|  | 10 | 1RB20 | 1RB0 | 0.3394 | 15 |
|  | 11 | 1RB30 | 1RB0 | 0.2463 | 15 |
|  | 12 | 1RB40 | 1RB0 | 0.1533 |  |
|  | 13 | 1RB51 | 1RB0 | 0.0509 | 15 |
| 20MHz + 30MHz | 14 | 1RB0 | - |  | 30 |
|  | 15 | 1RB50 | - |  | 30 |
|  | 16 | 1RB0 | 1RB0 | 0.4248 | 30 |
|  | 17 | 1RB0 | 1RB10 | 0.4995 | 30 |
|  | 18 | 1RB0 | 1RB20 | 0.5742 | 30 |
|  | 19 | 1RB0 | 1RB30 | 0.6489 | 30 |
|  | 20 | 1RB0 | 1RB40 | 0.7236 | 30 |
|  | 21 | 1RB0 | 1RB50 | 0.7983 | 30 |
|  | 22 | 1RB0 | 1RB60 | 0.8730 | 30 |
|  | 23 | 1RB0 | 1RB70 | 0.9477 | 30 |
|  | 24 | 1RB0 | 1RB77 | 1.0 | 30 |
|  | 25 | 1RB10 | 1RB0 | 0.3501 | 30 |
|  | 26 | 1RB20 | 1RB0 | 0.2754 | 30 |
|  | 27 | 1RB30 | 1RB0 | 0.2007 | 30 |
|  | 28 | 1RB40 | 1RB0 | 0.1260 | 30 |
|  | 29 | 1RB50 | 1RB0 | 0.0513 | 30 |
| 20MHz + 40MHz | 30 | 1RB0 | - |  | 30 |
|  | 31 | 1RB50 | - |  | 30 |
|  | 32 | 1RB0 | 1RB0 | 0.3509 | 30 |
|  | 33 | 1RB0 | 1RB10 | 0.4127 | 30 |
|  | 34 | 1RB0 | 1RB20 | 0.4745 | 30 |
|  | 35 | 1RB0 | 1RB30 | 0.5363 | 30 |
|  | 36 | 1RB0 | 1RB40 | 0.5981 | 30 |
|  | 37 | 1RB0 | 1RB50 | 0.6600 | 30 |
|  | 38 | 1RB0 | 1RB60 | 0.7218 | 30 |
|  | 39 | 1RB0 | 1RB70 | 0.7836 | 30 |
|  | 40 | 1RB0 | 1RB80 | 0.8454 | 30 |
|  | 41 | 1RB0 | 1RB90 | 0.9073 | 30 |
|  | 42 | 1RB0 | 1RB100 | 0.9691 | 30 |
|  | 43 | 1RB0 | 1RB105 | 1.0 | 30 |
|  | 44 | 1RB10 | 1RB0 | 0.2890 | 30 |
|  | 45 | 1RB20 | 1RB0 | 0.2272 | 30 |
|  | 46 | 1RB30 | 1RB0 | 0.1654 | 30 |
|  | 47 | 1RB40 | 1RB0 | 0.1036 | 30 |
|  | 48 | 1RB50 | 1RB0 | 0.0417 | 30 |
| 30MHz + 40MHz | 49 | 1RB0 | - |  | 30 |
|  | 50 | 1RB77 | - |  | 30 |
|  | 51 | 1RB0 | 1RB0 | 0.4449 | 30 |
|  | 52 | 1RB0 | 1RB10 | 0.4977 | 30 |
|  | 53 | 1RB0 | 1RB20 | 0.5506 | 30 |
|  | 54 | 1RB0 | 1RB30 | 0.6035 | 30 |
|  | 55 | 1RB0 | 1RB40 | 0.6563 | 30 |
|  | 56 | 1RB0 | 1RB50 | 0.7092 | 30 |
|  | 57 | 1RB0 | 1RB60 | 0.7621 | 30 |
|  | 58 | 1RB0 | 1RB70 | 0.8150 | 30 |
|  | 59 | 1RB0 | 1RB80 | 0.8678 | 30 |
|  | 60 | 1RB0 | 1RB90 | 0.9207 | 30 |
|  | 61 | 1RB0 | 1RB100 | 0.9736 | 30 |
|  | 62 | 1RB0 | 1RB105 | 1.0000 | 30 |
|  | 63 | 1RB10 | 1RB0 | 0.3920 | 30 |
|  | 64 | 1RB20 | 1RB0 | 0.3391 | 30 |
|  | 65 | 1RB30 | 1RB0 | 0.2862 | 30 |
|  | 66 | 1RB40 | 1RB0 | 0.2334 | 30 |
|  | 67 | 1RB50 | 1RB0 | 0.1805 | 30 |
|  | 68 | 1RB60 | 1RB0 | 0.1276 | 30 |
|  | 69 | 1RB70 | 1RB0 | 0.0748 | 30 |
|  | 70 | 1RB77 | 1RB0 | 0.0377 | 30 |
| 20MHz + 20MHz | 71 | 1RB0 | - |  | 30 |
|  | 72 | 1RB51 | - |  | 30 |
|  | 73 | 1RB0 | 1RB0 | 0.5304 | 30 |
|  | 74 | 1RB0 | 1RB10 | 0.6243 | 30 |
|  | 75 | 1RB0 | 1RB20 | 0.7182 | 30 |
|  | 76 | 1RB0 | 1RB30 | 0.8122 | 30 |
|  | 77 | 1RB0 | 1RB40 | 0.9061 | 30 |
|  | 78 | 1RB0 | 1RB50 | 1.0 | 30 |
|  | 79 | 1RB10 | 1RB0 | 0.4365 | 30 |
|  | 80 | 1RB20 | 1RB0 | 0.3426 | 30 |
|  | 81 | 1RB30 | 1RB0 | 0.2486 | 30 |
|  | 82 | 1RB40 | 1RB0 | 0.1547 | 30 |
|  | 83 | 1RB50 | 1RB0 | 0.0608 | 30 |
| 10MHz + 30MHz | 84 | 1RB0 | - |  | 30 |
|  | 85 | 1RB23 | - |  | 30 |
|  | 86 | 1RB0 | 1RB0 | 0.2768 | 30 |
|  | 87 | 1RB0 | 1RB10 | 0.3707 | 30 |
|  | 88 | 1RB0 | 1RB20 | 0.4646 | 30 |
|  | 89 | 1RB0 | 1RB30 | 0.5586 | 30 |
|  | 90 | 1RB0 | 1RB40 | 0.6525 | 30 |
|  | 91 | 1RB0 | 1RB50 | 0.7464 | 30 |
|  | 92 | 1RB0 | 1RB60 | 0.8403 | 30 |
|  | 93 | 1RB0 | 1RB70 | 0.9343 | 30 |
|  | 94 | 1RB0 | 1RB77 | 1.0 | 30 |
|  | 95 | 1RB10 | 1RB0 | 0.1829 | 30 |
|  | 96 | 1RB20 | 1RB0 | 0.0890 | 30 |
|  | 97 | 1RB23 | 1RB0 | 0.0608 | 30 |

Here, the aggregated CA bandwidth is CA bandwidth class B which is composed of 2 CCs (CC1 + CC2). The ratio of NGap/(NRB1+NRB2+ NGBchannel_CC1+ NGBchannel_CC2) is considered.

- R = NGap/(NRB1+NRB2+ NGBchannel_CC1+ NGBchannel_CC2)

  - NGap is the gap RB amount from CC1 RBstart  to CC2 RBend for SL contiguous CA when a single PSFCH or contiguous and non-contiguous allocation simultaneous PSFCHs is transmitted in each CC.

  - NGBchannel_CC1 and NGBchannel_CC2 is the number of RB which corresponds to the minimum guardbands for CC1 and CC2, respectively, which have been calculated using the following equation:

NGBchannel =  GBchannel / (SCS x 12)

GBchannel = (BWChannel - NRB x SCS x 12) / 2 - SCS/2,

where NRB are from Table 5.3.2-1 in TS38.101-1, and BWChannel is the channel bandwidth for each CC.

Figure 2.3-0 shows NGap, NRB1, NRB2, NGBchannel_CC1, and NGBchannel_CC2.

Figure 2.3-0: Parameters related to Ratio of R

Table 2.3-2 and Figure 2.3-1 show the PSFCH MPR simulation results.

Table 2.3-2: PSFCH MPR simulation results for SL Contiguous CA

| '10MHz+10MHz' | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 | #11 | #12 | #13 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | R | - | - | 0.53 | 0.62 | 0.71 | 0.80 | 0.90 | 1.00 | 0.43 | 0.34 | 0.25 | 0.15 | 0.05 |  |  |  |  |  |
|  |  | 2.66 | 0.00 | 6.91 | 6.91 | 11.14 | 11.14 | 10.68 | 10.70 | 2.04 | 0.84 | 0.00 | 0.00 | 0.00 |  |  |  |  |  |
| '20MHz+30MHz' | Scenario # | #14 | #15 | #16 | #17 | #18 | #19 | #20 | #21 | #22 | #23 | #24 | #25 | #26 | #27 | #28 | #29 |  |  |
|  | R | - | - | 0.42 | 0.50 | 0.57 | 0.65 | 0.72 | 0.80 | 0.87 | 0.95 | 1.00 | 0.35 | 0.28 | 0.20 | 0.13 | 0.05 |  |  |
|  |  | 1.59 | 0.00 | 6.37 | 5.88 | 5.40 | 10.63 | 11.08 | 10.63 | 10.63 | 10.17 | 10.18 | 2.04 | 0.84 | 0.00 | 0.00 | 0.00 |  |  |
| '20MHz+40MHz' | Scenario # | #30 | #31 | #32 | #33 | #34 | #35 | #36 | #37 | #38 | #39 |  |  |  |  |  |  |  |  |
|  | R | - | - | 0.35 | 0.41 | 0.47 | 0.54 | 0.60 | 0.66 | 0.72 | 0.78 |  |  |  |  |  |  |  |  |
|  |  | 2.58 | 0.00 | 1.68 | 6.39 | 5.90 | 5.90 | 11.10 | 10.64 | 10.18 | 9.71 |  |  |  |  |  |  |  |  |
|  | Scenario # | #40 | #41 | #42 | #43 | #44 | #45 | #46 | #47 | #48 |  |  |  |  |  |  |  |  |  |
|  | R | 0.85 | 0.91 | 0.97 | 1.00 | 0.29 | 0.23 | 0.17 | 0.10 | 0.04 |  |  |  |  |  |  |  |  |  |
|  |  | 10.64 | 11.10 | 10.64 | 10.65 | 1.22 | 0.84 | 0.00 | 0.00 | 0.00 |  |  |  |  |  |  |  |  |  |
| '30MHz+40MHz' | Scenario # | #49 | #50 | #51 | #52 | #53 | #54 | #55 | #56 | #57 | #58 | #59 |  |  |  |  |  |
|  | R | - | - | 0.44 | 0.50 | 0.55 | 0.60 | 0.66 | 0.71 | 0.76 | 0.82 | 0.87 |  |  |  |  |  |
|  |  | 3.15 | 0.00 | 5.45 | 6.42 | 5.94 | 11.14 | 10.68 | 9.75 | 10.68 | 10.68 | 11.14 |  |  |  |  |  |
|  | Scenario # | #60 | #61 | #62 | #63 | #64 | #65 | #66 | #67 | #68 | #69 | #70 |  |  |  |  |  |
|  | R | 0.92 | 0.97 | 1.00 | 0.39 | 0.34 | 0.29 | 0.23 | 0.18 | 0.13 | 0.07 | 0.04 |  |  |  |  |  |
|  |  | 10.68 | 10.22 | 11.16 | 2.04 | 2.04 | 1.21 | 1.21 | 0.00 | 0.00 | 0.00 | 0.00 |  |  |  |  |  |
| '20MHz+20MHz' | Scenario # | #71 | #72 | #73 | #74 | #75 | #76 | #77 | #78 | #79 | #80 | #81 | #82 | #83 |  |  |  |  |  |
|  | R | - | - | 0.53 | 0.62 | 0.72 | 0.81 | 0.91 | 1.00 | 0.44 | 0.34 | 0.25 | 0.15 | 0.06 |  |  |  |  |  |
|  |  | 2.66 | 0.00 | 6.43 | 10.68 | 10.68 | 11.14 | 10.68 | 10.70 | 2.48 | 0.84 | 0.00 | 0.00 | 0.00 |  |  |  |  |  |
| '10MHz+30MHz' | Scenario # | #84 | #85 | #86 | #87 | #88 | #89 | #90 | #91 | #92 | #93 | #94 | #95 | #96 | #97 |  |  |  |  |
|  | R | - | - | 0.28 | 0.37 | 0.46 | 0.56 | 0.65 | 0.75 | 0.84 | 0.93 | 1.00 | 0.18 | 0.09 | 0.06 |  |  |  |  |
|  |  | 2.66 | 0.00 | 1.31 | 2.14 | 5.45 | 5.45 | 10.22 | 11.14 | 10.68 | 10.22 | 10.70 | 0.84 | 0.00 | 0.00 |  |  |  |  |

Figure 2.3-1: PSFCH MPR simulation results for SL Contiguous CA vs Ratio ( R)

Table 2.3-3 shows the maximum value of simulation results for SL Contiguous CA considering the ratio of R.

R = NGap/(NRB1+NRB2+ NGBchannel_CC1+ NGBchannel_CC2)

Table 2.3-3 : PSFCH MPR simulation results for SL Contiguous CA

| MPR for ratio (R) in bandwidth class B(dB) |  |  |
| --- | --- | --- |
| R ≤ 0. 33 | 0.33 < R ≤ 0. 55 | 0.55 < R ≤ 1.0 |
| 1.31 | 6.91 | 11.16 |

The MPR can be proposed as Table 2.3-4 based on the simulation results when considering implementation margin.

Table 2.3-4 PSFCH MPR simulation results for SL Contiguous CA

| MPR for ratio (R) in bandwidth class B(dB) |  |  |
| --- | --- | --- |
| R ≤ 0. 33 | 0.33 < R ≤ 0. 55 | 0.55 < R ≤ 1.0 |
| 3.5 | 9.0 | 13.0 |
| Here, R = NGap/(NRB1+NRB2+ NGBchannel_CC1+ NGBchannel_CC2) |  |  |

Proposal 9: For SL intra-band C-CA PSFCH MPR, consider Table 2.3-4.

MPR for SLCA S-SSB

The following simulation scenarios are considered as Table 2.4-1 for SL contiguous CA.

Table 2.4-1: SL contiguous CA MPR test scenarios

| Aggregated CBW | Scenario | CC1 | CC2 | Inner/Outer1/Outer2 RB allocation | SCS |
| --- | --- | --- | --- | --- | --- |
| 10MHz + 10MHz | 1 | 11RB0 | 11RB41 | Outer2 | 15 |
|  | 2 | 11RB18 | 11RB22 | Outer2 | 15 |
|  | 3 | 11RB20 | 11RB22 | Outer1 | 15 |
|  | 4 | 11RB33 | 11RB17 | Outer1 | 15 |
|  | 5 | 11RB35 | 11RB7 | Inner | 15 |
|  | 6 | 11RB41 | 11RB0 | Inner | 15 |
| 20MHz + 30MHz | 7 | 11RB0 | 11RB67 | Outer2 | 30 |
|  | 8 | 11RB15 | 11RB32 | Outer2 | 30 |
|  | 9 | 11RB15 | 11RB30 | Outer1 | 30 |
|  | 10 | 11RB33 | 11RB7 | Outer1 | 30 |
|  | 11 | 11RB34 | 11RB7 | Inner | 30 |
|  | 12 | 11RB40 | 11RB0 | Inner | 30 |
| 20MHz + 40MHz | 13 | 11RB0 | 11RB95 | Outer2 | 30 |
|  | 14 | 11RB4 | 11RB30 | Outer2 | 30 |
|  | 15 | 11RB15 | 11RB30 | Outer1 | 30 |
|  | 16 | 11RB33 | 11RB7 | Outer1 | 30 |
|  | 17 | 11RB34 | 11RB7 | Inner | 30 |
|  | 18 | 11RB40 | 11RB0 | Inner | 30 |
| 30MHz + 40MHz | 19 | 11RB0 | 11RB95 | Outer2 | 30 |
|  | 20 | 11RB4 | 11RB30 | Outer2 | 30 |
|  | 21 | 11RB15 | 11RB30 | Outer1 | 30 |
|  | 22 | 11RB47 | 11RB7 | Outer1 | 30 |
|  | 23 | 11RB48 | 11RB7 | Inner | 30 |
|  | 24 | 11RB67 | 11RB0 | Inner | 30 |
| 20MHz + 20MHz | 25 | 11RB0 | 11RB40 | Outer2 | 30 |
|  | 26 | 11RB4 | 11RB30 | Outer2 | 30 |
|  | 27 | 11RB28 | 11RB7 | Outer1 | 30 |
|  | 28 | 11RB33 | 11RB7 | Outer1 | 30 |
|  | 29 | 11RB34 | 11RB0 | Inner | 30 |
|  | 30 | 11RB40 | 11RB0 | Inner | 30 |
| 10MHz + 30MHz | 31 | 11RB0 | 11RB67 | Outer2 | 30 |
|  | 32 | 11RB7 | 11RB32 | Outer2 | 30 |
|  | 33 | 11RB7 | 11RB31 | Outer1 | 30 |
|  | 34 | 11RB13 | 11RB1 | Outer1 | 30 |
|  | 35 | 11RB13 | 11RB0 | Inner | 30 |

Table 2.4-2 and Figure 2.4-1 show the S-SSB MPR simulation results.

Table 2.4-2: S-SSB MPR simulation results for SL Contiguous CA

| '10MHz+10MHz' | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 10.21 | 6.30 | 6.20 | 1.25 | 1.08 | 1.50 |
| '20MHz+30MHz' | Scenario # | #7 | #8 | #9 | #10 | #11 | #12 |
|  |  | 8.20 | 4.52 | 4.38 | 1.14 | 1.09 | 1.46 |
| '20MHz+40MHz' | Scenario # | #13 | #14 | #15 | #16 | #17 | #18 |
|  |  | 8.08 | 4.50 | 4.62 | 1.23 | 1.06 | 1.25 |
| '30MHz+40MHz' | Scenario # | #19 | #20 | #21 | #22 | #23 | #24 |
|  |  | 8.06 | 7.84 | 4.19 | 1.07 | 1.10 | 1.70 |
| '20MHz+20MHz' | Scenario # | #25 | #26 | #27 | #28 | #29 | #30 |
|  |  | 8.20 | 8.04 | 4.40 | 1.25 | 1.15 | 1.51 |
| '10MHz+30MHz' | Scenario # | #31 | #32 | #33 | #34 | #35 |  |
|  |  | 8.20 | 4.24 | 4.50 | 1.55 | 1.52 |  |

Figure 2.4-1: S-SSB MPR simulation results for SL Contiguous CA

Table 2.4-3 shows the maximum value of simulation results for SL Contiguous CA considering Inner, Outer1, and Outer 2 RB allocation ranges.

Table 2.4-3: S-SSB MPR simulation results for SL Contiguous CA

| MPR for bandwidth class B(dB) |  |  |
| --- | --- | --- |
| Inner | Outer1 | Outer2 |
| 1.70 | 6.20 | 10.21 |

The MPR can be proposed as Table 2.4-4 based on the simulation results when considering implementation margin.

Table 2.4-4 S-SSB MPR simulation results for SL Contiguous CA

| MPR for bandwidth class B(dB) |  |  |
| --- | --- | --- |
| Inner | Outer1 | Outer2 |
| 3.5 | 9.0 | 13.0 |

Proposal 10: Reuse Inner/Outer RB allocation and Inner/Outer1/Outer2 RB allocation of NR C-CA for SL C-CA S-SSB MPR.

Proposal 11: For SL intra-band C-CA S-SSB MPR, consider Table 2.4-4.

A-MPR for SLCA

So far, A-MPR for n47 was specified with NS_33 for CBW of 10MHz and with NS_52 for CBW of 40MHz.

Regarding the possible aggregated CBWs of 20MHz, 30MHz, 40MHz, 50MHz, 60MHz and 70MHz for SL intra-band contiguous CA, the SL CA A-MPR needs to be defined for the aggregated CBW of 40MHz.

For SL CA, a new NS value, ‘SL-CA NS_52’ needs to be considered. Table 2.5-1 shows one example.

Table 2.5-1: Mapping of network signaling label

| NR CA band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| CA_n47 | SL-CA_NS_01 | SL-CA_NS_33 | SL-CA_NS_52 |  |  |  |  |  |
| NOTE: additionalSpectrumEmission corresponds to an information element of the same name defined in clause 6.3.2 of TS 38.331 |  |  |  |  |  |  |  |  |

Proposal 12: Define SL intra-band C-CA A-MPR for aggregated CBW of 40MHz with ’SL-CA NS_52’.

A-MPR for SLCA PSSCH/PSCCH

For Contiguous RB allocation of SL contiguous CA

The following simulation scenarios are considered as Table 2.6-1 for Contiguous RB allocations.

Table 2.6-1: SL contiguous CA A-MPR test scenarios (Contiguous RB allocation)

| Aggregated CBW | Scenario | CC1 | CC2 | Inner/Outer RB allocation | SCS |
| --- | --- | --- | --- | --- | --- |
| 20MHz + 20MHz | 1 | 10RB41 | 10RB0 | Inner | 30 |
|  | 2 | 10RB41 | 12RB0 | Inner | 30 |
|  | 3 | 10RB41 | 15RB0 | Inner | 30 |
|  | 4 | 10RB41 | 25RB0 | Inner | 30 |
|  | 5 | 10RB41 | 30RB0 | Inner | 30 |
|  | 6 | 10RB41 | 36RB0 | Outer | 30 |
|  | 7 | 10RB41 | 50RB0 | Outer | 30 |
|  | 8 | 12RB39 | 36RB0 | Outer | 30 |
|  | 9 | 25RB26 | 36RB0 | Outer | 30 |
|  | 10 | 40RB11 | 40RB0 | Outer | 30 |
|  | 11 | 50RB1 | 50RB0 | Outer | 30 |
| 10MHz + 30MHz | 12 | 10RB41 | 10RB0 | Inner | 30 |
|  | 13 | 10RB41 | 12RB0 | Inner | 30 |
|  | 14 | 10RB41 | 15RB0 | Inner | 30 |
|  | 15 | 10RB41 | 20RB0 | Inner | 30 |
|  | 16 | 10RB41 | 25RB0 | Outer | 30 |
|  | 17 | 10RB41 | 50RB0 | Outer | 30 |
|  | 18 | 10RB41 | 75RB0 | Outer | 30 |
|  | 19 | 12RB12 | 36RB0 | Outer | 30 |
|  | 20 | 15RB9 | 60RB0 | Outer | 30 |
|  | 21 | 20RB4 | 75RB0 | Outer |  |
|  | 21 | 24RB0 | 75RB0 | Outer | 30 |

Table 2.6-2 and Figure 2.6-1 show SL CA NS_52 PSSCH/PSCCH A-MPR simulation results for the SL contiguous CA scenarios with Contiguous RB allocations.

Table 2.6-2: SL CA NS_52 PSSCH/PSCCH A-MPR simulation results for SL Contiguous CA with Contiguous RB allocations

| '20MHz+20MHz' | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 | #11 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 'QPSK' | 0.00 | 0.00 | 0.12 | 2.31 | 3.49 | 4.35 | 4.80 | 3.92 | 3.92 | 4.36 | 5.72 |  |  |  |  |  |  |  |
|  | '16QAM' | 0.65 | 0.95 | 0.94 | 2.69 | 3.49 | 3.92 | 4.80 | 3.92 | 3.92 | 4.36 | 5.72 |  |  |  |  |  |  |  |
|  | '64QAM' | 1.92 | 1.93 | 1.93 | 2.31 | 3.49 | 3.92 | 4.80 | 3.92 | 3.92 | 4.36 | 5.72 |  |  |  |  |  |  |  |
|  | '256QAM' | 3.48 | 3.91 | 3.92 | 3.91 | 3.92 | 3.91 | 4.80 | 3.91 | 3.92 | 4.35 | 5.72 |  |  |  |  |  |  |  |
| '10MHz+30MHz' | Scenario # | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 | #21 | #22 |  |  |  |  |  |  |  |
|  | 'QPSK' | 3.91 | 3.91 | 4.35 | 4.36 | 3.92 | 3.92 | 4.80 | 4.35 | 3.92 | 5.26 | 5.72 |  |  |  |  |  |  |  |
|  | '16QAM' | 3.92 | 3.91 | 3.91 | 4.35 | 3.91 | 3.92 | 4.80 | 4.35 | 3.92 | 5.26 | 5.72 |  |  |  |  |  |  |  |
|  | '64QAM' | 3.91 | 3.92 | 3.91 | 4.35 | 4.35 | 3.92 | 4.80 | 4.35 | 3.92 | 5.26 | 5.72 |  |  |  |  |  |  |  |
|  | '256QAM' | 3.91 | 3.91 | 3.91 | 4.35 | 4.36 | 3.92 | 4.80 | 4.35 | 3.92 | 5.26 | 5.72 |  |  |  |  |  |  |  |

- 20MHz + 20MHz

- 10MHz + 30MHz

Figure 2.6-1: SL CA NS_52 PSSCH/PSCCH A-MPR simulation results for SL Contiguous CA with Contiguous RB allocations

Table 2.6-3 shows the maximum value of simulation results for SL Contiguous CA with Contiguous RB allocations considering Inner RB allocation and Outer RB allocation as NR uplink Contiguous CA.

Table 2.6-3 : SL CA NS_52 PSSCH/PSCCH A-MPR simulation results for SL Contiguous CA with Contiguous RB allocations

| Modulation |  | A-MPR for bandwidth class B(dB) |  |
| --- | --- | --- | --- |
|  |  | inner | outer |
| CP-OFDM | QPSK | 4.36 | 5.72 |
|  | 16QAM | 4.35 | 5.72 |
|  | 64QAM | 4.35 | 5.72 |
|  | 256QAM | 4.35 | 5.72 |

The A-MPR can be proposed as Table 2.6-4 based on the simulation results when considering implementation margin.

Table 2.6-4 SL CA NS_52 PSSCH/PSCCH A-MPR for SL Contiguous CA with Contiguous RB allocations

| Modulation |  | A-MPR for bandwidth class B(dB) |  |
| --- | --- | --- | --- |
|  |  | inner | outer |
| CP-OFDM | QPSK | ≤ 7.0 | ≤ 8.5 |
|  | 16QAM | ≤ 7.0 | ≤ 8.5 |
|  | 64QAM | ≤ 7.0 | ≤ 8.5 |
|  | 256QAM | ≤ 7.0 | ≤ 8.5 |

Proposal 13: For SL-CA NS_52 PSSCH/PSCCH A-MPR, consider Table 2.6-4 for Contiguous RB allocation.

For Non-contiguous RB allocation of SL contiguous CA

The following test scenarios are considered as Table 2.6-5 for Non-contiguous RB allocations.

Table 2.6-5: SL contiguous CA A-MPR test scenarios (Non-contiguous RB allocation)

| Aggregated CBW | Scenario | CC1 | CC2 | Inner/Outer1/Outer2 RB allocation | SCS |
| --- | --- | --- | --- | --- | --- |
| 20MHz + 20MHz | 1 | 10RB0 | 10RB41 | Outer2 | 30 |
|  | 2 | 10RB17 | 10RB22 | Outer2 | 30 |
|  | 3 | 10RB18 | 10RB22 | Outer1 | 30 |
|  | 4 | 10RB27 | 10RB14 | Outer1 | 30 |
|  | 5 | 10RB33 | 10RB7 | Outer1 | 30 |
|  | 6 | 10RB34 | 10RB7 | Inner | 30 |
|  | 7 | 10RB41 | 10RB1 | Inner | 30 |
|  | 8 | 25RB0 | 25RB26 | Outer2 | 30 |
|  | 9 | 25RB18 | 25RB8 | Outer2 | 30 |
|  | 10 | 25RB19 | 25RB8 | Outer1 | 30 |
|  | 11 | 25RB26 | 25RB1 | Outer1 | 30 |
|  | 12 | 30RB0 | 30RB21 | Outer2 | 30 |
|  | 13 | 30RB18 | 30RB3 | Outer1 | 30 |
|  | 14 | 30RB18 | 30RB2 | Outer1 | 30 |
|  | 15 | 30RB21 | 30RB1 | Outer2 | 30 |
|  | 16 | 50RB0 | 50RB0 | Outer2 | 30 |
| 10MHz + 30MHz | 17 | 10RB0 | 10RB68 | Outer2 | 30 |
|  | 18 | 10RB6 | 10RB32 | Outer2 | 30 |
|  | 19 | 10RB6 | 10RB31 | Outer1 | 30 |
|  | 20 | 10RB14 | 10RB10 | Outer1 | 30 |
|  | 21 | 10RB14 | 10RB1 | Outer1 | 30 |
|  | 22 | 10RB0 | 12RB53 | Outer2 | 30 |
|  | 23 | 10RB3 | 12RB25 | Outer2 | 30 |
|  | 24 | 25RB4 | 12RB25 | Outer1 | 30 |
|  | 25 | 25RB12 | 12RB1 | Outer1 | 30 |
|  | 26 | 25RB0 | 30RB48 | Outer2 | 30 |
|  | 27 | 25RB3 | 30RB7 | Outer2 | 30 |
|  | 28 | 30RB3 | 30RB6 | Outer1 | 30 |
|  | 29 | 30RB12 | 30RB1 | Outer1 | 30 |
|  | 30 | 30RB0 | 50RB28 | Outer2 | 30 |
|  | 31 | 30RB0 | 50RB1 | Outer2 | 30 |
|  | 32 | 50RB0 | 75RB1 | Outer2 | 30 |

Table 2.6-6 and Figure 2.6-2 show SL CA NS_52 PSSCH/PSCCH A-MPR simulation results for the SL contiguous CA scenarios with Non-contiguous RB allocations.

Table 2.6-6: SL CA NS_52 PSSCH/PSCCH A-MPR simulation results for SL Contiguous CA with Non-contiguous RB allocations

| '20MHz+20MHz' | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 | #11 | #12 | #13 | #14 | #15 | #16 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 'QPSK' | 10.93 | 5.25 | 5.25 | 4.80 | 1.59 | 1.26 | 0.00 | 7.13 | 3.92 | 3.92 | 3.49 | 6.66 | 3.92 | 3.92 | 3.92 | 5.72 |  |  |
|  | '16QAM' | 10.93 | 5.25 | 5.25 | 4.80 | 1.59 | 1.25 | 0.65 | 7.14 | 3.92 | 3.92 | 3.49 | 6.66 | 3.92 | 3.92 | 3.92 | 5.72 |  |  |
|  | '64QAM' | 10.93 | 5.25 | 5.25 | 4.80 | 1.93 | 1.94 | 1.93 | 7.13 | 3.92 | 3.92 | 3.49 | 6.66 | 3.92 | 3.92 | 3.92 | 5.72 |  |  |
|  | '256QAM' | 10.93 | 5.25 | 5.26 | 4.80 | 3.49 | 3.49 | 3.49 | 7.14 | 3.92 | 3.92 | 3.92 | 6.66 | 3.92 | 3.93 | 3.92 | 5.72 |  |  |
| '10MHz+30MHz' | Scenario # | #17 | #18 | #19 | #20 | #21 | #22 | #23 | #24 | #25 | #26 | #27 | #28 | #29 | #30 | #31 | #32 |  |  |
|  | 'QPSK' | 10.93 | 5.25 | 5.25 | 4.80 | 3.91 | 10.45 | 5.25 | 5.26 | 4.35 | 8.56 | 4.35 | 4.36 | 4.35 | 6.66 | 4.80 | 5.72 |  |  |
|  | '16QAM' | 10.92 | 5.26 | 5.25 | 4.79 | 3.91 | 10.46 | 5.25 | 5.26 | 4.35 | 8.57 | 4.80 | 4.80 | 4.35 | 6.66 | 4.81 | 5.72 |  |  |
|  | '64QAM' | 10.92 | 5.25 | 5.26 | 4.79 | 3.92 | 10.45 | 5.25 | 5.25 | 4.35 | 8.08 | 4.80 | 4.80 | 4.35 | 6.66 | 4.80 | 5.72 |  |  |
|  | '256QAM' | 10.93 | 5.25 | 5.25 | 4.80 | 3.91 | 10.45 | 5.25 | 5.25 | 4.35 | 8.09 | 4.80 | 4.80 | 4.35 | 6.66 | 4.80 | 5.72 |  |  |

- 20MHz + 20MHz

- 10MHz + 30MHz

Figure 2.6-2: SL CA NS_52 PSSCH/PSCCH A-MPR simulation results for SL Contiguous CA with Non-contiguous RB allocations

Table 2.6-7 shows the maximum value of simulation results for SL Contiguous CA with Non-contiguous RB allocations considering Inner RB allocation and Outer RB allocation as NR uplink Contiguous CA.

Table 2.6-7 : SL CA NS_52 PSSCH/PSCCH A-MPR simulation results for SL Contiguous CA with Non-contiguous RB allocations

| Modulation |  | A-MPR for bandwidth class B(dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Inner | Outer1 | Outer2 |
| CP-OFDM | QPSK | 1.26 | 5.26 | 10.93 |
|  | 16QAM | 1.25 | 5.26 | 10.93 |
|  | 64QAM | 1.94 | 5.26 | 10.93 |
|  | 256QAM | 3.49 | 5.26 | 10.93 |

The A-MPR can be proposed as Table 2.6-8 based on the simulation results when considering implementation margin.

Table 2.6-8 SL CA NS_52 PSSCH/PSCCH A-MPR for SL Contiguous CA with Non-contiguous RB allocations

| Modulation |  | A-MPR for bandwidth class B(dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Inner | Outer1 | Outer2 |
| CP-OFDM | QPSK | ≤ 3.0 | ≤ 8.0 | ≤ 13.5 |
|  | 16QAM | ≤ 3.0 | ≤ 8.0 | ≤ 13.5 |
|  | 64QAM | ≤ 4.5 | ≤ 8.0 | ≤ 13.5 |
|  | 256QAM | ≤ 7.0 | ≤ 8.0 | ≤ 13.5 |

Proposal 14: For SL-CA NS_52 PSSCH/PSCCH A-MPR, consider Table 2.6-8 for Non-contiguous RB allocation.

A-MPR for SLCA PSFCH

The following test scenarios are considered as Table 2.7-1 for SL contiguous CA.

Table 2.7-1: SL contiguous CA MPR test scenarios

| Aggregated CBW | Scenario | CC1 | CC2 | R = NGap/(NRB1+NRB2+ NGBchannel_CC1+ NGBchannel_CC2) | SCS |
| --- | --- | --- | --- | --- | --- |
| 20MHz + 20MHz | 1 | 1RB0 | - |  | 30 |
|  | 2 | 1RB51 | - |  | 30 |
|  | 3 | 1RB0 | 1RB0 | 0.5304 | 30 |
|  | 4 | 1RB0 | 1RB10 | 0.6243 | 30 |
|  | 5 | 1RB0 | 1RB20 | 0.7182 | 30 |
|  | 6 | 1RB0 | 1RB30 | 0.8122 | 30 |
|  | 7 | 1RB0 | 1RB40 | 0.9061 | 30 |
|  | 8 | 1RB0 | 1RB50 | 1.0 | 30 |
|  | 9 | 1RB10 | 1RB0 | 0.4365 | 30 |
|  | 10 | 1RB20 | 1RB0 | 0.3426 | 30 |
|  | 11 | 1RB30 | 1RB0 | 0.2486 | 30 |
|  | 12 | 1RB40 | 1RB0 | 0.1547 | 30 |
|  | 13 | 1RB50 | 1RB0 | 0.0608 | 30 |
| 10MHz + 30MHz | 14 | 1RB0 | - |  | 30 |
|  | 15 | 1RB23 | - |  | 30 |
|  | 16 | 1RB0 | 1RB0 | 0.2768 | 30 |
|  | 17 | 1RB0 | 1RB10 | 0.3707 | 30 |
|  | 18 | 1RB0 | 1RB20 | 0.4646 | 30 |
|  | 19 | 1RB0 | 1RB30 | 0.5586 | 30 |
|  | 20 | 1RB0 | 1RB40 | 0.6525 | 30 |
|  | 21 | 1RB0 | 1RB50 | 0.7464 | 30 |
|  | 22 | 1RB0 | 1RB60 | 0.8403 | 30 |
|  | 23 | 1RB0 | 1RB70 | 0.9343 | 30 |
|  | 24 | 1RB0 | 1RB77 | 1.0 | 30 |
|  | 25 | 1RB10 | 1RB0 | 0.1829 | 30 |
|  | 26 | 1RB20 | 1RB0 | 0.0890 | 30 |
|  | 27 | 1RB23 | 1RB0 | 0.0608 | 30 |

Table 2.7-2 and Figure 2.7-1 show SL CA NS_52 PSFCH A-MPR simulation results for the SL contiguous CA scenarios.

Table 2.7-2: SL CA NS_52 PSFCH A-MPR simulation results for SL Contiguous CA

| '20MHz+20MHz' | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 | #11 | #12 | #13 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | R | - | - | 0.53 | 0.62 | 0.72 | 0.81 | 0.91 | 1.00 | 0.44 | 0.34 | 0.25 | 0.15 | 0.06 |  |  |  |  |  |
|  |  | 16.72 | 0.00 | 13.42 | 16.61 | 16.15 | 16.61 | 16.61 | 16.64 | 13.77 | 12.86 | 7.29 | 0.00 | 0.00 |  |  |  |  |  |
| '10MHz+30MHz' | Scenario # | #14 | #15 | #16 | #17 | #18 | #19 | #20 | #21 | #22 | #23 | #24 | #25 | #26 | #27 |  |  |  |  |
|  | R | - | - | 0.28 | 0.37 | 0.46 | 0.56 | 0.65 | 0.75 | 0.84 | 0.93 | 1.00 | 0.18 | 0.09 | 0.06 |  |  |  |  |
|  |  | 16.72 | 0.60 | 13.42 | 14.33 | 14.33 | 15.70 | 16.61 | 16.61 | 16.61 | 16.61 | 16.64 | 11.49 | 1.62 | 1.21 |  |  |  |  |

Figure 2.7-1: SL CA NS_52 PSFCH A-MPR simulation results for SL Contiguous CA vs Ratio ( R )

Table 2.7-3 shows the maximum value of simulation results for SL Contiguous CA considering the ratio of R.

R = NGap/(NRB1+NRB2+ NGBchannel_CC1+ NGBchannel_CC2)

Table 2.7-3 : SL CA NS_52 PSFCH A-MPR simulation results for SL Contiguous CA

| Modulation |  | A-MPR for ratio (R) in bandwidth class B(dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | R ≤ 0. 1 | 0.1 < R ≤ 0. 55 | 0.55 < R ≤ 1.0 |
| CP-OFDM | QPSK | 1.62 | 14.33 | 16.64 |

The A-MPR can be proposed as Table 2.7-4 based on the simulation results when considering implementation margin.

Table 2.7-4 SL CA NS_52 PSFCH A-MPR for SL Contiguous CA

| Modulation |  | A-MPR for ratio (R) in bandwidth class B(dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | R ≤0. 1 | 0.1 < R ≤ 0. 55 | 0.55 < R ≤ 1.0 |
| CP-OFDM | QPSK | 4.0 | 17.0 | 19.0 |

Proposal 15: For SL-CA NS_52 PSFCH A-MPR, consider Table 2.7-4.

A-MPR for SLCA S-SSB

The following test scenarios are considered as Table 2.8-1 for SL contiguous CA.

Table 2.8-1: SL contiguous CA MPR test scenarios

| Aggregated CBW | Scenario | CC1 | CC2 | Inner/Outer1/Outer2 RB allocation | SCS |
| --- | --- | --- | --- | --- | --- |
| 20MHz + 20MHz | 1 | 11RB0 | 11RB40 | Outer2 | 30 |
|  | 2 | 11RB4 | 11RB30 | Outer2 | 30 |
|  | 3 | 11RB28 | 11RB7 | Outer1 | 30 |
|  | 4 | 11RB33 | 11RB7 | Outer1 | 30 |
|  | 5 | 11RB34 | 11RB0 | Inner | 30 |
|  | 6 | 11RB40 | 11RB0 | Inner | 30 |
| 10MHz + 30MHz | 7 | 11RB0 | 11RB67 | Outer2 | 30 |
|  | 8 | 11RB7 | 11RB32 | Outer2 | 30 |
|  | 9 | 11RB7 | 11RB31 | Outer1 | 30 |
|  | 10 | 11RB13 | 11RB1 | Outer1 | 30 |
|  | 11 | 11RB13 | 11RB0 | Inner | 30 |

Table 2.8-2 and Figure 2.8-1 show SL CA NS_52 S-SSB A-MPR simulation results for the SL contiguous CA scenarios.

Table 2.8-2: SL CA NS_52 S-SSB A-MPR simulation results for SL Contiguous CA

| '20MHz+20MHz' | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 13.24 | 13.32 | 10.05 | 4.61 | 3.78 | 1.51 |
| '10MHz+30MHz' | Scenario # | #7 | #8 | #9 | #10 | #11 |  |
|  |  | 13.24 | 8.86 | 10.05 | 5.69 | 5.89 |  |

Figure 2.8-1: SL CA NS_52 S-SSB A-MPR simulation results for SL Contiguous CA

Table 2.8-3 shows the maximum value of simulation results for SL Contiguous CA considering Inner, Outer1, and Outer 2 RB allocation ranges.

Table 2.8-3 : SL CA NS_52 S-SSB A-MPR simulation results for SL Contiguous CA

| A-MPR for bandwidth class B(dB) |  |  |
| --- | --- | --- |
| Inner | Outer1 | Outer2 |
| 5.89 | 10.05 | 13.32 |

The A-MPR can be proposed as Table 2.8-4 based on the simulation results when considering implementation margin.

Table 2.8-4 SL CA NS_52 S-SSB A-MPR for SL Contiguous CA

| A-MPR for bandwidth class B(dB) |  |  |
| --- | --- | --- |
| Inner | Outer1 | Outer2 |
| ≤ 9.0 | ≤ 13.0 | ≤ 16.5 |

Proposal 16: For SL-CA NS_52 S-SSB A-MPR, consider Table 2.8-4.

Conclusion

In this contribution, we provided our view on the remained issues and MPR/A-MPR simulation results for SL intra-band contiguous CA. Based on those, the followings are proposed.

- Methodology on Tx requirements

Proposal 1: Take NR intra-band contiguous CA as methodology on Tx requirements for SL intra-band contiguous CA.

- Configured transmitted power

Proposal 2: Consider NR intra-band contiguous CA as baseline for configured transmitted power for sidelink intra-band CA.

Proposal 3: Consider the terminology of  ’PPowerClass,SL-CA’ for power class of SL intra-band contiguous CA.

Proposal 4: Define a capability of power class per band combination for supporting SL intra-band contiguous CA as Table 2.1-1.

Table 2.1-1: Capability of SL CA power class

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| powerClassSidelink-r18 This parameter indicates power class the UE supports when operating according to this band combination used for sidelink. If the field is absent, the UE supports the default power class. If this power class is higher than the power class that the UE supports on the individual bands of this band combination (ue-PowerClassSidelink-r16 in BandNR), the latter determines maximum TX power available in each band. The UE sets the power class parameter only in band combinations that are applicable as specified in TS 38.101-1 [2] and TS 38.101-3 [4]. | BC | No | N/A | FR1 only |

Proposal 5: Send LS to RAN2  on necessity of a capability of power class per band combination and a new IE which corresponds to PEMAX,CA for supporting SL intra-band contiguous CA.

- MPR for SLCA PSSCH/PSCCH

Proposal 6: Reuse Inner/Outer RB allocation and Inner/Outer1/Outer2 RB allocation of NR C-CA for SL C-CA PSSCH/PSCCH MPR.

Proposal 7: For SL intra-band C-CA PSSCH/PSCCH MPR, consider Table 2.2-4 for contiguous RB allocation.

Table 2.2-4: PSSCH/PSCCH MPR simulation results for SL Contiguous CA with Contiguous RB allocations

| Modulation |  | MPR for bandwidth class B(dB) |  |
| --- | --- | --- | --- |
|  |  | inner | outer |
| CP-OFDM | QPSK | ≤ 3.0 | ≤ 5.0 |
|  | 16QAM | ≤ 3.0 | ≤ 5.0 |
|  | 64QAM | ≤ 4.5 | ≤ 5.0 |
|  | 256QAM | ≤ 7.0 | ≤ 7.0 |

Proposal 8: For SL intra-band C-CA PSSCH/PSCCH MPR, consider Table 2.2-8 for Non-contiguous RB allocation.

Table 2.2-8 PSSCH/PSCCH MPR simulation results for SL Contiguous CA with Non-contiguous RB allocations

| Modulation |  | MPR for bandwidth class B(dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Inner | Outer1 | Outer2 |
| CP-OFDM | QPSK | ≤ 3.0 | ≤ 5.0 | ≤ 9.5 |
|  | 16QAM | ≤ 3.0 | ≤ 5.0 | ≤ 9.5 |
|  | 64QAM | ≤ 4.5 | ≤ 5.0 | ≤ 9.5 |
|  | 256QAM | ≤ 7.0 | ≤ 7.0 | ≤ 9.5 |

- MPR for SLCA PSFCH

Proposal 9: For SL intra-band C-CA PSFCH MPR, consider Table 2.3-4.

Table 2.3-4 PSFCH MPR simulation results for SL Contiguous CA

| MPR for ratio (R) in bandwidth class B(dB) |  |  |
| --- | --- | --- |
| R ≤ 0. 33 | 0.33 < R ≤ 0. 55 | 0.55 < R ≤ 1.0 |
| 3.5 | 9.0 | 13.0 |
| Here, R = NGap/(NRB1+NRB2+ NGBchannel_CC1+ NGBchannel_CC2) |  |  |

- MPR for SLCA S-SSB

Proposal 10: Reuse Inner/Outer RB allocation and Inner/Outer1/Outer2 RB allocation of NR C-CA for SL C-CA S-SSB MPR.

Proposal 11: For SL intra-band C-CA S-SSB MPR, consider Table 2.4-4.

Table 2.4-4 S-SSB MPR simulation results for SL Contiguous CA

| MPR for bandwidth class B(dB) |  |  |
| --- | --- | --- |
| Inner | Outer1 | Outer2 |
| 3.5 | 9.0 | 13.0 |

- A-MPR for SLCA

Proposal 12: Define SL intra-band C-CA A-MPR for aggregated CBW of 40MHz with ’SL-CA NS_52’.

- A-MPR for SLCA PSSCH/PSCCH

Proposal 13: For SL-CA NS_52 PSSCH/PSCCH A-MPR, consider Table 2.6-4 for Contiguous RB allocation.

Table 2.6-4 SL CA NS_52 PSSCH/PSCCH A-MPR for SL Contiguous CA with Contiguous RB allocations

| Modulation |  | A-MPR for bandwidth class B(dB) |  |
| --- | --- | --- | --- |
|  |  | inner | outer |
| CP-OFDM | QPSK | ≤ 7.0 | ≤ 8.5 |
|  | 16QAM | ≤ 7.0 | ≤ 8.5 |
|  | 64QAM | ≤ 7.0 | ≤ 8.5 |
|  | 256QAM | ≤ 7.0 | ≤ 8.5 |

Proposal 14: For SL-CA NS_52 PSSCH/PSCCH A-MPR, consider Table 2.6-8 for Non-contiguous RB allocation.

Table 2.6-8 SL CA NS_52 PSSCH/PSCCH A-MPR for SL Contiguous CA with Non-contiguous RB allocations

| Modulation |  | A-MPR for bandwidth class B(dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Inner | Outer1 | Outer2 |
| CP-OFDM | QPSK | ≤ 3.0 | ≤ 8.0 | ≤ 13.5 |
|  | 16QAM | ≤ 3.0 | ≤ 8.0 | ≤ 13.5 |
|  | 64QAM | ≤ 4.5 | ≤ 8.0 | ≤ 13.5 |
|  | 256QAM | ≤ 7.0 | ≤ 8.0 | ≤ 13.5 |

- A-MPR for SLCA PSFCH

Proposal 15: For SL-CA NS_52 PSFCH A-MPR, consider Table 2.7-4.

Table 2.7-4 SL CA NS_52 PSFCH A-MPR for SL Contiguous CA

| Modulation |  | A-MPR for ratio (R) in bandwidth class B(dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | R ≤0. 1 | 0.1 < R ≤ 0. 55 | 0.55 < R ≤ 1.0 |
| CP-OFDM | QPSK | 4.0 | 17.0 | 19.0 |

- A-MPR for SLCA S-SSB

Proposal 16: For SL-CA NS_52 S-SSB A-MPR, consider Table 2.8-4.

Table 2.8-4 SL CA NS_52 S-SSB A-MPR for SL Contiguous CA

| A-MPR for bandwidth class B(dB) |  |  |
| --- | --- | --- |
| Inner | Outer1 | Outer2 |
| ≤ 9.0 | ≤ 13.0 | ≤ 16.5 |

- References

[1] R4-2314739, “WF on NR_SL_enh2_UERF_part3”, Huawei

[2] R4-2310312, “TP for TR 38.786 on the updated TR structure for NR SL CA operation”, Meta Ireland