3GPP TSG-RAN WG4 Meeting # 108bis														R4-2315536

# Xiamen, China, Oct 9 – Oct 13, 2023

Agenda item:			5.30.2.3 Sidelink CA

Source:					LG Electronics

Title:	TP for TR 38.786 on SLCA MPR and Configured transmitted power

Document for:	Approval

- Background

This contribution provides text proposal on MPR.

- Text Proposal

<<<<<<<<<<< Start of changes in section 1 >>>>>>>>>>

6.3.2.1 MPR for NR SL CA operation

6.3.2.1.1	MPR for simultaneous PSSCH/PSCCH transmission

For intra-band contiguous NR SL carrier aggregation, the allowed Maximum Power Reduction (MPR) for the maximum output power in 6.3.1-1 with contiguous RB allocation is specified in Table 6.3.2.1.1-1 for UE power class 3 CA bandwidth classes B. The Inner/Outer RB allocation is specified in 6.2A.2.1 in TS 38.101-1.

For intra-band contiguous carrier aggregation, the allowed Maximum Power Reduction (MPR) for the maximum output power in Table Table 6.3.1.1-1 with non-contiguous RB allocation is specified in Table 6.3.2.1.1-2 for UE power class 3 CA bandwidth classes B. The Inner/Outer1/Outer2 RB allocation is specified in 6.2A.2.1 in TS 38.101-1.

Table 6.3.2.1.1-1: PSSCH/PSCCH MPR simulation results for SL Contiguous CA with Contiguous RB allocations

| Modulation |  | MPR for bandwidth class B(dB) |  |
| --- | --- | --- | --- |
|  |  | inner | outer |
| CP-OFDM | QPSK | ≤ 3.0 | ≤ 5.0 |
|  | 16QAM | ≤ 3.0 | ≤ 5.0 |
|  | 64QAM | ≤ 4.5 | ≤ 5.0 |
|  | 256QAM | ≤ 7.0 | ≤ 7.0 |

Table 6.3.2.1.1-2: PSSCH/PSCCH MPR simulation results for SL Contiguous CA with Non-contiguous RB allocations

| Modulation |  | MPR for bandwidth class B(dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Inner | Outer1 | Outer2 |
| CP-OFDM | QPSK | ≤ 3.0 | ≤ 5.0 | ≤ 9.5 |
|  | 16QAM | ≤ 3.0 | ≤ 5.0 | ≤ 9.5 |
|  | 64QAM | ≤ 4.5 | ≤ 5.0 | ≤ 9.5 |
|  | 256QAM | ≤ 7.0 | ≤ 7.0 | ≤ 9.5 |

6.3.2.1.2	MPR for PSFCH transmission

For intra-band contiguous NR SL carrier aggregation, the allowed Maximum Power Reduction (MPR) for the maximum output power in 6.3.1-1 is specified in Table 6.3.2.1.2-1 for UE power class 3 CA bandwidth classes B.

Table 6.3.2.1.2-1 PSFCH MPR simulation results for SL Contiguous CA

| MPR for ratio (R) in bandwidth class B(dB) |  |  |
| --- | --- | --- |
| R ≤ 0. 33 | 0.33 < R ≤ 0. 55 | 0.55 < R ≤ 1.0 |
| 3.5 | 9.0 | 13.0 |
| Here, R = NGap/(NRB1+NRB2+ NGBchannel_CC1+ NGBchannel_CC2) |  |  |

Here, the aggregated CA bandwidth is CA bandwidth class B which is composed of 2 CCs (CC1 + CC2). The ratio of NGap/(NRB1+NRB2+ NGBchannel_CC1+ NGBchannel_CC2) is considered.

- R = NGap/(NRB1+NRB2+ NGBchannel_CC1+ NGBchannel_CC2)

  - NGap is the gap RB amount from CC1 RBstart  to CC2 RBend for SL contiguous CA when a single PSFCH or contiguous and non-contiguous allocation simultaneous PSFCHs is transmitted in each CC.

  - NGBchannel_CC1 and NGBchannel_CC2 is the number of RB which corresponds to the minimum guardbands for CC1 and CC2, respectively, which have been calculated using the following equation:

NGBchannel =  GBchannel / (SCS x 12)

GBchannel = (BWChannel - NRB x SCS x 12) / 2 - SCS/2,

where NRB are from Table 5.3.2-1 in TS38.101-1, and BWChannel is the channel bandwidth for each CC.

6.3.2.1.3	MPR for S-SSB transmission

For intra-band contiguous NR SL carrier aggregation, the allowed Maximum Power Reduction (MPR) for the maximum output power in 6.3.1-1 is specified in Table 6.3.2.1.3-1 for UE power class 3 CA bandwidth classes B. The Inner/Outer1/Outer2 RB allocation is specified in 6.2A.2.1 in TS 38.101-1.

Table 6.3.2.1.3-1 S-SSB MPR simulation results for SL Contiguous CA

| MPR for ratio (R) in bandwidth class B(dB) |  |  |
| --- | --- | --- |
| Inner | Outer1 | Outer2 |
| 3.5 | 9.0 | 13.0 |

6.3.3	UE additional maximum output power reduction for NR SL CA operation

For the additional emission limits in Europe in ITS spectrum, RAN4 already defined the regulatory requirements in TS38.101-1. The ETSI regulation shall be considered to derive the A-MPR requirements for NR SL CA UE. For the US related regulatory requirements will be studied after final FCC announcement for the additional emission limits in US.

To derive A-MPR requirements for SL CA operation in ITS spectrum, RAN4 will assume the basic simulation parameters and assumptions in section 6.3.2 and the additional spectrum emission mask in Table 6.5E.2.3.1-1 and additional spurious emission requirements in Table 6.5E.3.4.2-1 and Table 6.5E.3.4.2-2  in TS38.101-1 will be considered when NS_33 is configured from the pre-configured radio parameters in the geometrical region or indicated in the cell.

6.3.3.1 A-MPR for NR SL CA operation

<Editor Note> Detail A-MPR requirements will be added.

6.3.4	Configured transmitted power for NR SL CA operation

For NR SL carrier aggregation the UE is allowed to set its configured maximum output power PCMAX,c for serving cell c and its total configured maximum output power PCMAX.

The total configured maximum output power PCMAX shall be set within the following bounds:

PCMAX_L ≤ PCMAX ≤ PCMAX_H

For NR SL intra-band contiguous carrier aggregation when same slot pattern is used in all aggregated serving cells,

PCMAX_L  = MIN{10 log10 ∑ pEMAX,c , PEMAX, CA, PPowerClass,SL-CA – MAX(MPR, A-MPR) + ΔTIB,c, P-MPR,c ), PRegulatory,c }

PCMAX_H  = MIN{10 log10 ∑ pEMAX,c , PEMAX, CA, PPowerClass,SL-CA, PRegulatory,c }

where

-	PCMAX is configured for PSSCH\PSCCH, S-SSB and PSFCH, respectively;

-    pEMAX,c is the linear value of PEMAX,c in clause 6.2E.4 in TS38.101-1.

PPowerClass,SL-CA is the maximum UE power in Table 6.3.1-1 without taking into account the tolerance;

-	MPR and A-MPR are in clause 6.3.2 and 6.3.3, respectively;

-   TIB,c,  and P-MPRc are specified in clause 6.2.4 in TS38.101-1

-	P-MPR is the power management term for the UE;

-	TC is the highest value TC,c among all serving cells c;

-   PRegulatory,c= 10 - Gpost connector dBm the V2X UE is within the protected zone [ETSI TS 102 792] of CEN DSRC tolling system and operating in Band n47; PRegulatory,c= 33 - Gpost connector dBm otherwise

-	PEMAX,CA is the value indicated by [TBD].

TREF and Teval are specified in Table 6.3.4-1 when same slot patterns are used in aggregated carriers. For each TREF, the PCMAX_L is evaluated per Teval and given by the minimum value taken over the transmission(s) within the Teval; the minimum PCMAX_L over the one or more Teval is then applied for the entire TREF. The lesser of PPowerClass,SL-CA and PEMAX,CA shall not be exceeded by the UE during any period of time.

Table 6.3.4-1: PCMAX evaluation window for different slot and channel durations

| TREF | Teval | Teval with frequency hopping |
| --- | --- | --- |
| TREF of largest slot duration over both SL CCs | Physical channel length | Min(Tno_hopping, Physical Channel Length) |

The measured maximum output power PUMAX over all serving cells with same slot pattern shall be within the following range:

PCMAX_L  – MAX{TL, TLOW(PCMAX_L) }  ≤  PUMAX  ≤  PCMAX_H  +  THIGH(PCMAX_H)

PUMAX = 10 log10 ∑ pUMAX,c

where pUMAX,c  denotes the measured maximum output power for serving cell c expressed in linear scale. The tolerances TLOW(PCMAX) and THIGH(PCMAX) for applicable values of PCMAX are specified in Table 6.3.4-2. The tolerance TL is the absolute value of the lower tolerance for applicable NR SL CA configuration as specified in Table 6.3.1-1 for intra-band carrier aggregation.

Table 6.3.4-2: PCMAX,c tolerance schemes for NR SL intra-band contiguous CA

| PCMAX,c (dBm) | Tolerance TLOW(PCMAX_L,c) (dB) | Tolerance THIGH(PCMAX_H,c) (dB) |
| --- | --- | --- |
| 22 ≤ PCMAX,c ≤ 23 | 5.0 | 2.0 |
| 21 ≤ PCMAX,c < 22 | 5.0 | 3.0 |
| 20 ≤ PCMAX,c < 21 | 6.0 | 4.0 |
| 16 ≤ PCMAX,c < 20 | 5.0 |  |
| 11 ≤ PCMAX,c < 16 | 6.0 |  |
| -40 ≤ PCMAX,c < 11 | 7.0 |  |

<<<<<<<<<<< End of changes in section 1 >>>>>>>>>>

- References

[1] R4-2315532, “UE RF requirements of SL CA”, LG Electronics

[2] TR 38.786 v1.0.0