3GPP TSG-RAN WG4 Meeting #108bis		R4-2315537

# Xiamen, China, October 09 – October 13, 2023

Source:	LG Electronics

Title:	UE RF requirements for simultaneous DL reception

Agenda item:	5.7.1.3

Document for:	Discussion

Introduction

In this paper, we provide our views on UE RF requirements for simultaneous DL reception in FR2-1 based on the approved WF [1] and the simulation results.

Discussion

The followings were agreed in [1].

1.1 Combining method to compute Pdirectional in metric - Companies are encouraged to provide analysis of pros & cons for each combining method - Option 1 – arithmetic mean - Option 2 – OR combining 1.2 AoA offsets for the UE RF requirement - The UE only needs to meet the requirement for 1 AoA offset. - Options: - 1. Define a requirement for each candidate AoA offset. - 2. The requirement is defined for just 1 AoA offset. - Companies are encouraged to provide the simulation results of at least 3 UE implementations (panels facing opposite directions, panels in adjacent sides, panels in same side). The results of both OR combing and arithmetic mean are expected to be provided. - The results of each AoA offset should be based on the best UE orientation, the best UE orientation can be different for each AoA offset. 1.3 Alternative calibration for a UE model - Besides the previously agreed calibration condition (UE meets REFSENS as well as EIS spherical coverage), an alternative calibration method for UE models is below: - The DL power level used in simulation will be adjusted to UE’s EIS CCDF@50%-xile. - This means the simulation results for PC3 provided by companies should not exceed 50% for any AoA offset. - The performance of UE model must meet the legacy REFSENS and EIS spherical coverage requirement 1.4 Details of V/H polarization signal receiving in simulation - For rank1 DL + MRC - The procedure above is used to calculate signal and interference levels when gain imbalance exists between V-pol and H-pol in UE module, along with the alternative calibration method for UEs. - When the procedure above is used for determining interference level, it implies that the interference is also optimally combined, i.e. it represents a worst case interferer polarization condition. - Companies are encouraged to further check the impact of imbalance between V/H in the final requirement based on the procedure above

Simulation assumptions:

- UE panel implementation

  - panel_A = panels facing opposite directions

  - panel_B = panels in adjacent sides

  - panel_C = panels in same side

- Antenna module performance

  - Case 1: antenna module#1 and antenna module#2 are assumed to have the same performance gain

  - Case 2: antenna module#1 is assumed to have a 3dB lower performance gain than antenna module#2

  - Case 3: antenna module#2 is assumed to have a 3dB lower performance gain than antenna module#1

- UE orientation

  - Z-axis oriented

  - Y-axis oriented

  - X-axis oriented

    - Probes are located in the xz plane in [2]

For each panel_A, panel_B, and panel_C, the pass ratio can be calculated as follows based on the [1].

- pass ratio = min(Case1 pass ratio, Case2 pass ratio, Case3 pass ratio)

  - Case1 pass ratio = max (X-axis oriented pass ratio, Y-axis oriented pass ratio, Z-axis oriented pass ratio)

  - Case2 pass ratio = max (X-axis oriented pass ratio, Y-axis oriented pass ratio, Z-axis oriented pass ratio)

  - Case3 pass ratio = max (X-axis oriented pass ratio, Y-axis oriented pass ratio, Z-axis oriented pass ratio)

Table 2.1 and Table 2.2 show the Pass ratio of ‘arithmetic mean’ and ‘OR combining’ respectively.

- Table 2.1: Pass ratio (%) with ‘arithmetic mean’

|  | 30o | 60o | 90o | 120o | 150o | 180o |
| --- | --- | --- | --- | --- | --- | --- |
| panels facing opposite directions | 0.2 | 6.5 | 14.9 | 23.9 | 27.3 | 29.3 |
| panels in adjacent sides | 5.3 | 10.3 | 14.8 | 16.6 | 18.5 | 18.7 |
| panels in same side | 13.2 | 8.7 | 13.5 | 8.7 | 1.3 | 0.0 |

- Table 2.2: Pass ratio with ‘OR’

|  | 30o | 60o | 90o | 120o | 150o | 180o |
| --- | --- | --- | --- | --- | --- | --- |
| panels facing opposite directions | 0.4 | 13.0 | 29.7 | 47.7 | 54.7 | 58.6 |
| panels in adjacent sides | 10.6 | 20.6 | 29.5 | 33.2 | 36.9 | 37.4 |
| panels in same side | 26.4 | 17.5 | 27.0 | 17.5 | 2.7 | 0.0 |

Table 2.3 shows the best Pass Ratio and the worst Pass Ratio between different panels which are highlighted with yellor color and blue color respectively.

- Table 2.3: Best/Worst pass ratio

| AoA Offset (Degree) | Best Pass Ratio (%) |  | Worst Pass Ratio (%) |  |
| --- | --- | --- | --- | --- |
|  | Arithmetic mean | OR combining | Arithmetic mean | OR combining |
| 30 | 13.2 | 26.4 | 0.2 | 0.4 |
| 60 | 10.3 | 20.6 | 6.5 | 13.0 |
| 90 | 14.9 | 29.7 | 13.5 | 27.0 |
| 120 | 23.9 | 47.7 | 8.7 | 17.5 |
| 150 | 27.3 | 54.7 | 1.3 | 2.7 |
| 180 | 29.3 | 58.6 | 0.0 | 0.0 |

Table 2.4 shows the difference of Pass Ratio between above 3 panel implementations.

- Table 2.4: Difference of Pass Ratio between panel implementationso

| AoA Offset (Degree) | Difference of Pass Ratio |  |
| --- | --- | --- |
|  | Arithmetic mean | OR combining |
| 30 | 13.0 | 26.0 |
| 60 | 3.8 | 7.6 |
| 90 | 1.4 | 2.7 |
| 120 | 15.1 | 30.2 |
| 150 | 26.0 | 52.0 |
| 180 | 29.3 | 58.6 |

The followings are observed.

Observation 1: Pass ratio of OR combining method is higher than that of arithmetic mean method.

Observation 2: The difference of Pass ratio between AOA offsets is lower in arithmetic mean method.

Observation 3: The difference of Pass ratio between panel implementations is the lowest in AOA offset of 90o.

For options of the requirements,

- Option 1: Define a requirement for each candidate AoA offset

- Option 2: The requirement is defined for just 1 AoA offset

For option 1, it is recommended to define the requirement for each candidate AOA offset considering the best Pass Ratio.

For option 2, it is recommended to define the requirement for just AOA offset of 90o because the difference of Pass Ratio between panel implementation is the lowest as observation 3.

Proposal 1: If option 1 is considered, define the requirement for each candidate AOA offset considering the best Pass Ratio. Table 2.5 can be one example of the requirement.

Proposal 2: If option 2 is considered, define the requirement for just AOA offset of 90o. Table 2.6 can be one example of the requirement.

For example, the requirement can be as Table 2.5 for option 1, and as Table 2.6 for option 2 by conider implementation margin.

- Table 2.5: Pass ratio for option 1

| AoA Offset (Degree) | Pass Ratio (%) |  |
| --- | --- | --- |
|  | Arithmetic mean | OR combining |
| 30 | 11 | 24 |
| 60 | 8 | 18 |
| 90 | 12 | 27 |
| 120 | 22 | 45 |
| 150 | 25 | 50 |
| 180 | 25 | 50 |

- Table 2.6: Pass ratio for option 2

| AoA Offset (Degree) | Pass Ratio (%) |  |
| --- | --- | --- |
|  | Arithmetic mean | OR combining |
| 90 | 12 | 27 |

Conclusion

In this contribution, we provided our views on the enhanced FR2-1 UE RF Rx requirements from the simulation results based on the WF[1]. Related observations and proposals are as follows.

Observation 1: Pass ratio of OR combining method is higher than that of arithmetic mean method.

Observation 2: The difference of Pass ratio between AOA offsets is lower in arithmetic mean method.

Observation 3: The difference of Pass ratio between panel implementations is the lowest in AOA offset of 90o.

- For options of the requirements

- Option 1: Define a requirement for each candidate AoA offset

- Option 2: The requirement is defined for just 1 AoA offset

Proposal 1: If option 1 is considered, define the requirement for each candidate AOA offset considering the best Pass Ratio. Table 2.5 can be one example of the requirement.

Proposal 2: If option 2 is considered, define the requirement for just AOA offset of 90o. Table 2.6 can be one example of the requirement.

- Table 2.5: Pass ratio for option 1

| AoA Offset (Degree) | Pass Ratio (%) |  |
| --- | --- | --- |
|  | Arithmetic mean | OR combining |
| 30o | 11 | 24 |
| 60 o | 8 | 18 |
| 90 o | 12 | 27 |
| 120 o | 22 | 45 |
| 150 o | 25 | 50 |
| 180 o | 25 | 50 |

- Table 2.6: Pass ratio for option 2

| AoA Offset (Degree) | Pass Ratio (%) |  |
| --- | --- | --- |
|  | Arithmetic mean | OR combining |
| 90 o | 12 | 27 |

- References

[1] R4-2314668, “WF on FR2_multiRx_UERF”, Apple

[2] R4-2302522, “System Parameter Assumptions for Multi-AoA Rx Testing”, Keysight Technologies

[3] R4-2307482, “UE RF requirements for simultaneous DL”, LG Electronics