3GPP TSG-RAN4 Meeting #108bis	R4-2315538

# Xiamen, CN, 9th Oct 2023 - 13th Oct 2023

Source:	LG Electronics

Title:	MSD analysis for 3Tx

Agenda item:	5.5.3.2

Document for:	Discussion

Introduction

In the last RAN4#108 meeting, the WF[1] on 3Tx and 4Rx requirements was approved and we could reach the many agreements, but some issues are still FSS. We would like to share our MSD analysis for CA_n26-n78 (PC2), CA_n41-n71 (PC1.5) and CA_n25-n77(PC1.5).

Discussion

CA_n26-n78 PC2 IMD 4

Fig 1. The architecture for CA_n26-n78

Table 1 Parameters for the CA_n26-n78 IMD4 MSD ananlysis (dB)

| Antenna isolation, dB | 10 |
| --- | --- |
| Front end loss, dB (FDD) | 4 |
| Front end loss, dB (TDD) | 5 |
| duplexer up/down isolation | 50 |
| duplexer high frequency band isolation | 15 |
| Diplexer H/L band isolation | 15 |
| Filter isolation | 30 |
| PCB isolation | 60 |
| PA forward IP4 | 30 |
| PA reverse IP4 | 30 |

Table 2 Main/Div total IMD 4 power and MSD value

| Main total IMD 4 | -60 dBm |
| --- | --- |
| Div total IMD 4 | -77 dBm |
| MSD after MRC at SNR = -1 (with 1.5 dB margin) | 21 dB |

Proposal 1: The IMD4 MSD for 3Tx CA_n26A-n78A with PC2 is 21 dB

CA_n71-n41 PC1.5 IMD4

Fig 2. The architecture for CA_n41-n71

Table 3 Parameters for the CA_n71-n41 IMD4 MSD ananlysis (dB)

| Antenna isolation, dB | 10 |
| --- | --- |
| Front end loss, dB (FDD) | 4 |
| Front end loss, dB (TDD) | 5 |
| duplexer up/down isolation | 50 |
| duplexer high frequency band isolation | 15 |
| Diplexer H/L band isolation | 15 |
| Filter isolation | 30 |
| PCB isolation | 60 |
| PA forward IP4 | 30 |
| PA reverse IP4 | 30 |

Table 4 Main/Div total IMD 4 power and MSD value

| Main total IMD 4 | -55.2 dBm |
| --- | --- |
| Div total IMD 4 | -72.2 dBm |
| MSD after MRC at SNR = -1 (with 1.5 dB margin) | 25.4 dB |

Proposal 2: The IMD4 MSD for 3Tx CA_n71A-n41A with PC2 is 25.4 dB

CA_n25A-n77A PC1.5 IMD 2/4

Fig 2. The architecture for CA_n25-n77

Table 5 Parameters for the CA_n71-n41 IMD4 MSD ananlysis (dB)

| Antenna isolation, dB | 10 |
| --- | --- |
| Front end loss, dB (FDD) | 4 |
| Front end loss, dB (TDD) | 5 |
| duplexer up/down isolation | 50 |
| duplexer high frequency band isolation | 15 |
| Diplexer H/L band isolation | 15 |
| Filter isolation | 30 |
| PCB isolation | 60 |
| PA forward IP4 | 30 |
| PA reverse IP4 | 30 |
| PA forward IP2 | 28.5 |
| PA reverse IP2 | 40 |

Table 6 Main/Div total IMD 2 power and MSD value

| Main total IMD 2 | -51.6 dBm |
| --- | --- |
| Div total IMD 2 | -58.9 dBm |
| MSD after MRC at SNR = -1 (with 1.5 dB margin) | 37.3 dB |

Table 7 Main/Div total IMD 2 power and MSD value

| Main total IMD 4 | -55.2 dBm |
| --- | --- |
| Div total IMD 4 | -72.2 dBm |
| MSD after MRC at SNR = -1 (with 1.5 dB margin) | 24.7 dB |

Proposal 3: The IMD2/4 MSD for 3Tx CA_n25A-n77A with PC2 are 37.3 dB/24.7 dB respectively.

Conclusion

Proposal 1: The IMD4 MSD for 3Tx CA_n26A-n78A with PC2 is 21 dB

Proposal 2: The IMD4 MSD for 3Tx CA_n71A-n41A with PC2 is 25.4 dB

Proposal 3: The IMD2/4 MSD for 3Tx CA_n25A-n77A with PC2 are 37.3 dB/24.7 dB respectively.

- References

[1]R4-2314705, WF on 3Tx and 4Rx requirements (OPPO)