3GPP TSG-RAN4 Meeting #108bis	R4-2315540

# Xiamen, CN, 9th Oct 2023 - 13th Oct 2023

Source:	LG Electronics

Title:	Discussion for FR2-1 UL256QAM MPR

Agenda item:	5.6.2

Document for:	Discussion

Introduction

In the last RAN4#108 meeting, the WF[1] was approved, which includes MPR simulation assumptions and PTRS configuration. In this contribution, we would like to share our additional MPR simulation results, and propose the MPR value for FR2-1 UL256QAM PC1. In addition, we share our point of view for new issue “What’s about the MPR for 39 GHz” based on our simulation results.

Discussion

MPR simulation results for FR2-1 UL256QAM PC1

Table 1 phase noise profiles

For 29 GHz phase noise profile - option 2 (QC’s) % fz = [3e3 7e5 9e5]; % fp = [1 1e6 1.1e6]; % alphaz = [2.37 4.7 2]; % alphap = [3.3 5.3 2.5]; % PSD0 = 33; For 39 GHz phase noise profi - option 4 (MTK’s) % fz = [3e3 620e3 240e6]; % fp = [1 1.6e6 30e6]; % alphaz = [2.37 2.7 2.53]; % alphap = [3.3 3.3 1]; % PSD0 = 31.76;

At the last RAN4#108 meeting, we provided our MPR simulation results[2] for FR2-1 UL256QAM PC2. We also ran the MPR simulation for PC1 in a similar manner, and the results are shown in Table 2 and 3.

Observation 1: The lower MPR value with low AM-PM distortion PA than MPR value with general PA is observed.

Table 2 Simulated MPR results for 29 GHz PC1 UL256QAM

|  | PA model | PN Profile | Region 1 [22, 20] | Region 2 [16, 32] | Outer 1 [0, 64] | Outer 2 [0, 10] |
| --- | --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM (CBW: 100 MHz) | General | Option 2 | 7.52 | 7.30 | 7.73 | 6.89 |
|  | Low AM-PM distortion | Option 2 | 6.03 | 6.02 | 6.04 | 6.36 |
|  | PA model | PN Profile | Region 1 [22, 22] | Region 2 [16, 32] | Outer 1 [0, 66] | Outer 2 [0, 10] |
| CP-OFDM (CBW: 100 MHz) | General | Option 2 | 8.89 | 9.54 | 9.53 | 8.90 |
|  | Low AM-PM distortion | Option 2 | 8.67 | 8.67 | 8.67 | 8.05 |
|  | PA model | PN Profile | Region 1 [60, 60] | Region 2 [60, 120] | Outer 1 [0, 240] | Outer 2 [0, 40] |
| DFT-s-OFDM (CBW: 400 MHz) | General | Option 2 | 8.03 | 8.44 | 8.86 | 7.93 |
|  | Low AM-PM distortion | Option 2 | 7.62 | 7.82 | 7.52 | 8.03 |
|  | PA model | PN Profile | Region 1 [60, 60] | Region 2 [60, 132] | Outer 1 [0, 264] | Outer 2 [0, 40] |
| CP-OFDM (CBW: 400 MHz) | General | Option 2 | 10.43 | 11.08 | 10.75 | 10.34 |
|  | Low AM-PM distortion | Option 2 | 10.11 | 10.65 | 10.65 | 10.12 |

[start RB position, allocated RB number]

Table 3 Simulated MPR results for 39 GHz PC1 UL256QAM

|  | PA model | PN Profile | Region 1 [22, 20] | Region 2 [16, 32] | Outer 1 [0, 64] | Outer 2 [0, 10] |
| --- | --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM (CBW: 100 MHz) | General | Option 4 | 7.62 | 7.62 | 7.73 | 6.46 |
|  | Low AM-PM distortion | Option 4 | 6.25 | 5.83 | 6.25 | 6.36 |
|  | PA model | PN Profile | Region 1 [22, 22] | Region 2 [16, 32] | Outer 1 [0, 66] | Outer 2 [0, 10] |
| CP-OFDM (CBW: 100 MHz) | General | Option 4 | 9.75 | 9.75 | 9.65 | 8.78 |
|  | Low AM-PM distortion | Option 4 | 9.32 | 8.55 | 9.10 | 8.15 |
|  | PA model | PN Profile | Region 1 [60, 60] | Region 2 [60, 120] | Outer 1 [0, 240] | Outer 2 [0, 40] |
| DFT-s-OFDM (CBW: 400 MHz) | General | Option 4 | 8.13 | 8.86 | 9.18 | 8.03 |
|  | Low AM-PM distortion | Option 4 | 7.82 | 8.13 | 8.44 | 7.72 |
|  | PA model | PN Profile | Region 1 [60, 60] | Region 2 [60, 132] | Outer 1 [0, 264] | Outer 2 [0, 40] |
| CP-OFDM (CBW: 400 MHz) | General | Option 4 | 10.64 | 11.38 | 11.96 | 10.55 |
|  | Low AM-PM distortion | Option 4 | 10.32 | 10.87 | 10.86 | 10.22 |

[start RB position, allocated RB number]

Table 4 MPR results for 29 GHz PC1 UL256QAM with margin

| CBW (100 MHz) | PA model | P/N profile | Region 1 | Region 2 | Outer |
| --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM | General | Option 2 | 9 | 9 | 9 |
|  | Low AM-PM distortion | Option 2 | 7.5 | 7.5 | 8 |
| CP-OFDM | General | Option 2 | 10.5 | 11 | 11 |
|  | Low AM-PM distortion | Option 2 | 10 | 10 | 10 |
| CBW (400 MHz) | PA model | P/N profile | Region 1 | Region 2 | Outer |
| DFT-s-OFDM | General | Option 2 | 9.5 | 10 | 10.5 |
|  | Low AM-PM distortion | Option 2 | 9 | 9.5 | 9.5 |
| CP-OFDM | General | Option 2 | 12 | 12.5 | 12.5 |
|  | Low AM-PM distortion | Option 2 | 11.5 | 12 | 12 |

[start RB position, allocated RB number]

Table 5 MPR results for 39 GHz PC1 UL256QAM with margin

| CBW (100 MHz) | PA model | P/N profile | Region 1 | Region 2 | Outer |
| --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM | General | Option 4 | 9 | 9 | 9 |
|  | Low AM-PM distortion | Option 4 | 8 | 7.5 | 8 |
| CP-OFDM | General | Option 4 | 11.5 | 11.5 | 11 |
|  | Low AM-PM distortion | Option 4 | 11 | 10 | 10.5 |
| CBW (400 MHz) | PA model | P/N profile | Region 1 | Region 2 | Outer |
| DFT-s-OFDM | General | Option 4 | 9.5 | 10.5 | 10.5 |
|  | Low AM-PM distortion | Option 4 | 9.5 | 9.5 | 10 |
| CP-OFDM | General | Option 4 | 12 | 13 | 13.5 |
|  | Low AM-PM distortion | Option 4 | 12 | 12.5 | 12.5 |

[start RB position, allocated RB number]

Table 4 and table 5 show the MPR value with the margin. We observed the MPR values using Low AM-PM distortion PA are lower than MPR value with general PA model. But, the characteristic of low AM-PM distortion PA is from our mmWave products, so we need to more margin for the MPR valu with low AM-PM distortion. So, we propose the MPR value as the average of the MPR results using the general PA and the MPR results using the low AM-PM distortion PA.

Proposal 1: Suggest the MPR value for FR2-1 UL256QAM PC1. (29 GHz: Table 6, 39 GHz: Table 7)

Table 6 The proposed MPR values for 29 GHz PC1 UL256QAM

| CBW (100 MHz) | Region 1 | Region 2 | Outer |
| --- | --- | --- | --- |
| DFT-s-OFDM | 8.5 | 8 | 8.5 |
| CP-OFDM | 10.5 | 10.5 | 10.5 |
| CBW (400 MHz) | Region 1 | Region 2 | Outer |
| DFT-s-OFDM | 9.5 | 9.5 | 10 |
| CP-OFDM | 12 | 12.5 | 12 |

Table 7 The proposed MPR values for 39 GHz PC1 UL256QAM

| CBW (100 MHz) | Region 1 | Region 2 | Outer |
| --- | --- | --- | --- |
| DFT-s-OFDM | 8.5 | 8 | 8.5 |
| CP-OFDM | 11 | 10.5 | 11 |
| CBW (400 MHz) | Region 1 | Region 2 | Outer |
| DFT-s-OFDM | 9.5 | 10 | 10.5 |
| CP-OFDM | 12 | 12.5 | 13 |

New issue: What’s about the MPR for 39GHz

New issue: What’s about the MPR for 39GHz? - Proposals - Option 1: Defined the same MPR with 29GHz - Option 2: Consider some margin for 39GHz because the phase noise profile performance between 29GHz and 39GHz is different - Option 3: Others Agreement: - FFS

At the last meeting, we rasised the new issue of “What’s about the MPR for 39GHz” due to the difference in MPR values between 29 GHz and 39 GHz frequency bands. We ran the MPR simulations for the 29 and 39 GHz respectively, and we observed that the 39 GHz MPR values are higher than the 29 GHz MPR values about 0~1 dB. So, we need to discuss that how to deal with this frequency dependency.

Proposal 2: Need to discuss how to deal with the frequency dependency for FR2-1 UL256QAM.

Conclusion

Observation 1: The lower MPR value with low AM-PM distortion PA than MPR value with general PA is observed.

Proposal 1: Suggest the MPR value for FR2-1 UL256QAM PC1. (29 GHz: Table 6, 39 GHz: Table 7)

Table 6 The proposed MPR values for 29 GHz PC1 UL256QAM

| CBW (100 MHz) | Region 1 | Region 2 | Outer |
| --- | --- | --- | --- |
| DFT-s-OFDM | 8.5 | 8 | 8.5 |
| CP-OFDM | 10.5 | 10.5 | 10.5 |
| CBW (400 MHz) | Region 1 | Region 2 | Outer |
| DFT-s-OFDM | 9.5 | 9.5 | 10 |
| CP-OFDM | 12 | 12.5 | 12 |

Table 7 The proposed MPR values for 39 GHz PC1 UL256QAM

| CBW (100 MHz) | Region 1 | Region 2 | Outer |
| --- | --- | --- | --- |
| DFT-s-OFDM | 8.5 | 8 | 8.5 |
| CP-OFDM | 11 | 10.5 | 11 |
| CBW (400 MHz) | Region 1 | Region 2 | Outer |
| DFT-s-OFDM | 9.5 | 10 | 10.5 |
| CP-OFDM | 12 | 12.5 | 13 |

Proposal 2: Need to discuss how to deal with the frequency dependency for FR2-1 UL256QAM.

- References

# [1] R4-2314677, WF on UL 256QAM requirements (Xiaomi)

# [2] R4-2312237, Discussion for FR2-1 UL256QAM MPR (LGE)

# APPENDIX

Table 8 The proposed MPR values for 29 GHz PC2 UL256QAM [2]

| CBW (100 MHz) | Inner RB allocations, Region 1 | Edge RB allocations |
| --- | --- | --- |
| DFT-s-OFDM | 8 | 8 |
| CP-OFDM | 10.5 | 10.5 |
| CBW (400 MHz) | Inner RB allocations, Region 1 | Edge RB allocations |
| DFT-s-OFDM | 10 | 9.5 |
| CP-OFDM | 11.5 | 11.5 |

Table 9 The proposed MPR values for 39 GHz PC2 UL256QAM[2]

| CBW (100 MHz) | Inner RB allocations, Region 1 | Edge RB allocations |
| --- | --- | --- |
| DFT-s-OFDM | 8 | 8.5 |
| CP-OFDM | 10.5 | 10.5 |
| CBW (400 MHz) | Inner RB allocations, Region 1 | Edge RB allocations |
| DFT-s-OFDM | 10 | 9.5 |
| CP-OFDM | 12 | 12 |