# 3GPP TSG-RAN WG4 Meeting # 108 bis		R4-2315565

# Xiamen, China, October 9 – 13, 2023

Source:	Sony

Title:	Views on low-power wake-up signal and receiver for NR

Agenda item:	5.20.2

Document for:	Approval

- Background

Low-power wake-up signal and receiver for NR was discussed in RAN4 #108. A WF has been approved [1]. Latest Status Report to TSG is found in [2] and latest revised SID is found in [3].

In this tdoc we will further discuss aspects of different wake-up receiver architectures.

- Consideration of wake-up receiver architectures

The SID [3] was updated after RAN #101 to focus on OOK and OFDM based wake-up signals. The technical report [4] discuss studies of different WUS designs and WUR designs together with its KPIs. To balance energy consumption versus coverage and network resources is the crux of the WUS/WUR design.

Observation 1	There is a delicate balance between complexity/energy consumption and coverage and network resources.

The coverage is determined by a combination of the LP-WUR design and the LP-WUS design, since coverage depends on the LP-WUR receiver noise figure and the required SNR, which in turn depends on the LP-WUS design.

Observation 2	The coverage is determined by a combination of the LP-WUR design and the LP-WUS design.

The main target for low-power LP-WUS/WUR are power-sensitive, small form-factor devices including IoT use cases (such as industrial sensors, controllers) which in many cases are static or low mobility devices. It may be a waste of precious resources (device current consumption and/or network resources) to design the system always using a set-up for the worst scenario in terms of coverage and/or channel property. The LP-WUR could adapt its sensitivity level according to the prevailing situation (i.e. deployment for a device with very low or zero mobility) in order not to consume unnecessary power. A capability for signaling this feature to the base station may be needed.

Proposal 1	The LP-WUR could adapt its sensitivity level according to the prevailing situation in order not to consume unnecessary power.

- Conclusion

In this contribution, we have discussed low-power wake-up signal and receiver for NR. The following observations and proposal have been made:

Observation 1	There is a delicate balance between complexity/energy consumption and coverage and network resources.

Observation 2	The coverage is determined by a combination of the LP-WUR design and the LP-WUS design.

Proposal 1	The LP-WUR could adapt its sensitivity level according to the prevailing situation in order not to consume unnecessary power.

- References

R4-2314932		“WF on FS_NR_LPWUS”, vivo

RP-231812		“Status report for SI Study on low-power wake-up signal and receiver”, vivo

RP-232672		“Revised SID: Study on low-power Wake-up Signal and Receiver for NR”, vivo

TR 38.869		“Study on low-power wake up signal and receiver for NR”

R4-2313199		“Views on low-power wake-up signal and receiver for NR”, Sony