3GPP TSG-RAN WG4 #108bis	R4-2315582

# Xiamen, China, October 09 – October 13, 2023

Agenda item:	5.4.1.1

Source: 	Qualcomm Incorporated

Title: 	Channel Raster Enhancements

Document for:	Discussion

# Introduction

The channel raster enhancements to accommodate nested channels in bands with the 100kHz channel raster has been under discussion for several meetings. In RAN4#108, a WF listing only a few options was agreed in [1]. In this paper we further discuss this issue and present a proposal on how to capture the new channel raster granularity in the specifications. We also analyze the impact of the new raster to other requirements.

# Discussion

## Channel Raster Enhancements

In [1] it was agreed that to introduce 10kHz channel raster granularity if approach 1 is chosen. As already analyzed in [2], this is the simplest option with the least impact to the specifications.

Proposal 1. Adopt Approach 1: Specify a new channel raster [1] with a step size of 10kHz.

This change should be operated to both UE and BS specifications such that there is complete alignment between the specifications. As also discussed in [3], the capability should be defined per band based on operator requests. Adopting Approach 1 would also make the introduction of this feature per band simpler than Approach 2.

Proposal 2: The new channel raster entries should be specified for both UE and base station based on operator requests.

The specification changes needed to enable Approach 1 are given in the Annex. As can be seen, this approach leads to the simplest changes.

Observation 1: Adding new channel raster points leads to the simplest specification changes.

## Impact to other requirements

Other potential specifications/requirements impacted by changes to the channel raster are channel spacing and the synchronization raster.

Channel spacing

The channel spacing defines the space between channel centers and is used also as a criterion to differentiate intra-band contiguous and non-contiguous CA. It would be highly desirable not to modify this requirement such that there is no impact to the CA specifications or legacy UEs. The new channel raster is introduced to enable placing of a narrower channel inside a wider channel in the bands bands using a 100kHz raster. The vast majority of these bands are FDD and it is not expected that intra-band contiguous CA would be used with nested channels. In this case, a new channel bandwidth covering the entire spectrum holding of the operator would be introduced (the problem that triggered this entire work is the need to use larger spectrum holdings with a single channel). Hence, since there is no need to support intra-band contiguous CA with narrower channels, no changes are needed to the channel spacing specifications. In the case of intra-band non-contiguous CA(separate spectrum holdings), it can be further discussed if there is a need to modify the specifications, however, this is unlikely.

Observation 2: Intra-band contiguous CA with nested channels is not needed.

Proposal 3: Do not make any changes to the channel spacing specifications.

Sync raster

As explained in [5], the NR synchronization raster was designed to cover only channels placed on the 100kHz raster. With the introduction of additional channel raster points, there will be some channels (5MHz) that would not be covered by the channel raster(there would not be any channel raster point where a SSB could be placed such that it would still fit inside the channel).

However, considering that this narrow channel would be nested inside a wider channel(otherwise the channel could simply be placed on the 100kHz “legacy” raster), it could be placed such that it overlaps the SSB of the wider channel. Hence, in practice, there would be no problem with finding a placement even for a 5MHz channel placed on the enhanced channel raster such that the SSB can be fit within the channel.

Observation 3: Sync raster will not cover all the channels placed on the enhanced channel raster, however, in practice, an SSB will always fit within a channel placed on the enhanced channel raster.

Taking into account the considerations above, it might be useful to introduce a note in the table with the sync raster definition or a sentence in the body of the clause defining the sync raster to clarify that the sync raster does not cover all the channels placed on the enhanced channel raster.

Proposal 4: Introduce some text in the sync raster specifications to clarify that they sync raster does not cover all the channels placed on the enhanced channel raster.

# Conclusion

In this paper we analyzed the channel raster enhancement options and its impact. We made the following proposals and observations:

Proposal 1. Adopt Approach 1: Specify a new channel raster [1] with a step size of 10kHz.

Proposal 2: The new channel raster entries should be specified for both UE and base station based on operator requests.

Observation 1: Adding new channel raster points leads to the simplest specification changes.

Observation 2: Intra-band contiguous CA with nested channels is not needed.

Proposal 3: Do not make any changes to the channel spacing specifications.

Observation 3: Sync raster will not cover all the channels placed on the enhanced channel raster, however, in practice, an SSB will always fit within a channel placed on the enhanced channel raster.

Proposal 4: Introduce some text in the sync raster specifications to clarify that they sync raster does not cover all the channels placed on the enhanced channel raster.

The text could be a note in the sync raster definition table or a paragraph in the clause containing the sync raster definition.

# References

- R4-2314681, “WF on Channel Raster Enhancements”, Ericsson, RAN4#107

- R4-2312439, “Channel Raster Enhancements”, Qualcomm Incorporated, RAN4#108

- R4-2315581, “Channel Raster Enhancement Capability”, Qualcomm Incorporated, RAN4#108Bis

- R4-2308787, “Channel Raster Enhancements”, Qualcomm Incorporated, RAN4#107

- R4-2305517, “Channel Raster Enhancements”, Qualcomm Incorporated, RAN4#106-bis-e

# Annex

Specification changes needed to enable 10kHz channel raster granularity for Bands n1,n2,n3,n5,n7. Different channel raster granularity can be enable with a corresponding step size. Support for each band is indicated through Note 5.

#### 5.4.2.3	Channel raster entries for each operating band

The RF channel positions on the channel raster in each NR operating band are given through the applicable NR-ARFCN in Table 5.4.2.3-1, using the channel raster to resource element mapping in clause 5.4.2.2.

For NR operating bands with 100 kHz channel raster, ΔFRaster = 20 × ΔFGlobal. In this case every 20th NR-ARFCN within the operating band are applicable for the channel raster within the operating band and the step size for the channel raster in Table 5.4.2.3-1 is given as <20>.

For NR operating bands with 100 kHz channel raster, ΔFRaster = 2 × ΔFGlobal is supported if the UE is capable of  “enhancedChannelRasterSupport-rel18”. In this case every 10th NR-ARFCN within the operating band are applicable for the channel raster within the operating band and the step size for the channel raster in Table 5.4.2.3-1 is <10>.

For NR operating bands with 15 kHz channel raster below 3GHz, ΔFRaster = I × ΔFGlobal, where I ϵ {3,6}. Every Ith NR-ARFCN within the operating band are applicable for the channel raster within the operating band and the step size for the channel raster in Table 5.4.2.3-1 is given as < I >.

For NR operating bands with 15 kHz channel raster above 3GHz, ΔFRaster = I × ΔFGlobal, where I ϵ {1,2}. Every Ith  NR-ARFCN within the operating band are applicable for the channel raster within the operating band and the step size for the channel raster in table 5.4.2.3-1 is given as <I>.

In frequency bands with two or more ΔFRaster, the higher ΔFRaster: For 15 kHz and 30 kHz channel raster applies to channels using only the SCS that is equal to or larger than the higher ΔFRaster and SSB SCS is equal to the higher ∆FRaster.

Table 5.4.2.3-1: Applicable NR-ARFCN per operating band

| NR operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n15 | 100 | 384000 – <20> – 396000 | 422000 – <20> – 434000 |
| n25 | 100 | 370000 – <20> – 382000 | 386000 – <20> – 398000 |
| n35 | 100 | 342000 – <20> – 357000 | 361000 – <20> – 376000 |
| n55 | 100 | 164800 – <20> – 169800 | 173800 – <20> – 178800 |
| n75 | 100 | 500000 – <20> – 514000 | 524000 – <20> – 538000 |
| n8 | 100 | 176000 – <20> – 183000 | 185000 – <20> – 192000 |
| n12 | 100 | 139800 – <20> – 143200 | 145800 – <20> – 149200 |
| n13 | 100 | 155400 – <20> – 157400 | 149200 – <20> – 151200 |
| n14 | 100 | 157600 – <20> – 159600 | 151600 – <20> – 153600 |
| n18 | 100 | 163000 – <20> – 166000 | 172000 – <20> – 175000 |
| n20 | 100 | 166400 – <20> – 172400 | 158200 – <20> – 164200 |
| n24 | 100 | 325300 – <20> – 332100 | 305000 – <20> – 311800 |
| n25 | 100 | 370000 – <20> – 383000 | 386000 – <20> – 399000 |
| n26 | 100 | 162800 – <20> – 169800 | 171800 – <20> – 178800 |
| n28 | 100 | 140600 – <20> – 149600 | 151600 – <20> – 160600 |
| n29 | 100 | N/A | 143400 – <20> – 145600 |
| n30 | 100 | 461000 – <20> – 463000 | 470000 – <20> – 472000 |
| n34 | 100 | 402000 – <20> – 405000 | 402000 – <20> – 405000 |
| n38 | 100 | 514000 – <20> – 524000 | 514000 – <20> – 524000 |
| n39 | 100 | 376000 – <20> – 384000 | 376000 – <20> – 384000 |
| n40 | 100 | 460000 – <20> – 480000 | 460000 – <20> – 480000 |
| n41 | 15 | 499200 – <3> – 537999 | 499200 – <3> – 537999 |
|  | 30 | 499200 – <6> – 537996 | 499200 – <6> – 537996 |
| n462 | 15 | 743334 – <1> – 795000 | 743334 – <1> – 795000 |
| n47 | 15 | 790334 – <1> – 795000 | 790334 – <1> – 795000 |
| n48 | 15 | 636667 – <1> – 646666 | 636667 – <1> – 646666 |
|  | 30 | 636668 – <2> – 646666 | 636668 – <2> – 646666 |
| n50 | 100 | 286400 – <20> – 303400 | 286400 – <20> – 303400 |
| n51 | 100 | 285400 – <20> – 286400 | 285400 – <20> – 286400 |
| n53 | 100 | 496700 – <20> – 499000 | 496700 – <20> – 499000 |
| n54 | 100 | 334000 – <20> – 335000 | 334000 – <20> – 335000 |
| n65 | 100 | 384000 – <20> – 402000 | 422000 – <20> – 440000 |
| n66 | 100 | 342000 – <20> – 356000 | 422000 – <20> – 440000 |
| n67 | 100 | N/A | 147600 – <20> – 151600 |
| n70 | 100 | 339000 – <20> – 342000 | 399000 – <20> – 404000 |
| n71 | 100 | 132600 – <20> – 139600 | 123400 – <20> – 130400 |
| n74 | 100 | 285400 – <20> – 294000 | 295000 – <20> – 303600 |
| n75 | 100 | N/A | 286400 – <20> – 303400 |
| n76 | 100 | N/A | 285400 – <20> – 286400 |
| n77 | 15 | 620000 – <1> – 680000 | 620000 – <1> – 680000 |
|  | 30 | 620000 – <2> – 680000 | 620000 – <2> – 680000 |
| n78 | 15 | 620000 – <1> – 653333 | 620000 – <1> – 653333 |
|  | 30 | 620000 – <2> – 653332 | 620000 – <2> – 653332 |
| n79 | 15 | 693334 – <1> – 733333 | 693334 – <1> – 733333 |
|  | 30 | 693334 – <2> – 733332 | 693334 – <2> – 733332 |
| n80 | 100 | 342000 – <20> – 357000 | N/A |
| n81 | 100 | 176000 – <20> – 183000 | N/A |
| n82 | 100 | 166400 – <20> – 172400 | N/A |
| n83 | 100 | 140600 – <20> –149600 | N/A |
| n84 | 100 | 384000 – <20> – 396000 | N/A |
| n85 | 100 | 139600 – <20> – 143200 | 145600 – <20> – 149200 |
| n86 | 100 | 342000 – <20> – 356000 | N/A |
| n89 | 100 | 164800 – <20> – 169800 | N/A |
| n90 | 15 | 499200 – <3> – 537999 | 499200 – <3> – 537999 |
|  | 30 | 499200 – <6> – 537996 | 499200 – <6> – 537996 |
|  | 100 | 499200 – <20> – 538000 | 499200 – <20> – 538000 |
| n91 | 100 | 166400 – <20> – 172400 | 285400 – <20> – 286400 |
| n92 | 100 | 166400 – <20> – 172400 | 286400 – <20> – 303400 |
| n93 | 100 | 176000 – <20> – 183000 | 285400 – <20> – 286400 |
| n94 | 100 | 176000 – <20> – 183000 | 286400 – <20> – 303400 |
| n95 | 100 | 402000 – <20> – 405000 | N/A |
| n963 | 15 | 795000 – <1> – 875000 | 795000 – <1> – 875000 |
| n97 | 100 | 460000 – <20> – 480000 | N/A |
| n98 | 100 | 376000 – <20> – 384000 | N/A |
| n99 | 100 | 325300 – <20> – 332100 | N/A |
| n100 | 100 | 174880 – <20> – 176000 | 183880 – <20> – 185000 |
| n101 | 100 | 380000 – <20> – 382000 | 380000 – <20> – 382000 |
| n1024 | 15 | 795000 – <1> – 828333 | 795000 – <1> – 828333 |
| n104 | 15 | 828334 – <1> – 875000 | 828334 – <1> – 875000 |
|  | 30 | 828334 – <2> – 875000 | 828334 – <2> – 875000 |
| n105 | 100 | 132600 – <20> – 140600 | 122400 – <20> – 130400 |
| NOTE 1: The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. NOTE 2: The following NREF are allowed for operation in Band n46: see Table 5.4.2.3-2. NOTE 3: The following NREF are allowed for operation in Band n96: see Table 5.4.2.3-3. NOTE 4: The following NREF are allowed for operation in Band n102: see Table 5.4.2.3-4. NOTE 5: The band supports the enhanced channel raster with step size <2>. |  |  |  |