3GPP TSG-RAN WG4 Meeting #108bis	R4-2315588

# Xiamen, China, 9th – 13th October 2023

Source:	Ericsson

Title:	Discussion on FR2 PUSCH 256QAM demodulation requirements

Agenda item:	5.6.4.1

Document for:	Discussion

# Introduction

According to the WF in RAN4#108 [1], following agreements are achieved.

Issue 1-1: PUSCH Requirements

Agreement: The demodulation performance requirements for UL 256QAM will be defined only for PUSCH

Issue 1-2: Bands for FR2 256 QAM

Agreement: Introduce only one FR2-1 PUSCH with 256QAM demodulation requirements up to 39 GHz (aligned with RF).

Define one set of requirements for frequency range of up to 39GHz, carrier frequency of 39GHz shall be used for the requirements definition.

Interested companies who would like to evaluate the performance for carrier frequency 30GHz compared to 39GHz, are invited to submit simulations for RAN4#108-bis but this is not mandatory.

Issue 1-3: BS Declaration

Agreement: Align with FR1: The PUSCH requirement with FR2-1 UL 256QAM is only applied for BS declared to support it. Introduce the BS declaration for FR2-1 UL 256QAM.

Issue 2-1: EVM Impact

Agreement: The impact of EVM can be considered in impairment results, and interested companies are encouraged to provide ideal simulations with and without TxEVM for alignment at RAN4#108bis, TxEVM value to be 3.5%.

Note: Test requirement parameters do not capture TxEVM and TEs don’t add additional TxEVM during test.

Issue 2-2: SNR Limit

RAN4 will attempt to keep simulation parameters to reach an SNR operating point less than 20dB, but will await simulation results from interested companies at RAN4#108-bis.

Request input from TE vendor on maximum testable SNR at RAN4#108bis

Issue 2-3: Phase Noise Model

Interested companies are encouraged to bring simulations to RAN4#108-bis with and without phase noise model, and provide information of phase noise model, with PT-RS enabled and disable.

- Alignment to happen without phase model (therefore with priority)

- Companies encouraged to deliver simulation results for some cases of Phase Noise model, with PT-RS enabled and disabled.

Issue 2-4: Channel

The following candidate channel models can be considered for simulation at RAN4 #108 bis and down selected at later meetings for requirements.

- TDLA 30-35

- TDLD 30-35

Issue 2-5: Rank

Agreement: Rank 1

Issue 2-6: Carrier BW for 60 kHz SCS and Issue 2-7: Carrier BW for 120 kHz SCS

Interested companies are encouraged to bring simulation on the following config: 60 kHz SCS/50 MHz, 120 kHz SCS/50 MHz, 100 and 200 MHz, discussion for further downselection based on simulation results at for RAN4#108-bis.

Issue 2-8: Additional DMRS

Interested companies to provide simulation results with single (1+0) and additional (1+1) DMRS for RAN4#108-bis.

Issue 2-9: DMRS Mapping Type

Agreement: Mapping Type B

Issue 2-10: PTRS

Interested companies to provide simulation results with both enabled and disable for RAN4#108-bis.

Configuration: K_PTRS: 2 and L_PTRS =1

Issue 2.11: Waveform Type

Agreement: CP-OFDM

Issue 2-12: MCS

RAN4 considers the following candidate MCS before down selection for requirements definition at RAN4#108 bis (all in Table 2): MCS 20, 21 and 22 with a priority on MCS 20

Issue 2-13: Testing Metric

Agreement: 70% of Max Throughput

Issue 2-14: Frequency Domain Resource Allocation

Agreement: Full RB allocation of the applicable BW

Issue 2-15: Frequency Hopping

Agreement: Frequency hopping disabled

Issue 2-16: Number of HARQ Transmissions

Agreement: 4

### Sub-topic 2-15: TPMI Index

Agreement: Not applicable, due to Rank 1

Issue 2-18: Code Block Group

Agreement: disabled

Issue 2-19: Limited buffer rate matching

Agreement: disabled

In this contribution, open issues are furtherly analyzed.

# Discussion

Antenna configuration

The antenna configuration in the simulation results summary of previous meeting is 2Tx2Rx. But the agreed rank is only 1 for UL 256QAM. Although 2Tx could possibly have diversity gain compared to 1Tx, but all legacy requirements defined for 2Tx are rank 2 scenario. The reason behind is only consider the worst case from BS side and 1Tx transmission is also typical for UE. In that case, we propose only consider 1Tx2Rx rather than 2Tx2Rx for FR2 PUSCH 256QAM demodulation.

Proposal 1 	Only consider 1Tx2Rx as antenna configuration for FR2 PUSCH 256QAM demodulation requirement.

MCS

According to our simulation results, the ideal results for MCS20 is close to 20dB SNR limit. Regarding possible large margin will be added on top of ideal results, we propose only consider MCS20 for the requirements.

Proposal 2 	Only consider MCS20 for FR2 PUSCH 256QAM demodulation requirement.

EVM impact

According to our simulation results [2], the EVM impact is small for MCS20 no matter if PN is considered. In that case, no EVM impact results could be used for alignment.

Proposal 3 	Companies use no Tx EVM results for alignment.

PN impact

Based on our simulation with PN model Example-1 in TS38.801, the degradation with PN is not so obvious (~1dB). The Example-2 would cause even larger degradation than Example-1 in most of cases based on previous experience.

Channel model

TDLD channel could have average 2dB more margin to 20dB SNR limit than TDLA channel. In that case, all impairment degradation (including EVM, PN etc.) could be within the SNR limit without extra modification on AWGN level etc.

Proposal 5 	Consider TDLD30-35 as the channel model for FR2 PUSCH 256QAM demodulation requirement.

CBW per SCS

There is no much performance difference between different CBW per SCS, in that case, only the minimum CBW could be used for the requirements to save the effort.

Proposal 6 	Take 60kHz SCS/50MHz and 120kHz SCS/50MHz for FR2 PUSCH 256QAM demodulation requirement.

Additional DM-RS

The DM-RS configuration depends on manufacture declaration which allow BS support preferred configuration. Basically, both configurations could be defined if feasibility is OK. But DM-RS 1+1 configuration is more typical one than 1+0 configuration. If down selection is necessary, DM-RS 1+1 might be more suitable for the requirement.

Proposal 7 	Prioritize DM-RS 1+1 for FR2 PUSCH 256QAM demodulation requirement.

# Conclusions

In the previous sections we made the following observations:

No table of figures entries found.

Based on the discussion in the previous sections we propose the following:

Proposal 1 	Only consider 1Tx2Rx as antenna configuration for FR2 PUSCH 256QAM demodulation requirement.

Proposal 2 	Only consider MCS20 for FR2 PUSCH 256QAM demodulation requirement.

Proposal 3 	Companies use no Tx EVM results for alignment.

Proposal 5 	Consider TDLD30-35 as the channel model for FR2 PUSCH 256QAM demodulation requirement.

Proposal 6 	Take 60kHz SCS/50MHz and 120kHz SCS/50MHz for FR2 PUSCH 256QAM demodulation requirement.

Proposal 7 	Prioritize DM-RS 1+1 for FR2 PUSCH 256QAM demodulation requirement.

# References

[1]	R4-2313946, Way Forward for 256 QAM BS Demod, Nokia, Nokia Shanghai Bell

[2]	R4-2315589, Simulation results for FR2 UL 256QAM, Ericsson.