3GPP TSG-RAN WG4 Meeting #108bis	R4-2315589

# Xiamen, China, 9th – 13th October 2023

Source:	Ericsson

Title:	Simulation results for FR2 PUSCH 256QAM demodulation requirements

Agenda item:	5.6.4.1

Document for:	Information

# Introduction

According to the WF in RAN4#108 [1], following simulation assumptions are agreed.

Table 1-1 Simulation parameters

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern (Note 1) |  | 60 kHz and 120kHz SCS: 3D1S1U, S=10D:2G:2U |
| HARQ | Maximum number of HARQ transmissions | 4 |
| RV sequence |  | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
| DM-RS duration |  | single-symbol DM-RS |
| Additional DM-RS symbols |  | pos0, pos1 |
| Number of DM-RS CDM group(s) without data |  | 2 |
| Ratio of PUSCH EPRE to DM-RS EPRE |  | -3 dB |
| DM-RS port(s) |  | {0} |
| DM-RS sequence generation |  | NID=0, nSCID =0 |
| Time domain | PUSCH mapping type | B |
| resource | Start symbol index | 0 |
| Allocation length |  | 10 |
| Frequency domain | RB assignment | Full applicable test bandwidth |
| resource | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| PT-RS configuration | Frequency density (KPT-RS) | 2, Disabled |
|  | Time density (LPT-RS) | 1, Disabled |
| Tx EVM |  | 3.5% |
| Rx phase noise model |  | Example -1 in TR38.803 |
| NOTE 1: The same requirements are applicable to TDD with different UL-DL patterns |  |  |

In this contribution, simulation results are provided.

# Simulation results

Table 2-1 Simulation results for FR2 PUSCH 256QAM MCS20

| PT-RS configuration | SCS and CBW | Channel model | DM-RS configuration | SNR (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | No Tx EVM No PN | Only Tx EVM | Only Rx PN | Tx EVM and Rx PN |
| PT-RS Enabled | 60kHz, 50MHz | TDLA30-35 | DMRS 1+1 | 19.07 | 19.45 | 20.01 | 20.5 |
|  |  | TDLA30-35 | DMRS 1+0 | 19.19 | 19.59 | 20.19 | 20.7 |
|  |  | TDLD30-35 | DMRS 1+1 | 17.09 | 17.49 | 18.06 | 18.57 |
|  |  | TDLD30-35 | DMRS 1+0 | 17.33 | 17.69 | 18.33 | 18.77 |
|  | 120kHz, 50MHz | TDLA30-35 | DMRS 1+1 | 18.84 | 19.23 | 19.76 | 20.24 |
|  |  | TDLA30-35 | DMRS 1+0 | 19.15 | 19.57 | 20.25 | 20.78 |
|  |  | TDLD30-35 | DMRS 1+1 | 17.12 | 17.52 | 18.02 | 18.55 |
|  |  | TDLD30-35 | DMRS 1+0 | 17.55 | 17.88 | 18.65 | 19.14 |
|  | 120kHz, 100MHz | TDLA30-35 | DMRS 1+1 | 19.26 | 19.65 | 20.27 | 20.77 |
|  |  | TDLA30-35 | DMRS 1+0 | 19.26 | 19.67 | 20.36 | 20.87 |
|  |  | TDLD30-35 | DMRS 1+1 | 17.22 | 17.6 | 18.25 | 18.71 |
|  |  | TDLD30-35 | DMRS 1+0 | 17.48 | 17.82 | 18.58 | 19 |
|  | 120kHz, 200MHz | TDLA30-35 | DMRS 1+1 | 18.94 | 19.36 | 20.00 | 20.54 |
|  |  | TDLA30-35 | DMRS 1+0 | 18.94 | 19.39 | 20.13 | 20.68 |
|  |  | TDLD30-35 | DMRS 1+1 | 17.21 | 17.6 | 18.3 | 18.75 |
|  |  | TDLD30-35 | DMRS 1+0 | 17.49 | 17.82 | 18.63 | 19.09 |
| PT-RS disabled | 60kHz, 50MHz | TDLA30-35 | DMRS 1+1 | 18.37 | 18.69 | 19.29 | 19.69 |
|  |  | TDLA30-35 | DMRS 1+0 | 18.48 | 18.81 | 22.82 | 23.61 |
|  |  | TDLD30-35 | DMRS 1+1 | 16.45 | 16.73 | 17.4 | 17.75 |
|  |  | TDLD30-35 | DMRS 1+0 | 16.68 | 16.95 | 20.29 | 21.01 |
|  | 120kHz, 50MHz | TDLA30-35 | DMRS 1+1 | 18.14 | 18.48 | 18.98 | 19.39 |
|  |  | TDLA30-35 | DMRS 1+0 | 18.47 | 18.82 | 20.73 | 21.28 |
|  |  | TDLD30-35 | DMRS 1+1 | 16.5 | 16.77 | 17.33 | 17.7 |
|  |  | TDLD30-35 | DMRS 1+0 | 16.84 | 17.19 | 18.7 | 19.17 |
|  | 120kHz, 100MHz | TDLA30-35 | DMRS 1+1 | 18.52 | 18.86 | 19.5 | 19.91 |
|  |  | TDLA30-35 | DMRS 1+0 | 18.5 | 18.84 | 20.65 | 21.21 |
|  |  | TDLD30-35 | DMRS 1+1 | 16.58 | 16.84 | 17.54 | 17.87 |
|  |  | TDLD30-35 | DMRS 1+0 | 16.78 | 17.09 | 18.72 | 19.18 |
|  | 120kHz, 200MHz | TDLA30-35 | DMRS 1+1 | 18.26 | 18.61 | 19.31 | 19.75 |
|  |  | TDLA30-35 | DMRS 1+0 | 18.23 | 18.59 | 20.86 | 21.48 |
|  |  | TDLD30-35 | DMRS 1+1 | 16.58 | 16.84 | 17.64 | 17.97 |
|  |  | TDLD30-35 | DMRS 1+0 | 16.79 | 17.08 | 19.03 | 19.63 |

Tx EVM has 0.3~0.5dB degradation which is small.

Target SNR of MCS20 with TDLA30-35 would be larger than 20dB limit in most of the cases.

The performance difference is small between different CBW per SCS.

PT-RS enabled could have benefit on PN migration for DM-RS 1+0 cases.

DM-RS 1+1 have 1dB better performance for PT-RS disabled cases.

# Conclusions

In the previous sections we made the following observations:

Observation 1	Tx EVM has 0.3~0.5dB degradation which is small.

Observation 2	Target SNR of MCS20 with TDLA30-35 would be larger than 20dB limit in most of the cases.

Observation 3	The performance difference is small between different CBW per SCS.

Observation 4	PT-RS enabled could have benefit on PN migration for DM-RS 1+0 cases.

Observation 5	DM-RS 1+1 have 1dB better performance for PT-RS disabled cases.

Based on the discussion in the previous sections we propose the following:

No table of figures entries found.

# References

[1]	R4-2313946, Way Forward for 256 QAM BS Demod, Nokia, Nokia Shanghai Bell