3GPP TSG-RAN WG4 Meeting # 108bis	R4-2315662

Xiamen, China, October 09 – October 13, 2023

Title: 	Discussion on performance requirements for mobility enhancements

Source: 	Huawei, HiSilicon

Agenda item:	5.24.3

Document for:	Discussion

Introduction

This contribution provides analysis on performance requirements for mobility enhancement.

Discussion

LTM

- Accuracy

The R17 existing accuracy for L1-RSRP measurements are defined for the intra-frequency L1-RSRP measurement. Herein for FR1 the relative L1-RSRP measurement accuracy is less than absolute measurement accuracy as the UE implementation margin can be eliminated when two cells on the same frequency are compared. If inter-frequency L1-RSRP measurement is supported, the inter-frequency measurement accuracy may need to be analyzed. In theory for FR1 inter-frequency measurement the relative accuracy is supposed be the same as absolute accuracy because the UE RF margin is different on different frequency.

In R18 LTM, the L1-RSRP based event trigger is not supported. When network indicates L1-RSRP resource to report, UE performs corresponding L1-RSRP measurement. For the beam selection for SSB based L1-RSRP measurement report, the following is agreed.

- For the value of M, L - the RRC configured candidate values are: - M = 1, 2, 3, 4 - L = [1], 2, 3, 4 - Note: the maximum value of M*L and combination of M and L is up to UE capability

The selection of cells for the L1 measurement report is up to UE implementation. From this aspect perspective, the use case of using relative accuracy requirements for inter-frequency L1-RSRP measurement is not clear.

Therefore whether to specify new relative accuracy requirements for FR1 inter-frequency L1-RSRP measurements need further consideration.

Proposal 1: Whether to specify new relative accuracy requirements for FR1 inter-frequency L1-RSRP measurements needs further consideration.

The core requirements of LTM are under discussion. As per current progress, the following test cases can be specified:

- Test cases for cell switching delay

- Test cases for intra-frequency L1-RSRP measurement with UE capability within CP

- Test cases for inter-frequency L1-RSRP measurement

Other tests are not precluded based on further conclusion of core requirements.

Proposal 2: For LTM the following test cases can be specified:

- Test case for cell switching delay

- Test case for intra-frequency L1-RSRP measurement with UE capability within CP

- Test case for inter-frequency L1-RSRP measurement

Other tests are not precluded based on further conclusion of core requirements.

NR-DC with selective activation of cell groups via L3 enhancements

The core requirements are settled down. The test case for Subsequent conditional PSCell change is to be specified.

Proposal 3: The test case for Subsequent conditional PSCell change is to be specified.

Improvement on SCell/SCG setup delay

As so far there is no concrete solution for this objective, the test cases related discussion would wait for more progress.

Proposal 4: The performance part of improvement on SCell/SCG setup delay wait for more progress on core part.

Enhanced CHO configurations

The core requirements for CHO including target MCG and target SCG (obj#3) and CHO including target MCG and candidate SCG for CPC/CPA in NR-DC (obj#4). The following test cases can be defined:

-Conditional handover including target MCG in FR1 and target SCG in FR1 in NR-DC

-Conditional handover including target MCG in FR1 and target SCG in FR2 in NR-DC

-Conditional handover including target MCG in FR1 and candidate SCG for CPC/CPA in FR1 in NR-DC

-Conditional handover including target MCG in FR1 and candidate SCG for CPC/CPA in FR2 in NR-DC

Although the second and the fourth tests have testability issues, they can be specified in test case with a clarification that UE does not have to pass the test case.

Proposal 5: The following test cases can be defined:

-Conditional handover including target MCG in FR1 and target SCG in FR1 in NR-DC

-Conditional handover including target MCG in FR1 and target SCG in FR2 in NR-DC

-Conditional handover including target MCG in FR1 and candidate SCG for CPC/CPA in FR1 in NR-DC

-Conditional handover including target MCG in FR1 and candidate SCG for CPC/CPA in FR2 in NR-DC

Conclusions

This contribution provides analysis on performance requirements for mobility enhancement. The following proposals are provided:

Proposal 1: Whether to specify new relative accuracy requirements for FR1 inter-frequency L1-RSRP measurements needs further consideration.

Proposal 2: For LTM the following test cases can be specified:

- Test case for cell switching delay

- Test case for intra-frequency L1-RSRP measurement with UE capability within CP

- Test case for inter-frequency L1-RSRP measurement

Other tests are not precluded based on further conclusion of core requirements.

Proposal 3: The test case for Subsequent conditional PSCell change is to be specified.

Proposal 4: The performance part of improvement on SCell/SCG setup delay wait for more progress on core part.

Proposal 5: The following test cases can be defined:

-Conditional handover including target MCG in FR1 and target SCG in FR1 in NR-DC

-Conditional handover including target MCG in FR1 and target SCG in FR2 in NR-DC

-Conditional handover including target MCG in FR1 and candidate SCG for CPC/CPA in FR1 in NR-DC

-Conditional handover including target MCG in FR1 and candidate SCG for CPC/CPA in FR2 in NR-DC

Reference