3GPP TSG-RAN WG4 Meeting # 108bis										R4-2315678

Xiamen, China, October 09 – October 13, 2023

Agenda item:	6.4.3

Source: 	ZTE Corporation

Title: 	Discussion on UE RF requirements for FDD band (L+S band) for IoT NTN operation

Document for:	Approval

Introduction

In RAN#99 meeting, the work item [1] on FDD band (L+S band) for IoT NTN operation was approved. Part of the objective is as follows:

- Specify the following new FDD frequency bands for NB-IoT/eMTC NTN operation:

  - 1610 – 1626.5 MHz in UL, and 2483.5 – 2500 MHz in DL

- Support UE power class 3 (+23dBm) and power class 5 (+20dBm)

- In this contribution, we want to share some initial views on UE RF requirements for FDD band (L+S band) for IoT NTN operation.

Discussion

In the last meeting, an agreement was achieved for out-of-band blocking requirement as follows:

Issue 2-2-1: For band B254 category M1, should the general requirements for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz, i.e., also for band B255, be re-used as Out-of-Band Blocking (OOBB)? - Proposals - Option 1: Yes - Option 2: No, please propose a new value. - Recommended WF - Option 1? Agreement: Agree on Option 1.

However, the above agreement does not involve the details whether relaxation is needed for the requirement of range 3 out-of-band blocking requirement. Based on the analysis of our contribution [3], the note of “The power level of the interferer (Pinterferer) for Range 3 shall be modified to -20 dBm for Finterferer > 2585 MHz and FInterferer < 2775 MHz.” also should be introduced for band 254.

Proposal 1: For the new FDD frequency bands for NB-IoT/eMTC NTN operation, the note of “The power level of the interferer (Pinterferer) for Range 3 shall be modified to -20 dBm for Finterferer > 2585 MHz and FInterferer < 2775 MHz.” should be introduced for band 254.

Similar analysis [2] also applies to the NS value for band 254 and two NS values should be introduced to support ETSI regulations for the new FDD frequency bands as shown in Table 1.

Table 1: NS values for ETSI regulation

| NS value | Regulatory domain | Applicable range (MHz) | In-band emissions | Out-of-band emissions |
| --- | --- | --- | --- | --- |
| NS_04N | ETSI | 1610-1618.25 | ETSI (1610-1618.25) | ETSI (1559-1610MHz) ETSI (>1628.5MHz) |
| NS_05N |  | 1618.25-1626.5 | ETSI (1618.25-1626.5) |  |

Proposal 2: Two NS values should be introduced to support ETSI regulations for the new FDD frequency bands as shown in Table 1.

Conclusions

In this contribution, we want to share some further views on UE RF requirements for Introduction of the satellite L-/S-band. Some proposals are made as following:

Proposal 1: For the new FDD frequency bands for NB-IoT/eMTC NTN operation, the note of “The power level of the interferer (Pinterferer) for Range 3 shall be modified to -20 dBm for Finterferer > 2585 MHz and FInterferer < 2775 MHz.” should be introduced for band 254.

Proposal 2: Two NS values should be introduced to support ETSI regulations for the new FDD frequency bands as shown in Table 1.

References

- RP-230792, “New WID: FDD band (L+S band) for IoT NTN operation”, MediaTek Inc. Apple, Globalstar, Kepler Communications, Qualcomm, ZTE

- R4-2315670, “Discussion on UE RF requirements for the satellite L-band and S-band”, ZTE