3GPP TSG-RAN WG4 Meeting #108bis	R4-2315695

Xiamen, China, October 09 – October 13, 2023

Agenda Item:	5.34.4

Source:	Ericsson

Title:	Discussion on measurement without gaps test cases

Document for:	Discussion

# Introduction

In last meeting, RAN4 discussed the core requirements of SSB-less SCell activation in NES. Although there are still some open issues for SSB-less SCell activation, RAN4 can still start the analysis for the general test case design. In this contribution, we’ll discuss the general test case and test case list.

# SSB-less SCell activation test case

In last meeting, RAN4 spent much time on scenario and RTD condition discussion and achieve some agreements. In our understanding, RAN4 can firstly focus on the test case design based on current agreements. Firstly, RAN4 can agree to define test case for scenario 1 with RTD equal or less than CP.

In legacy SCell activation requirement, generally, RAN4 will differentiate the test cases with known and unknown conditions. The main different between known and unknown cell is whether L3 measurement is performed within 3ms before SCell activation command. However, known condition seems unnecessary for SSB-less SCell activation since UE will refer on the coarse timing from the reference cell. The AGC retuning samples due to power difference is uncertain. Thus, we suggest to wait core requirement to further consider whether to introduce different test cases.

Based on above discussions, we suggest to define the following TC for SSB-less SCell activation as a start point.

Proposal 1: RAN4 to define the following test cases for SSB-less SCell activation with RTD<=CP

- TC1: SSB-less SCell Activation and deactivation of known SCell in FR1 in non-DRX

# Other NES procedures

Furthermore, other NES procedures, such as Cell DTX/DRX and CHO enhancement discussion are on-going in other WGs. RAN4 can check whether new test cases for Cell DTX/DRX and CHO enhancement are needed until RAN4 achieves the agreements to identify the impact in RAN4 spec. Thus, we suggest to wait core requirements’ discussion to further consider whether to introduce new test cases for Cell DTX/DRX and CHO enhancement.

Proposal 2: RAN4 to wait core requirement progress on Cell DTX/DRX and CHO enhancement test case design.

# Conclusion

In the contribution, we discuss the test case design for NES. We have the following proposals:

Proposal 1: RAN4 to define the following test cases for SSB-less SCell activation with RTD<=CP

- TC1: SSB-less SCell Activation and deactivation of known SCell in FR1 in non-DRX

Proposal 2: RAN4 to wait core requirement progress on Cell DTX/DRX and CHO enhancement test case design.

# Reference

[1] R4-2314381, “WF on RRM requirements for NR network energy saving”, Huawei

[2] RP-223540, “New WID: Network energy savings for NR”, Huawei