# 3GPP TSG-RAN WG4 Meeting # 108bis                            R4-2315700

# Xiamen, China, October 09 – October 13, 2023

# Source:        ZTE Corporation

# Title:           Discussion on demodulation for FR2-1 UL 256QAM

# Agenda Item:   5.6.4.1

# Document for:  Approval

- Introduction

In RAN 97 meeting, a revised WID [1] for NR RF requirements enhancement for frequency range 2 (FR2), Phase 3 was approved, the objective for demodulation is:

UL 256QAM

  - Specify the BS demodulation performance

In this contribution, we give some discussions on simulation assumption and initial simulation results for reference.

- Discussion

## 2.1 Simulation assumption for FR2-1 UL 256QAM

The following issues were identified in the previous meeting and need further discussion[2].

Sub-topic 2-2: SNR Limit Issue 2-2: SNR Limit RAN4 will attempt to keep simulation parameters to reach an SNR operating point less than 20dB, but will await simulation results from interested companies at RAN4#108-bis. Request input from TE vendor on maximum testable SNR at RAN4#108bis Sub-topic 2-3: Phase Noise Model Issue 2-3: Phase Noise Model Interested companies are encouraged to bring simulations to RAN4#108-bis with and without phase noise model, and provide information of phase noise model, with PT-RS enabled and disable. - Alignment to happen without phase model (therefore with priority) - Companies encouraged to deliver simulation results for some cases of Phase Noise model, with PT-RS enabled and disabled. Sub-topic 2-4: Channel Issue 2-4: Channel The following candidate channel models can be considered for simulation at RAN4 #108 bis and down selected at later meetings for requirements. - TDLA 30-35 - TDLD 30-35 Sub-topic 2-6: Carrier BW and SCS Issue 2-6: Carrier BW for 60 kHz SCS and Issue 2-7: Carrier BW for 120 kHz SCS Interested companies are encouraged to bring simulation on the following config: 60 kHz SCS/50 MHz, 120 kHz SCS/50 MHz, 100 and 200 MHz, discussion for further downselection based on simulation results at for RAN4#108-bis. Sub-topic 2-7: DMRS Issue 2-8: Additional DMRS Interested companies to provide simulation results with single (1+0) and additional (1+1) DMRS for RAN4#108-bis. Sub-topic 2-10: MCS Issue 2-12: MCS RAN4 considers the following candidate MCS before down selection for requirements definition at RAN4#108 bis (all in Table 2): MCS 20, 21 and 22 with a priority on MCS 20 Sub-topic 2-15: TPMI Index Agreement: Not applicable, due to Rank 1

It can be found that the antenna configuration is not reflected in the above agreements. And a related issue agreed above is the TPMI index which says that TPMI Index is not applicable due to Rank 1. The number of Tx is recommended to be 1.

Proposal 1: To consider the antenna configuration with 1Tx and 2Rx and FFS other number of Rx.

With regard to the phase noise for 39GHz, the following conclusion was reached in RAN4 #107 meeting[3].

Issue 1-1-2: Phase noise profiles evaluation for 39GHz - Proposals - Option 1: The new phase noise profile from MTK for 39GHz is feasible for MPR simulation. (Xiaomi, ZTE) - New phase noise profile using the pole-zero method based on following function: Sfo=PSD0n=1N1+(fofz,n)αz,nm=1M1+(fofp,m)αp,m - Parameters from MTK - Option 2：The new phase noise profiles from MTK and adopting min(example1-based, example2) for 39GHz are feasible for MPR simulation, where ‘example1-based’ refers to the example phase noise profiles from vivo and Anritsu, ‘example2’ refers to the example phase noise profile in TR38.803. (Anritsu) - Option 3: Further discuss feasible phase noise profile for 39GHz. (Vivo, LGE) Agreement: - Take Option 1 as the starting point.

It was agreed in RAN4 #108 meeting that carrier frequency of 39GHz shall be used for the requirements definition so the phase noise of 39GHz needs to be confirmed.

Proposal 2: To agree on option 1 as phase noise model of 39GHz for demodulation requirements.

## 2.2 Simulation results

## In RAN4 # 108 meeting Type B mapping is agreed for FR2-1 UL 256QAM demodulation requirement. TDLA and TDLD are the alternatives as channel model for demodulation requirement. The bandwidth and MCS need to be down selected. Table 2-1 shows the operating SNR of MCS20 and TDLA channel model.

## The simulation assumptions are listed in table A-1 and the throughput curves are shown in table A-2 to A-3.

Table 2-1 Simulation results of PUSCH demodulation for FR2-1 UL 256QAM

| Channel | SCS [kHz] /CBW [MHz] | DMRS Config | PTRS | SNR(dB) |
| --- | --- | --- | --- | --- |
| TDLA30-35Low | 60/50 | 1+1 | disabled | 17.6 |
|  |  |  | enabled | 18.2 |
|  |  | 1+0 | disabled | 17.8 |
|  |  |  | enabled | 19.1 |
|  | 120/50 | 1+1 | disabled | 16.4 |
|  |  |  | enabled | 17.0 |
|  |  | 1+0 | disabled | 16.9 |
|  |  |  | enabled | 17.9 |
| TDLD30-35Low | 60/50 | 1+1 | disabled | 17.0 |
|  |  |  | enabled | 17.1 |
|  |  | 1+0 | disabled | 17.3 |
|  |  |  | enabled | 17.7 |

From table 2-1 we can see that the operating SNR @MCS20 are under 20dB and the operating SNR for TDLD can be lower than that of TDLA about 1dB. It can be seen that CPE with PTRS does not provide a positive gain for operating SNR. As pointed out last RAN4 meeting, TS 38.104 defines UL 256qam requirement for FR1 with a code rate of 682.5/1024 which corresponds to MCS20 in Table 5.1.3.1-2 in TS 38.214[4]. From table 2-1, we can find that the performance gap between (1+0)DMRS and (1+1) DMRS is small.

Observation 1: CPE with PTRS does not provide positive gain for operating SNR.

Observation 2: The performance gap between 1+0 DMRS and 1+1 DMRS is less than 0.5dB.

Proposal 3: To consider MCS20 for FR2-1 UL 256QAM demodulation requirements.

Proposal 4: To select one DMRS configuration between 1+0 and 1+1 DMRS configuration.

- Conclusion

Based on the simulation discussion, the following proposal is given:

Proposal 1: To consider the antenna configuration with 1Tx and 2Rx and FFS other number of Rx.

Proposal 2: To agree on option 1 as phase noise model of 39GHz for demodulation requirements.

Observation 1: CPE with PTRS does not provide positive gain for operating SNR.

Observation 2: The performance gap between 1+0 DMRS and 1+1 DMRS is less than 0.5dB.

Proposal 3: To consider MCS20 for FR2-1 UL 256QAM demodulation requirements.

Proposal 4: To select one DMRS configuration between 1+0 and 1+1 DMRS configuration.

- Reference

RP-222228 Revised WID: NR RF requirements enhancement for frequency range 2 (FR2), Phase 3, Nokia, Xiaomi

R4-2313946 WF for NR_RF_FR2_req_Ph3_Demod, Nokia, Nokia Shanghai Bell

R4-2310260 WF on UL 256QAM requirements, Xiaomi

3GPP TS 38.214 V17.6.0

- Annex

Table A-1 Simulation assumption for PUSCH demodulation for 256QAM

| Parameter | FR2-1 |
| --- | --- |
| Transform precoding | Disabled |
| Number of Tx | 1 |
| Number of Rx | 2 |
| Number of layers | 1 |
| DMRS type | type 1 |
| Number of DMRS symbols | 1, 1+1 |
| symbols length | 10 |
| start symbol index | 0 |
| Time domain resource allocation type | type B |
| Frequency domain resource | full RB allocation of the applicable BW |
| MCS index | 20 in table 5.1.3.1-2 in TS 38.214[3] (R=682.5/1024) |
| Carrier frequency (GHz) | 30 |
| Propagation condition | TDL-A/D 30ns delay spread, 35Hz Doppler frequency |
| SCS and BW | 60kHz: 50MHz 120kHz: 50, 100, 200MHz |
| PTRS | Disabled,KPT-RS =2, LPT-RS =1 |
| Timing offset | 0 |
| Frequency offset | 0 |
| Code block group | Disabled |
| Frequency hopping | Disabled |
| Number of HARQ transmissions and RV sequence | 4 , {0,2,3,1} |
| PN model for Tx and Rx | Tx：New phase noise as agreed for MPR simulation Rx: none |
| Testing metric | SNR @70% of maximum throughput |

Table A-2 BS UL 256QAM PUSCH demodulation for FR2-1@39GHz TDLA MCS20 type B mapping

Table A-3 BS UL 256QAM PUSCH demodulation for FR2-1@39GHz TDLD MCS20 type B mapping