# 3GPP TSG-RAN WG4 Meeting #108bis                              R4-2315706

# Xiamen, China, 09 – 13 October 2023

# Title: 	Discussion on 8Rx demodulation requirements

# Source: 	ZTE Corporation

# Agenda item:	5.3.3.1.2

# Document for:	Approval

# 1 Introduction

- In RAN4#108 meeting, 8Rx UE demodulation and CSI requirements were discussed and a WF was agreed[1]. In this paper, we will provide the detailed discussions on 8 Rx PDSCH demodulation requirements for CA.

# 2 Discussion

Issue 4-3-1: CBW for PDSCH CA requirements - Option 1: Follow the existing CA requirements defined for 2Rx and 4Rx, i.e. - FDD: 15kHz SCS: 5/10/15/20/25/30/35/40/45/50 MHz - TDD: 30kHz SCS: 5/10/15/20/25/30/40/50/60/80/90/100 MHz - Option 2: Single bandwidth for each CC: 15kHz/10MHz for FDD and 30kHz/40MHz for TDD separately

- Regarding CBW for CA requirements, in RF session which had a following agreement [2] in RAN4#108 meeting.

Agreement: - RAN4 does not define example band combinations for 8Rx CA/DC - 8Rx requirements should apply to the band(s) implemented with 8Rx in all existing band combinations including intra/inter band CA and DC (including EN-DC)

- From above agreement, RAN4 does not define example band combinations for 8RX CA/DC and requirements should apply to all existing band combinations. Hence, for demodulation part, 8Rx requirements could be apply to all existing band combinations for CA.

- Proposal 1. Follow the existing CA requirements defined for 2Rx and 4Rx, i.e.

- FDD: 15kHz SCS: 5/10/15/20/25/30/35/40/45/50 MHz

- TDD: 30kHz SCS: 5/10/15/20/25/30/40/50/60/80/90/100 MHz

Issue 4-3-2: Duplex model - Option 1: Consider the following duplex model for 8Rx CA - FDD 15kHz + TDD 30kHz - FDD 15kHz + FDD 15kHz - TDD 30kHz + TDD 30kHz - Other options not precluded

- Regarding duplex model, 8Rx needs to cover TDD bands(n41, n77/ n78, n79) and FDD band(n7). Hence, the following combinations could be considered for 8Rx CA.

- FDD 15kHz + TDD 30kHz

- FDD 15kHz + FDD 15kHz

- TDD 30kHz + TDD 30kHz

Proposal 2. Consider the following duplex model for 8Rx CA:

- FDD 15kHz + TDD 30kHz

- FDD 15kHz + FDD 15kHz

- TDD 30kHz + TDD 30kHz

Issue 4-3-3: Test scenario One single test scenario to be defined, with same test configurations as single carrier case except the bandwidth: - Option 1: Rank8 case - Option 2: Rank2 case

- For test scenario, we prefer to consider rank 8 case.

- Proposal 3. Consider rank 8 case for 8Rx CA.

# 3 Conclusion

In this contribution, we give some discussions on demodulation performance requirements for 8Rx , The conclusions are:

- Proposal 1. Follow the existing CA requirements defined for 2Rx and 4Rx, i.e.

- FDD: 15kHz SCS: 5/10/15/20/25/30/35/40/45/50 MHz

- TDD: 30kHz SCS: 5/10/15/20/25/30/40/50/60/80/90/100 MHz

Proposal 2. Consider the following duplex model for 8Rx CA:

        - FDD 15kHz + TDD 30kHz

        - FDD 15kHz + FDD 15kHz

        - TDD 30kHz + TDD 30kHz

- Proposal 3. Consider rank 8 case for 8Rx CA.

# 4 References

- R4-2313877, WF for 8Rx UE performance requirements. Huawei, HiSilicon.

- R4-2314695, WF on support of 8Rx UE RF. NTT DOCOMO, INC.