# 3GPP TSG-RAN WG4 Meeting # 108bis                                R4-2315709

# Xiamen, China, Oct 9 – Oct 13, 2023

# Title: 	Discussion on BS demodulation performance requirements for less than 5MHz

# Source: 	ZTE Corporation

# Agenda item:	5.14.6.2

# Document for:	Approval

# 1 Introduction

In RAN#95e meeting, a revised WID on NR support for dedicated spectrum less than 5MHz for FR1 was approved [1]. It focuses on market request from vertical industry with the operator bandwidth less than 5MHz, such as Future Railway Mobile Communication System (FRMCS) in Europe, Smart Grids in USA and Public Safety in Europe. In those above deployment scenarios, the possible available bandwidth for NR operation is 2.8~3.6 MHz for FRMCS or 3MHz for smart grids and Public Safety.

The following objectives shall be included for dedicated FDD spectrum in FR1: - Specify necessary RAN4 requirements to support deploying NR in spectrum allocations from approximately 3 MHz up to below 5 MHz [RAN4], including in bands n100, n106, n26, n28 and n85: - Specify system parameters (including channel and sync rasters) for the associated dedicated spectrum. - Minimize impact on RF requirements: - Reuse 5 MHz channel bandwidth at least for FRMCS use case (assuming co-located NR and GSM-R with same operator). - Specify the required RF requirements for optional 3 MHz channel bandwidth in bands n100, n106, n26, n28 and n85. - Specify RRM requirements while minimizing specification impact to support operation in dedicated spectrum allocations from approximately 3 MHz up to below 5 MHz. Specify necessary UE/BS performance requirements for NR operation in dedicated FDD FR1 spectrum allocations from approximately 3MHz up to below 5MHz, corresponding to the core requirements: - Specify necessary RRM performance requirements (RAN4) - Specify necessary UE demodulation performance and CSI reporting requirements (RAN4) - Specify necessary BS demodulation performance requirements (RAN4) - Specify necessary BS conformance tests (RAN4)

In this document, we focus on the RAN4 demodulation aspects and discuss the potential enhancement for the dedicated spectrum less than 5MHz.

# 2 Discussion

## 2.1 PUSCH requirements

Considering maximum transmission bandwidth has defined the number of NRB for 3MHz in existing .

The maximum transmission bandwidth configuration NRB for each UE channel bandwidth and subcarrier spacing is specified in following Table 5.3.2-1 [2].

Table 5.3.2-1: Maximum transmission bandwidth configuration NRB

| SCS (kHz) | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB |
| 15 | 15 | 25 | 52 | 79 | 106 | 133 | 160 | 188 | 216 | 242 | 270 | N/A | N/A | N/A | N/A | N/A |
| 30 | N/A | 11 | 24 | 38 | 51 | 65 | 78 | 92 | 106 | 119 | 133 | 162 | 189 | 217 | 245 | 273 |
| 60 | N/A | N/A | 11 | 18 | 24 | 31 | 38 | 44 | 51 | 58 | 65 | 79 | 93 | 107 | 121 | 135 |

Hence, to make sure the integrity of RAN4 demodulation performance. We can consider defining 3MHz requirements for PUSCH.

However, we notice that the current release has defined 5MHz demodulation requirements for PUSCH in TS 38.101-4. And in our understanding, there is no significant performance difference between 5Mhz and 3MHz.

Proposal 1. From RAN4 demodulation perspective, RAN4 could consider defining 3MHz requirements for PUSCH in 15kHz .

Regarding antenna configuration, considering 1Tx and 2Tx two layer for PUSCH requirements.

Proposal 2. Considering 1Tx and 2Tx two layer for PUSCH requirements.

## 2.2 PUCCH requirements

The physical uplink control channel can support multiple formats as shown in following Table [3], which the frequency domain resources as described in Table 2 [4].

According to Table 2, we can observed that format 0, format 1 and format 4 only support 1PRB. And for format 2 and format 3 can support 16PRBs at most. The number of PRBs actually scheduled depends on the RRC signalling configuration.

Table 6.3.2.1-1: PUCCH formats.

| PUCCH format | Length in OFDM symbols | Number of bits |
| --- | --- | --- |
| 0 | 1 – 2 | ≤2 |
| 1 | 4 – 14 | ≤2 |
| 2 | 1 – 2 | >2 |
| 3 | 4 – 14 | >2 |
| 4 | 4 – 14 | >2 |

Table 2: Frequency domain resources of PUCCH formats

PUCCH-format0 ::=                               SEQUENCE {

initialCyclicShift                              INTEGER(0..11),

nrofSymbols                                     INTEGER (1..2),

startingSymbolIndex                             INTEGER(0..13)

}

PUCCH-format1 ::=                               SEQUENCE {

initialCyclicShift                              INTEGER(0..11),

nrofSymbols                                     INTEGER (4..14),

startingSymbolIndex                             INTEGER(0..10),

timeDomainOCC                                   INTEGER(0..6)

}

PUCCH-format2 ::=                               SEQUENCE {

nrofPRBs                                        INTEGER (1..16),

nrofSymbols                                     INTEGER (1..2),

startingSymbolIndex                             INTEGER(0..13)

}

PUCCH-format3 ::=                               SEQUENCE {

nrofPRBs                                        INTEGER (1..16),

nrofSymbols                                     INTEGER (4..14),

startingSymbolIndex                             INTEGER(0..10)

}

PUCCH-format4 ::=                               SEQUENCE {

nrofSymbols                                     INTEGER (4..14),

occ-Length                                      ENUMERATED {n2,n4},

occ-Index                                       ENUMERATED {n0,n1,n2,n3},

startingSymbolIndex                             INTEGER(0..10)

Observation 1. The flexibility of PUCCH configurations in frequency domain for 3MHz can be supported with no significant impact.

Proposal 3. From RAN4 demodulation perspective, RAN4 could consider defining 3MHz requirements for PUCCH in 15kHz .

For antenna configuration, RAN4 can consider reuse the existing configuration which are 2Rx , 4Rx and 8Rx for PUCCH.

Proposal 4. Considering 2Rx , 4Rx and 8Rx for PUCCH requirements.

## 2.3 PRACH requirements

Regarding PRACH, Tables 6.3.3.1-1 and 6.3.3.1-2 in TS 38.211 indicate the PRACH preamble formats available depending on the different sequence lengths LRA and different numerology . For Table 6.3.3.1-1 covers  and ΔfRA∈1.25, 5 kHz, and Table 6.3.3.1-2 covers LRA∈139, 571, 1151 and ΔfRA=15⋅2μ kHz where μ∈0,1,2,3.

To support dedicated spectrum allocations from approximately 3 MHz up to below 5 MHz, preamble formats 0,1,2 and preamble formats A1, A2, A3, B1, B2, B3, B4, C0 and C1 with  and  can be supported.

Considering PRACH occasion in frequency domain, we can observe that only 12RBs occupied for preamble formats with  and  as shown in following Table.

Table 6.3.3.2-1: Supported combinations of ΔfRA and Δf, and the corresponding value of k.

|  | ΔfRA for PRACH | for PUSCH | , allocation expressed in number of RBs for PUSCH |  |
| --- | --- | --- | --- | --- |
| ... | ... | ... | ... | ... |
| 139 | 15 | 15 | 12 | 2 |
| ... | ... | ... | ... | ... |

  - Observation 2. Legacy formats with  and  can be supported under dedicated spectrum with approximately 3 MHz system bandwidth.

  - Regarding PRACH requirements, we notice that current release covered legacy formats with  and  requirements in TS 38.104.

Table 8.4.2.2-2: PRACH missed detection requirements for Normal Mode, 15 kHz SCS

| Number of |  | Propagation | Frequency | SNR (dB) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TX antennas | Number of RX antennas | conditions and correlation matrix (Annex G) | offset | Burst format A1 | Burst format A2 | Burst format A3 | Burst format B4 | Burst format C0 | Burst format C2 |
| 1 | 2 | AWGN | 0 | -9.3 | -12.6 | -14.2 | -16.8 | -6.3 | -12.5 |
|  |  | TDLC300-100 Low | 400 Hz | -2.1 | -4.8 | -6.6 | -8.8 | 0.8 | -4.9 |
|  | 4 | AWGN | 0 | -11.6 | -14.3 | -16.0 | -19.0 | -8.7 | -14.1 |
|  |  | TDLC300-100 Low | 400 Hz | -7.3 | -10.3 | -11.7 | -13.8 | -4.3 | -10.2 |
|  | 8 | AWGN | 0 | -13.8 | -16.7 | -18.2 | -21.2 | -11.1 | -16.6 |
|  |  | TDLC300-100 Low | 400 Hz | -11.0 | -13.9 | -15.2 | -17.3 | -8.1 | -13.9 |

Proposal 5. From RAN4 demodulation perspective, RAN4 could consider defining 3MHz requirements for PRACH.

Proposal 6. For PRACH demodulation requirements, reuse the existing requirements for 3Mhz .

# 3 Conclusion

In this contribution, we give some discussions on demodulation performance requirements for less than 5MHz  demodulation requirements , The conclusions are:

Observation 1. The flexibility of PUCCH configurations in frequency domain for 3MHz can be supported with no significant impact.

  - Observation 2. Legacy formats with  and  can be supported under dedicated spectrum with approximately 3 MHz system bandwidth.

Proposal 1. From RAN4 demodulation perspective, RAN4 could consider defining 3MHz requirements for PUSCH in 15kHz .

Proposal 2. Considering 1Tx and 2Tx two layer for PUSCH requirements.

Proposal 3. From RAN4 demodulation perspective, RAN4 could consider defining 3MHz requirements for PUCCH in 15kHz .

Proposal 4. Considering 2Rx , 4Rx and 8Rx for PUCCH requirements.

Proposal 5. From RAN4 demodulation perspective, RAN4 could consider defining 3MHz requirements for PRACH.

Proposal 6. For PRACH demodulation requirements, reuse the existing requirements for 3Mhz .

# 4 References

- RP-222645, Revised WID: NR support for dedicated spectrum less than 5MHz for FR1, RAN#95e.

- TS 38.101-1, “NR; Physical channels and modulation”, 3GPP Technical Specification, version 17.2.0.

- TS 38.211, “NR; Physical channels and modulation”, 3GPP Technical Specification, version 17.2.0.

- TS 38.331, “NR; Radio Resource Control (RRC); Protocol specification”, 3GPP Technical Specification, version 17.1.0.