3GPP TSG RAN WG4 Meeting #108-bis	R4-2315731

Xiamen, China, Oct 09th – Oct 13rd, 2023

Agenda item:	5.24.2.4

Source: 	vivo

Title: 	Discussion on Enhanced CHO configurations

Document for:	Discussion

- Introduction

In the RAN4#108 meeting, RAN4 further discussed the delay requirements on CHO with PSCell change/addition (obj.3) and CHO with CPAC (obj.4). Among them, most of issues have been reached in agreements while there still have one pending issue regarding ‘CHO-only’ case. In this paper, we would provide further analysis on it.

- Discussion

2.1 Enhanced CHO including target MCG and candidate SCGs for CPC/CPA

For the delay requirements on CHO with CPAC, there is one issue left due to different interpretation on ‘CHO-only’ from companies. Therefore, we would like to provide our understanding on ‘CHO only’.

In our interpretation, ‘CHO-only’ is another configuration which is independent of the configuration of CHO with CPAC. there is only CHO configuration is included while no configuration related to CPAC in ‘CHO-only’ configuration. Optionally, the threshold of triggered events in ‘CHO-only’ and ‘CHO with CPAC’ can be configured differently.

From procedure perspective, if both ‘CHO with CPAC’ and ‘CHO only’ configurations are configured, UE is allowed to parallelly evaluate these two configurations. Once the CHO conditions in ‘CHO-only’ configuration is satisfied firstly, UE would directly execute CHO without any further evaluation of CPAC, which is exactly what the legacy Conditional Handover procedure performs. Therefore, we can just keep the following agreement in previous meeting. And when the CHO execution condition is met but no CPC execution condition is met, if there is an available independent CHO-only configuration, the UE can directly perform the legacy conditional handover.

Agreement in RAN4#107 meeting Issue 3-2-5: if complementary CHO-only configuration is provided - Agreements: - Legacy conditional handover can be reused for conditional PCell handover if complementary CHO-only configuration is provided

Proposal 1: When the CHO execution condition is met but no CPC execution condition is met in CHO with CPAC configurations, if there is an available independent CHO-only configuration and the CHO condition in which configuration is met, UE can directly perform the legacy conditional handover based on CHO-only configuration.

- Conclusions

Proposal 1: When the CHO execution condition is met but no CPC execution condition is met in CHO with CPAC configurations, if there is an available independent CHO-only configuration and the CHO condition in which configuration is met, UE can directly perform the legacy conditional handover based on CHO-only configuration.

- Reference

[1] R4-2314332 WF on NR Mobility Enhancements (Part 2), Apple.