3GPP TSG-RAN WG4 Meeting # 108-bis                               R4-2315763

Xiamen, CN, Oct 09th – Oct 13rd, 2023

Title:	Reply LS on impact/interruption for PDCCH order RACH

Response to:	R1-2304276

Release:	Rel-18

Work Item:	NR_mob_enh2-Core

Source:	RAN WG 4

To:	RAN WG 1

Cc:

Contact person:

Name:	Lingyu Gao

E-mail Address:	gaolingyu@catt.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: None

1 Overall description

RAN4 thanks RAN1 for the LS sent in R1-2304276 asking about the delay requirements and the impact/interruption on UL Tx of serving cell due to the PRACH Tx on a candidate cell that is not a current serving cell with PUCCH/PUSCH. The related issue is copied below:

- Time gap between a PDCCH order and the corresponding PRACH transmission RAN1 discussed the time gap between a PDCCH order and the corresponding PRACH transmission for LTM. RAN1 believes that this will require that the time gap is increased at least for the following scenario - For PDCCH-order based PRACH on a candidate cell that is not a current serving cell with PUCCH/PUSCH or inter-frequency with the current serving cell RAN1 relies on RAN4: - to verify the need for the above additional latency and, if so, the corresponding value is needed. - to investigate any impact/interruption on UL Tx of serving cell due to the PRACH Tx on a candidate cell that is not a current serving cell with PUCCH/PUSCH - to verify the need for any update is required to ΔBWPSwitching, ΔDelay if so, the corresponding values and whether UE capability is needed Potential RAN1 spec update will be based on RAN4’s feedback.

RAN4 reached initial agreements on the delay requirements for PDCCH-order RACH on candidate cell in the last meeting. Due to limited time, interruption requirements were not discussed last meeting.

In this meeting, RAN4 further discuss the impact/interruption on UL Tx and/or DL Rx of serving cell due to the PRACH Tx on a candidate cell that is not a current serving cell with PUCCH/PUSCH. RAN4 sincerely asks RAN1 to consider potential spec update based on RAN4’s following feedback.

RAN4 feedback:

For any impact/interruption on UL Tx and/or DL Rx of serving cell due to the PRACH Tx on a candidate cell that is not a current serving cell with PUCCH/PUSCH,

- There will be interruption on UL Tx of all the serving cells during PRACH transmission.

- If RACH bandwidth of neighbor cell is not in the UL active BWP, there will be interruption on UL Tx and DL Rx of all the serving cells due to RF retuning to candidate cell.

- If RACH bandwidth of neighbor cell is not in the UL active BWP, there will be interruption on UL Tx and DL Rx of all the serving cells due to returning to serving cell after performing RACH on neighbour cell.

Note: The above interruption requirements will be captured in RAN4’s spec.

2 Actions

To RAN WG1 group

ACTION: 	RAN4 sincerely asks RAN1 to take into account the above feedback into consideration.

3 Dates of Next TSG RAN WG4 Meetings

TSG-RAN WG4 Meeting #109	13 Nov –17 Nov 2023	     Chicago , US

TSG-RAN WG4 Meeting #109-bis	22 Jan – 26 Jan 2023		     TBD