3GPP TSG-RAN WG4 Meeting # 108-bis	R4-2315766

# Xiamen, China, October 09 – October 13, 2023

Source:				Ericsson

Title:					NTN enhancements – UE RF requirements

Agenda item:		5.26.5.1

Document for:		Approval

# Introduction

This document further discusses the NTN UE RF requirements aspects in the scope of the NTN enhancements WI ([1]),  based on the Way Forward [2] agreed in last RAN4#108 meeting, and our contribution ([10]) from that same meeting.

# Discussion

## NTN UE phased array antenna

RAN4 has discussed the NTN UEs characteristics and requirements considering parabolic and phased array types of antenna, with the intention to specify RF requirements for NTN UEs and each type of antenna.

Nevertheless, it should be noted that, in the scope of the coexistence study, NTN UE with phased array antenna is currently out of scope.

There are indeed diverging views on the fact that, from a coexistence study’s point of view, both antenna type would give the same results from coexistence’s point of view. And, so far, RAN4 only agreed on the parameters for parabolic antenna; the parameters for phased array antenna remain FFS (see [12] table 2.3.1-4).

Moreover, the coexistence study considering NTN UE with phased array antenna is not precluded but clearly stated as FFS:

To our understanding, RAN4 would not be able to finalize the RF requirements for NTN UE with phased array type of antenna without any conclusion from the coexistence study. Support of phased array antenna should then be clarified.

Proposal1: RAN4 shall clarify if NTN UE with phased array antenna is for further study or not. And if not, the coexistence study shall consider this case.

## Off-axis EIRP density

### Background

In RAN4#107 meeting, it was agreed ([2]) that on-axis and off-axis EIRP requirements shall be defined. In last RAN4#108 meeting, the contribution [11] compared the ITU and FCC regulations with the characteristics used for the coexistence study. Its conclusion was that the considered NTN UEs would be compliant with ITU and FCC regulations. Furthermore, it was proposed to use ITU S.524-9 as the main reference.

### Analysis

ITU S.524-9 specifies the following EIRP density for the Ka-band:

FCC 25.218 specifies the following EIRP density envelope for the Ka-band:

Comparing ITU and FCC requirements, they are not strictly equivalent:

- The last 2 intervals of limits’ applicablility are not identical:

  - 9.2 o to 48o  and 48 o to 180 o for ITU S.524-9.

  - 9.2 o to 19.1o and 19.1o to 180o for FCC.

- The reference bandwidth is 40 kHz for ITU S.524-9 and 1 MHz for FCC, meaning FCC would allow some spurs that ITU would not.

- After doing the reference bandwith scaling (~14dB), the specified limits are not strictly identical. FCC would be 0.5 dB stricter in the tangent plane, while it would 2.5dB more relaxed in the perpendicular plane.

### Conclusion  and proposals

Based on the observations made in previous sub-sections, it seems not relevant to only consider one of these 2 types of EIRP density requirements (FCC or ITU) for NTN UE operating in the Ka-band.

One option would be to merge the FCC and ITU requirements, taking the most stringent limits of both. This could be done if RAN4 would like to specify one EIRP density requirement applicable for all NTN Ka-bands.

On the other hand, as RAN4 has specified disctinct Ka-band bands for CEPT and FCC, we would make the following proposals:

Proposal2: Specify EIRP density envelope as specified in FCC 25.218 for bands n510 and n511.

Proposal3: Specify EIRP density envelope as specified in ITU S.524-9 for band n512.

It should be noted that those requirements could be specified for each band, not using the NS feature.

Observation1: FCC and ITU-R S.524-9 EIRP density requirements could be specified for each NTN Ka-band without using NS feature.

## Off-axis and On-axis spurious requirements

Similar to the off-axis and on-axis EIRP requirements agreement, to control the level of interference with other satellite and/or services, the on-axis and off-axis spurious requirements shall also be defined.

Proposal4: RAN4 shall specify on-axis and off-axis spurious requirements. EN 301 459 could be used as a basis.

## Other considerations related to mobile VSAT/ESIM/ESOMP

At least the following requirements should also be considered for mobile VSAT / ESIM / ESOMP:

- From ECC Dec(13)01:

Figure 1: EIRP spectral density - ECC Decision(13)01

Figure 2: PFD mask for ESOMP on aircraft - ECC Decision(13)01

Figure 3: PFD max value for ESOMPs on vessels - ECC Decision(13)01

- From ITU-R Resolution 169 (WRC-19)

Figure 4: EIRP spectral density for maritime ESIM- ITU-R Resolution 169

Proposal5: Capture at least the CEPT and ITU requirements mentioned above:

- EIRP spectral density, PFD mask for ESOMP on aircraft and PFD max value for ESOMP on vessel  from ECC Dec(13)01.

- EIRP spectral density for maritime ESIM from ITU-R Resolution 169.

# Conclusion

In this contribution, we analyzed the UE RF open issues for the NTN Ka-band. We made the following proposals:

Proposal1: RAN4 shall clarify if NTN UE with phased array antenna is for further study or not. And if not, the coexistence study shall consider this case.

Proposal2: Specify EIRP density envelope as specified in FCC 25.218 for bands n510 and n511.

Proposal3: Specify EIRP density envelope as specified in ITU S.524-9 for band n512.

It should be noted that those requirements could be specified for each band, not using the NS feature.

Observation1: FCC and ITU-R S.524-9 EIRP density requirements could be specified for each NTN Ka-band without using NS feature.

Proposal4: RAN4 shall specify on-axis and off-axis spurious requirements. EN 301 459 could be used as a basis.

Proposal5: Capture at least the CEPT and ITU requirements mentioned above:

- EIRP spectral density, PFD mask for ESOMP on aircraft and PFD max value for ESOMP on vessel  from ECC Dec(13)01.

- EIRP spectral density for maritime ESIM from ITU-R Resolution 169.

# Reference

- RP-230809, NR NTN (Non-Terrestrial Networks) enhancements, Thales

- R4-2314934, WF on UE RF requirements for NR NTN, ZTE, Samsung

- ECC Decision(05)01, The use of the band 27.5-29.5 GHz by the Fixed Service and uncoordinated Earth stations of the Fixed-Satellite Service (Earth-to-space)

- ECC Decision(13)01,  The harmonised use, free circulation and exemption from individual licensing of Earth Stations On Mobile Platforms (ESOMPs) within the frequency bands 17.3-20.2 GHz and 27.5-30.0 GHz

- ITU-R S.2278, Use of very small aperture terminals (VSATs).

- FCC 47 CFR 25.103

- Uncoordinated ubiquitous FSS earth station coexistence with FWA in the 28 GHz band, ACMA

- R4-2216515, NTN enhancements – Regulatory aspects, Ericsson

- R4-2302714, Satellite broadband user equipment, HISPASAT, Hughes Network Systems, Thales, ESA, Eutelsat, Lockheed Martin, Intelsat, Inmarsat, Airbus

- R4-2313241, NTN enhancements – UE RF requirements, Ericsson

- R4-2312120, Updates for NTN UE terminal requirements and NF in above 10 GHz, Thales

- R4-2313890, Simulation assumption for NTN co-existence study, Samsung