3GPP TSG-RAN WG4 Meeting#108bis	R4-2315780

# Xiamen, China, 9 – 13 October 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | CRNum | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enhancement of the channel raster: entries for UE-specific channel bandwidths |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_channel_raster_enh-Core |  |  |  |  |  | Date: |  |  | 2023-10-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Enhance the channel raster such that configuring a narrower UE channel BW inside a wider gNB channel BW is always possible also in bands with 100 kHz channel raster. Correct the channel raster to resource element mapping [within the RF channel]. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Instead of a new channel raster, introduce additional applicable entries on the global raster only for the UE-specific channel bandwidth located within an NR carrier on the 100 kHz channel raster. Clause 3.1: definition of a UE-specific channel bandwidth. Clause 5.3.1: add information that a UE channel bandwidth must not necessarily cover the entire carrier bandwidth. Clause 5.4.2.2: correction of the mapping of the carrier resource element mapping to the channel raster for an RF channel. Clause 5.4.2.3: introduction of additional global raster entries for UE-specific channel bandwidths configured within an NR carrier. Introduce a limited set of requirements applicable for supported UE channel bandwidths configured at these entries: Clause 6.2.1: maximum output power for the configured bandwidth. Clause 6.5.0 (new): - occupied bandwidth according to clause 6.5.1 for the configured bandwidth - unwanted emissions according to clauses 6.5.2 and 6.5.3 outside the NR carrier with a bandwidth configured within. Clause 7.3.1: the REFSENS requirements according to clause 7.3.2 for the configured bandwidth. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Configuring a narrower UE channel BW inside a wider gNB channel BW may not always be possible in bands with 100 kHz channel raster for some UE implementations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 5.3.1, 5.4.2.2, 5.4.2.3, 6.2.1, 6.5.0 (new), 7.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  | . |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< start of changes >

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Aggregated Channel Bandwidth: The RF bandwidth in which a UE transmits and receives multiple contiguously aggregated carriers.

Carrier aggregation: Aggregation of two or more component carriers in order to support wider transmission bandwidths.

Carrier aggregation band: A set of one or more operating bands across which multiple carriers are aggregated with a specific set of technical requirements.

Carrier aggregation bandwidth class: A class defined by the aggregated transmission bandwidth configuration and maximum number of component carriers supported by a UE.

Carrier aggregation configuration: A combination of CA operating band(s) and CA bandwidth class(es) supported by a UE.

Con-current operation: The simultaneous transmission and reception of sidelink and Uu interfaces while operation is agnostic of the service used on each interface.

Contiguous carriers: A set of two or more carriers configured in a spectrum block where there are no RF requirements based on co-existence for un-coordinated operation within the spectrum block.

Contiguous resource allocation: A resource allocation of consecutive resource blocks within one carrier or across contiguously aggregated carriers. The gap between contiguously aggregated carriers due to the nominal channel spacing is allowed.

Contiguous spectrum: Spectrum consisting of a contiguous block of spectrum with no sub-block gaps.

Inter-band carrier aggregation: Carrier aggregation of component carriers in different operating bands.

NOTE:	Carriers aggregated in each band can be contiguous or non-contiguous.

Intra-band contiguous carrier aggregation: Contiguous carriers aggregated in the same operating band.

Intra-band non-contiguous carrier aggregation: Non-contiguous carriers aggregated in the same operating band.

RedCap UE: The UE with reduced capabilities as defined in clause 4.2.21.1 from TS38.306 [15].

Sub-band:  For a UE that supports shared spectrum channel access in wideband operation, a sub-band is the set of RBs within an approximately 20 MHz segment of the channel where the wideband channel is uniformly divided into an integer number of 20 MHz sub-bands.  Sub-bands may be separately allocated in uplink and downlink.

Sub-block: This is one contiguous allocated block of spectrum for transmission and reception by the same UE. There may be multiple instances of sub-blocks within an RF bandwidth.

Sub-block bandwidth: The bandwidth of one sub-block.

Sub-block gap: A frequency gap between two consecutive sub-blocks within an RF bandwidth, where the RF requirements in the gap are based on co-existence for un-coordinated operation.

UE-specific channel bandwidth: a UE channel bandwidth configured within an NR carrier as part of serving cell configuration.

UE transmission bandwidth configuration: Set of resource blocks located within the UE channel bandwidth which may be used for transmitting or receiving by the UE.

Vehicular UE: A UE embedded in a vehicle, permanently connected to an embedded antenna system that radiates externally for NR operating bands.

NOTE:	Vehicular UE does not refer to other UE form factors placed inside the vehicle.

Wideband operation: For a UE that supports shared spectrum channel access, wideband operation refers to operation within a channel larger than 20 MHz in which intra-cell guard bands may be configured to distinguish individual RB-sets

## 3.2	Symbols

< text omitted >

### 5.3.1	General

The UE channel bandwidth supports a single NR RF carrier or parts thereof in the uplink or downlink at the UE. From a BS perspective, different UE channel bandwidths may be supported within the same spectrum for transmitting to and receiving from UEs connected to the BS. Transmission of multiple carriers to the same UE (CA) or multiple carriers to different UEs within the BS channel bandwidth can be supported.

From a UE perspective, the UE is configured with one or more BWP / carriers, each with its own UE channel bandwidth. The UE does not need to be aware of the BS channel bandwidth or how the BS allocates bandwidth to different UEs.

The placement of the UE channel bandwidth for each UE carrier is flexible but can only be completely within the BS channel bandwidth.

The relationship between the channel bandwidth, the guardband and the maximum transmission bandwidth configuration is shown in Figure 5.3.1-1.

Figure 5.3.1-1: Definition of the channel bandwidth and the maximum transmission bandwidth configuration for one NR channel

### 5.3.2	Maximum transmission bandwidth configuration

< text omitted >

#### 5.4.2.2	Channel raster to resource element mapping

The mapping between the RF reference frequency on the channel raster and the corresponding resource element is given in Table 5.4.2.2-1 and can be used to identify the RF channel position. The mapping depends on the total number of RBs that are allocated in the channel and applies to both UL and DL. The mapping must apply to at least one numerology supported by the UE.

Table 5.4.2.2-1: Channel raster to resource element mapping

|  | NRBmod2 = 0 | NRBmod2 = 1 |
| --- | --- | --- |
| Resource element index | 0 | 6 |
| Physical resource block number |  |  |

nPRB is the PRB index, k the resource element index within this PRB and NRB  the transmission bandwidth configuration specified in sub-clause 5.3.1.

#### 5.4.2.3	Channel raster entries for each operating band

The RF channel positions on the channel raster in each NR operating band are given through the applicable NR-ARFCN in Table 5.4.2.3-1, using the channel raster to resource element mapping in clause 5.4.2.2.

For NR operating bands with 100 kHz channel raster, ΔFRaster = 20 × ΔFGlobal. In this case every 20th NR-ARFCN within the operating band are applicable for the channel raster within the operating band and the step size for the channel raster in Table 5.4.2.3-1 is given as <20>.

For NR operating bands with 15 kHz channel raster below 3GHz, ΔFRaster = I × ΔFGlobal, where I ϵ {3,6}. Every Ith NR-ARFCN within the operating band are applicable for the channel raster within the operating band and the step size for the channel raster in Table 5.4.2.3-1 is given as < I >.

For NR operating bands with 15 kHz channel raster above 3GHz, ΔFRaster = I × ΔFGlobal, where I ϵ {1,2}. Every Ith  NR-ARFCN within the operating band are applicable for the channel raster within the operating band and the step size for the channel raster in table 5.4.2.3-1 is given as <I>.

In frequency bands with two or more ΔFRaster, the higher ΔFRaster: For 15 kHz and 30 kHz channel raster applies to channels using only the SCS that is equal to or larger than the higher ΔFRaster and SSB SCS is equal to the higher ∆FRaster.

A UE indicating [ue-SpecCBW-WithoutChRasterRestriction] for an NR operating band with a 100 kHz channel raster can be configured with a UE-specific channel bandwidth within an RF channel by ServingCellConfig without a restriction that this UE channel bandwidth be mapped on the 100 kHz channel raster according to clause 5.4.2.2 [38.306].

For UEs supporting [ue-SpecCBW-WithoutChRasterRestriction], the RF positions of UE-specific channel bandwidths within an RF channel are given through the applicable NR-ARFCN in Table 5.4.2.3-5 for NR operating bands with a 100 kHz channel raster. In this case every 2nd and 4th NR-ARFCN within an RF channel on the channel raster are applicable for the 15 kHz and 30 kHz numerologies, respectively. The mapping of these NR-ARFCN to a resource element within the UE-specific channel bandwidth is given by clause 5.4.2.2 but with NRB the carrierBandwidth of the downlinkChannelBW-PerSCS-List and the uplinkChannelBW-PerSCS-List configured by ServingCellConfig for the respective transmission direction and numerology.

Table 5.4.2.3-1: Applicable NR-ARFCN per operating band

| NR operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n1 | 100 | 384000 – <20> – 396000 | 422000 – <20> – 434000 |
| n2 | 100 | 370000 – <20> – 382000 | 386000 – <20> – 398000 |
| n3 | 100 | 342000 – <20> – 357000 | 361000 – <20> – 376000 |
| n5 | 100 | 164800 – <20> – 169800 | 173800 – <20> – 178800 |
| n7 | 100 | 500000 – <20> – 514000 | 524000 – <20> – 538000 |
| n8 | 100 | 176000 – <20> – 183000 | 185000 – <20> – 192000 |
| n12 | 100 | 139800 – <20> – 143200 | 145800 – <20> – 149200 |
| n13 | 100 | 155400 – <20> – 157400 | 149200 – <20> – 151200 |
| n14 | 100 | 157600 – <20> – 159600 | 151600 – <20> – 153600 |
| n18 | 100 | 163000 – <20> – 166000 | 172000 – <20> – 175000 |
| n20 | 100 | 166400 – <20> – 172400 | 158200 – <20> – 164200 |
| n24 | 100 | 325300 – <20> – 332100 | 305000 – <20> – 311800 |
| n25 | 100 | 370000 – <20> – 383000 | 386000 – <20> – 399000 |
| n26 | 100 | 162800 – <20> – 169800 | 171800 – <20> – 178800 |
| n28 | 100 | 140600 – <20> – 149600 | 151600 – <20> – 160600 |
| n29 | 100 | N/A | 143400 – <20> – 145600 |
| n30 | 100 | 461000 – <20> – 463000 | 470000 – <20> – 472000 |
| n34 | 100 | 402000 – <20> – 405000 | 402000 – <20> – 405000 |
| n38 | 100 | 514000 – <20> – 524000 | 514000 – <20> – 524000 |
| n39 | 100 | 376000 – <20> – 384000 | 376000 – <20> – 384000 |
| n40 | 100 | 460000 – <20> – 480000 | 460000 – <20> – 480000 |
| n41 | 15 | 499200 – <3> – 537999 | 499200 – <3> – 537999 |
|  | 30 | 499200 – <6> – 537996 | 499200 – <6> – 537996 |
| n462 | 15 | 743334 – <1> – 795000 | 743334 – <1> – 795000 |
| n47 | 15 | 790334 – <1> – 795000 | 790334 – <1> – 795000 |
| n48 | 15 | 636667 – <1> – 646666 | 636667 – <1> – 646666 |
|  | 30 | 636668 – <2> – 646666 | 636668 – <2> – 646666 |
| n50 | 100 | 286400 – <20> – 303400 | 286400 – <20> – 303400 |
| n51 | 100 | 285400 – <20> – 286400 | 285400 – <20> – 286400 |
| n53 | 100 | 496700 – <20> – 499000 | 496700 – <20> – 499000 |
| n54 | 100 | 334000 – <20> – 335000 | 334000 – <20> – 335000 |
| n65 | 100 | 384000 – <20> – 402000 | 422000 – <20> – 440000 |
| n66 | 100 | 342000 – <20> – 356000 | 422000 – <20> – 440000 |
| n67 | 100 | N/A | 147600 – <20> – 151600 |
| n70 | 100 | 339000 – <20> – 342000 | 399000 – <20> – 404000 |
| n71 | 100 | 132600 – <20> – 139600 | 123400 – <20> – 130400 |
| n74 | 100 | 285400 – <20> – 294000 | 295000 – <20> – 303600 |
| n75 | 100 | N/A | 286400 – <20> – 303400 |
| n76 | 100 | N/A | 285400 – <20> – 286400 |
| n77 | 15 | 620000 – <1> – 680000 | 620000 – <1> – 680000 |
|  | 30 | 620000 – <2> – 680000 | 620000 – <2> – 680000 |
| n78 | 15 | 620000 – <1> – 653333 | 620000 – <1> – 653333 |
|  | 30 | 620000 – <2> – 653332 | 620000 – <2> – 653332 |
| n79 | 15 | 693334 – <1> – 733333 | 693334 – <1> – 733333 |
|  | 30 | 693334 – <2> – 733332 | 693334 – <2> – 733332 |
| n80 | 100 | 342000 – <20> – 357000 | N/A |
| n81 | 100 | 176000 – <20> – 183000 | N/A |
| n82 | 100 | 166400 – <20> – 172400 | N/A |
| n83 | 100 | 140600 – <20> –149600 | N/A |
| n84 | 100 | 384000 – <20> – 396000 | N/A |
| n85 | 100 | 139600 – <20> – 143200 | 145600 – <20> – 149200 |
| n86 | 100 | 342000 – <20> – 356000 | N/A |
| n89 | 100 | 164800 – <20> – 169800 | N/A |
| n90 | 15 | 499200 – <3> – 537999 | 499200 – <3> – 537999 |
|  | 30 | 499200 – <6> – 537996 | 499200 – <6> – 537996 |
|  | 100 | 499200 – <20> – 538000 | 499200 – <20> – 538000 |
| n91 | 100 | 166400 – <20> – 172400 | 285400 – <20> – 286400 |
| n92 | 100 | 166400 – <20> – 172400 | 286400 – <20> – 303400 |
| n93 | 100 | 176000 – <20> – 183000 | 285400 – <20> – 286400 |
| n94 | 100 | 176000 – <20> – 183000 | 286400 – <20> – 303400 |
| n95 | 100 | 402000 – <20> – 405000 | N/A |
| n963 | 15 | 795000 – <1> – 875000 | 795000 – <1> – 875000 |
| n97 | 100 | 460000 – <20> – 480000 | N/A |
| n98 | 100 | 376000 – <20> – 384000 | N/A |
| n99 | 100 | 325300 – <20> – 332100 | N/A |
| n100 | 100 | 174880 – <20> – 176000 | 183880 – <20> – 185000 |
| n101 | 100 | 380000 – <20> – 382000 | 380000 – <20> – 382000 |
| n1024 | 15 | 795000 – <1> – 828333 | 795000 – <1> – 828333 |
| n104 | 15 | 828334 – <1> – 875000 | 828334 – <1> – 875000 |
|  | 30 | 828334 – <2> – 875000 | 828334 – <2> – 875000 |
| n105 | 100 | 132600 – <20> – 140600 | 122400 – <20> – 130400 |
| NOTE 1: The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. NOTE 2: The following NREF are allowed for operation in Band n46: see Table 5.4.2.3-2. NOTE 3: The following NREF are allowed for operation in Band n96: see Table 5.4.2.3-3. NOTE 4: The following NREF are allowed for operation in Band n102: see Table 5.4.2.3-4. |  |  |  |

Table 5.4.2.3-2: Allowed NREF (NR-ARFCN) for operation in Band n46

| Channel Bandwidth | Allowed NREF |
| --- | --- |
| 10 MHz | 782000, 788668 |
| 20 MHz | 744000, 745332, 746668, 748000, 749332, 750668, 752000, 753332, 754668, 756000, 765332, 766668, 768000, 769332, 770668, 772000, 773332, 774668, 776000, 777332, 778668, 780000, 781332, 783000, 784332, 785668, 787000, 788332, 789668, 791000, 792332, 793668 |
| 40 MHz | 744668, 746000, 748668, 751332, 754000, 755332, 766000, 767332, 770000, 772668, 775332, 778000, 780668, 783668, 786332, 787668, 790332, 793000 |
| 60 MHz | 745332, 746668, 748000, 752000, 753332, 754668, 766668, 768000, 769332, 773332, 774668, 778668, 780000, 784332, 785668, 791000, 792332 |
| 80 MHz | 746000, 747332, 752668, 754000, 767332, 768668, 774000, 779332, 785000, 791668 |
| 100 MHz | 746668, 753332, 768000, 791000 |
| NOTE: 10 MHz channel bandwidth shall only apply in certain regions where the absence of non 3GPP technologies can be guaranteed on a long-term basis in this version of specification. |  |

Table 5.4.2.3-3: Allowed NREF (NR-ARFCN) for operation in Band n96

| Channel Bandwidth | Allowed NREF |
| --- | --- |
| 20 MHz | 797000, 798332, 799668, 801000, 802332, 803668, 805000, 806332, 807668, 809000, 810332, 811668, 813000, 814332, 815668, 817000, 818332, 819668, 821000, 822332, 823668, 825000, 826332, 827668, 829000, 830332, 831668, 833000, 834332, 835668, 837000, 838332, 839668, 841000, 842332, 843668, 845000, 846332, 847668, 849000, 850332, 851668, 853000, 854332, 855668, 857000, 858332, 859668, 861000, 862332, 863668, 865000, 866332, 867668, 869000, 870332, 871668, 873000, 874332 |
| 40 MHz | 797668, 800332, 803000, 805668, 808332, 811000, 813668, 816332, 819000, 821668, 824332, 827000, 829668, 832332, 835000, 837668, 840332, 843000, 845668, 848332, 851000, 853668, 856332, 859000, 861668, 864332, 867000, 869668, 872332 |
| 60 MHz | 798332, 799668, 803668, 805000, 809000, 810332, 814332, 815668, 819668, 821000, 825000, 826332, 830332, 831668, 835668, 837000, 841000, 842332, 846332, 847668, 851668, 853000, 857000, 858332, 862332, 863668, 867668, 869000, 873000 |
| 80 MHz | 799000, 804332, 809668, 815000, 820332, 825668, 831000, 836332, 841668, 847000, 852332, 857668, 863000, 868332 |
| 100 MHz | 799668, 803668, 810332, 814332, 821000, 825000, 831668, 835668, 842332, 846332, 853000, 857000, 863668, 867668, 869000, 870332, 871668 |

Table 5.4.2.3-4: Allowed NREF (NR-ARFCN) for operation in Band n102

| Channel Bandwidth | Allowed NREF |
| --- | --- |
| 20 MHz | 797000, 798332, 799668, 801000, 802332, 803668, 805000, 806332, 807668, 809000, 810332, 811668, 813000, 814332, 815668, 817000, 818332, 819668, 821000, 822332, 823668, 825000, 826332, 827668 |
| 40 MHz | 797668, 800332, 803000, 805668, 808332, 811000, 813668, 816332, 819000, 821668, 824332, 827000 |
| 60 MHz | 798332, 799668, 803668, 805000, 809000, 810332, 814332, 815668, 819668, 821000, 825000, 826332 |
| 80 MHz | 799000, 804332, 809668, 815000, 820332, 825668 |
| 100 MHz | 799668, 803668, 810332, 814332, 821000, 825000 |

Table 5.4.2.3-5: Applicable NR-ARFCN for the RF position of a configured UE-specific channel bandwidth within an NR carrier for operating bands with a 100 kHz channel raster

| NR operating band | SCS of NR carrier (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n1 | 15 | 384000 – <2> – 396000 | 422000 – <2> – 434000 |
|  | 30 | 384000 – <4> – 396000 | 422000 – <4> – 434000 |
| n2 | 15 | 370000 – <2> – 382000 | 386000 – <2> – 398000 |
|  | 30 | 370000 – <4> – 382000 | 386000 – <4> – 398000 |
| n3 | 15 | 342000 – <2> – 357000 | 361000 – <2> – 376000 |
|  | 30 | 342000 – <4> – 357000 | 361000 – <4> – 376000 |
| n5 | 15 | 164800 – <2> – 169800 | 173800 – <2> – 178800 |
|  | 30 | 164800 – <4> – 169800 | 173800 – <4> – 178800 |
| n7 | 15 | 500000 – <2> – 514000 | 524000 – <2> – 538000 |
|  | 30 | 500000 – <4> – 514000 | 524000 – <4> – 538000 |
| n8 | 15 | 176000 – <2> – 183000 | 185000 – <2> – 192000 |
|  | 30 | 176000 – <4> – 183000 | 185000 – <4> – 192000 |
| n12 | 15 | 139800 – <2> – 143200 | 145800 – <2> – 149200 |
|  | 30 | 139800 – <4> – 143200 | 145800 – <4> – 149200 |
| n13 | 15 | 155400 – <2> – 157400 | 149200 – <2> – 151200 |
| n14 | 15 | 157600 – <2> – 159600 | 151600 – <2> – 153600 |
| n18 | 15 | 163000 – <2> – 166000 | 172000 – <2> – 175000 |
|  | 30 | 163000 – <4> – 166000 | 172000 – <4> – 175000 |
| n20 | 15 | 166400 – <2> – 172400 | 158200 – <2> – 164200 |
|  | 30 | 166400 – <4> – 172400 | 158200 – <4> – 164200 |
| n24 | 15 | 325300 – <2> – 332100 | 305000 – <2> – 311800 |
| n25 | 15 | 370000 – <2> – 383000 | 386000 – <2> – 399000 |
|  | 30 | 370000 – <4> – 383000 | 386000 – <4> – 399000 |
| n26 | 15 | 162800 – <2> – 169800 | 171800 – <2> – 178800 |
|  | 30 | 162800 – <4> – 169800 | 171800 – <4> – 178800 |
| n28 | 15 | 140600 – <2> – 149600 | 151600 – <2> – 160600 |
|  | 30 | 140600 – <4> – 149600 | 151600 – <4> – 160600 |
| n29 | 15 | N/A | 143400 – <2> – 145600 |
| n30 | 15 | 461000 – <2> – 463000 | 470000 – <2> – 472000 |
| n34 | 15 | 402000 – <2> – 405000 | 402000 – <2> – 405000 |
|  | 30 | 402000 – <4> – 405000 | 402000 – <4> – 405000 |
| n38 | 15 | 514000 – <2> – 524000 | 514000 – <2> – 524000 |
|  | 30 | 514000 – <4> – 524000 | 514000 – <4> – 524000 |
| n39 | 15 | 376000 – <2> – 384000 | 376000 – <2> – 384000 |
|  | 30 | 376000 – <4> – 384000 | 376000 – <4> – 384000 |
| n40 | 15 | 460000 – <2> – 480000 | 460000 – <2> – 480000 |
|  | 30 | 460000 – <4> – 480000 | 460000 – <4> – 480000 |
| n50 | 15 | 286400 – <2> – 303400 | 286400 – <2> – 303400 |
|  | 30 | 286400 – <4> – 303400 | 286400 – <4> – 303400 |
| n53 | 15 | 496700 – <2> – 499000 | 496700 – <2> – 499000 |
| n65 | 15 | 384000 – <2> – 402000 | 422000 – <2> – 440000 |
|  | 30 | 384000 – <4> – 402000 | 422000 – <4> – 440000 |
| n66 | 15 | 342000 – <2> – 356000 | 422000 – <2> – 440000 |
|  | 30 | 342000 – <4> – 356000 | 422000 – <4> – 440000 |
| n67 | 15 | N/A | 147600 – <2> – 151600 |
|  | 30 | N/A | 147600 – <4> – 151600 |
| n70 | 15 | 339000 – <2> – 342000 | 399000 – <2> – 404000 |
|  | 30 | 339000 – <4> – 342000 | 399000 – <4> – 404000 |
| n71 | 15 | 132600 – <2> – 139600 | 123400 – <2> – 130400 |
|  | 30 | 132600 – <4> – 139600 | 123400 – <4> – 130400 |
| n74 | 15 | 285400 – <2> – 294000 | 295000 – <2> – 303600 |
|  | 30 | 285400 – <4> – 294000 | 295000 – <4> – 303600 |
| n75 | 15 | N/A | 286400 – <2> – 303400 |
|  | 30 | N/A | 285400 – <4> – 286400 |
| n80 | 15 | 342000 – <2> – 357000 | N/A |
|  | 30 | 342000 – <4> – 357000 | N/A |
| n81 | 15 | 176000 – <2> – 183000 | N/A |
|  | 30 | 176000 – <4> – 183000 | N/A |
| n82 | 15 | 166400 – <2> – 172400 | N/A |
|  | 30 | 166400 – <4> – 172400 | N/A |
| n83 | 15 | 384000 – <2> – 396000 | N/A |
|  | 30 | 384000 – <4> – 396000 | N/A |
| n84 | 15 | 384000 – <2> – 396000 | N/A |
|  | 30 | 384000 – <4> – 396000 | N/A |
| n85 | 15 | 139600 – <2> – 143200 | 145600 – <2> – 149200 |
|  | 30 | 139600 – <4> – 143200 | 145600 – <4> – 149200 |
| n86 | 15 | 342000 – <2> – 356000 | N/A |
|  | 30 | 342000 – <4> – 356000 | N/A |
| n89 | 15 | 164800 – <2> – 169800 | N/A |
|  | 30 | 164800 – <4> – 169800 | N/A |
| n90 | 15 | 499200 – <2> – 538000 | 499200 – <2> – 538000 |
|  | 30 | 499200 – <4> – 538000 | 499200 – <4> – 538000 |
| n91 | 15 | 166400 – <2> – 172400 | N/A |
| n92 | 15 | 166400 – <2> – 172400 | 286400 – <2> – 303400 |
|  | 30 | 166400 – <4> – 172400 | 286400 – <4> – 303400 |
| n93 | 15 | 176000 – <2> – 183000 | N/A |
| n94 | 15 | 176000 – <2> – 183000 | 286400 – <2> – 303400 |
|  | 30 | 176000 – <4> – 183000 | 286400 – <4> – 303400 |
| n95 | 15 | 402000 – <2> – 405000 | N/A |
|  | 30 | 402000 – <4> – 405000 | N/A |
| n97 | 15 | 460000 – <2> – 480000 | N/A |
|  | 30 | 460000 – <4> – 480000 | N/A |
| n98 | 15 | 376000 – <2> – 384000 | N/A |
|  | 30 | 376000 – <4> – 384000 | N/A |
| n99 | 15 | 325300 – <2> – 332100 | N/A |
| n101 | 15 | 380000 – <2> – 382000 | 380000 – <2> – 382000 |
| n105 | 15 | 132600 – <2> – 140600 | 122400 – <2> – 130400 |
|  | 30 | 132600 – <4> – 140600 | 122400 – <4> – 130400 |
| NOTE 1: Values of NR-ARFCN that designate RF positions of a UE-specific channel bandwidth not configurable by ServingCellConfig within a NR carrier with carrier frequencies as specified in Table 5.4.3.2-1 shall not be used. NOTE 2: Operating bands or SCS for carriers of operating bands that do not allow configuration of UE-specific channel bandwidths within the maximum channel bandwidth supported by each band and SCS according to Table 5.3.5-1 are not listed. |  |  |  |

### 5.4.3	Synchronization raster

< text omitted >

## 6.2	Transmitter power

### 6.2.1	UE maximum output power

The following UE Power Classes define the maximum output power for any transmission bandwidth within the channel bandwidth of NR carrier unless otherwise stated. The period of measurement shall be at least one sub frame (1ms).

Table 6.2.1-1: UE Power Class

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n1 |  |  |  |  | 26 | +2/-3 | 23 | ±2 |
| n2 |  |  |  |  |  |  | 23 | ±23 |
| n3 |  |  |  |  | 26 | +2/-33 | 23 | ±23 |
| n5 |  |  |  |  |  |  | 23 | ±2 |
| n7 |  |  |  |  |  |  | 23 | ±23 |
| n8 |  |  |  |  |  |  | 23 | ±23 |
| n12 |  |  |  |  |  |  | 23 | ±23 |
| n13 |  |  |  |  |  |  | 23 | ±2 |
| n14 | 316 | +2/-3 |  |  |  |  | 23 | ±2 |
| n18 |  |  |  |  |  |  | 23 | ±2 |
| n20 |  |  |  |  |  |  | 23 | ±23 |
| n24 |  |  |  |  |  |  | 23 | +2/-33 |
| n25 |  |  |  |  |  |  | 23 | ±23 |
| n26 |  |  |  |  |  |  | 23 | ±23 |
| n28 |  |  |  |  |  |  | 23 | +2/-2.5 |
| n30 |  |  |  |  |  |  | 23 | ±2 |
| n34 |  |  | 295 | +2/-3 | 26 | +2/-3 | 23 | ±2 |
| n38 |  |  |  |  |  |  | 23 | ±2 |
| n39 |  |  |  |  | 26 | +2/-3 | 23 | ±2 |
| n40 |  |  | 295 | +2/-3 | 26 | +2/-3 | 23 | ±2 |
| n41 |  |  | 295 | +2/-33 | 26 | +2/-33 | 23 | ±23 |
| n47 |  |  |  |  |  |  | 23 | ±2 |
| n48 |  |  |  |  |  |  | 23 | +2/-3 |
| n50 |  |  |  |  |  |  | 23 | ±2 |
| n51 |  |  |  |  |  |  | 23 | ±2 |
| n53 |  |  |  |  |  |  | 23 | ±2 |
| n54 |  |  |  |  |  |  | 23 | ±2 |
| n65 |  |  |  |  |  |  | 23 | ±2 |
| n66 |  |  |  |  |  |  | 23 | ±2 |
| n70 |  |  |  |  |  |  | 23 | ±2 |
| n71 | 316 | +2/-3 |  |  |  |  | 23 | +2/-2.5 |
| n74 |  |  |  |  |  |  | 23 | ±2 |
| n77 | 316 | +2/-3 | 295 | +2/-3 | 26 | +2/-3 | 23 | +2/-3 |
| n78 |  |  | 295 | +2/-3 | 26 | +2/-3 | 23 | +2/-3 |
| n79 |  |  | 295 | +2/-3 | 26 | +2/-3 | 23 | +2/-3 |
| n80 |  |  |  |  | 26 | +2/-33 | 23 | ±23 |
| n81 |  |  |  |  |  |  | 23 | ±2 |
| n82 |  |  |  |  |  |  | 23 | ±2 |
| n83 |  |  |  |  |  |  | 23 | +2/-2.5 |
| n84 |  |  |  |  | 26 | +2/-3 | 23 | ±2 |
| n85 | 316 | +2/-3 |  |  |  |  | 23 | ±23 |
| n86 |  |  |  |  |  |  | 23 | ±2 |
| n89 |  |  |  |  |  |  | 23 | ±2 |
| n91 |  |  |  |  |  |  | 23 | ±23, 4 |
| n92 |  |  |  |  |  |  | 23 | ±23, 4 |
| n93 |  |  |  |  |  |  | 23 | ±23, 4 |
| n94 |  |  |  |  |  |  | 23 | ±23, 4 |
| n95 |  |  |  |  | 26 | +2/-3 | 23 | ±2 |
| n97 |  |  |  |  | 26 | +2/-3 | 23 | ±2 |
| n98 |  |  |  |  | 26 | +2/-3 | 23 | ±2 |
| n99 |  |  |  |  |  |  | 23 | +2/-33 |
| n100 | 316 |  |  |  |  |  | 23 | ±2 |
| n101 | 316 |  |  |  |  |  | 23 | ±2 |
| n104 |  |  |  |  | 26 | +2/-3 | 23 | +2/-3 |
| n105 |  |  |  |  |  |  | 23 | +2/-2.5 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: Power class 3 is default power class unless otherwise stated NOTE 3: Refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. NOTE 4: The maximum output power requirement is relaxed by reducing the lower tolerance limit by 0.3 dB NOTE 5: Achieved via dual Tx NOTE 6: Generally, PC1 UE is not targeted for smartphone form factor. The UE power class 1 requirements for Band n14 are applicable for public safety scenario only. |  |  |  |  |  |  |  |  |

If a UE supports a different power class than the default UE power class for the band and the supported power class enables the higher maximum output power than that of the default power class:

-	if the field of UE capability maxUplinkDutyCycle-PC2-FR1 is absent and the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than 50% (The exact evaluation period is no less than one radio frame); or

-	if the field of UE capability maxUplinkDutyCycle-PC2-FR1 is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-PC2-FR1 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame); or

-	if the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is not absent and half the percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-PC1dot5-MPE-FR1 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame); or

-	if the IE P-Max as defined in TS 38.331 [7] is provided and set to the maximum output power of the default power class or lower;

-	shall apply all requirements for the default power class to the supported power class and set the configured transmitted power as specified in clause 6.2.4;

-	else if the UE does not support a power class with higher maximum output power than PC2; or

-	if the field of UE capability maxUplinkDutyCycle-PC2-FR1 is absent and the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than 25% (The exact evaluation period is no less than one radio frame); or

-	if the field of UE capability maxUplinkDutyCycle-PC2-FR1 is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than 0.5*maxUplinkDutyCycle-PC2-FR1 (The exact evaluation period is no less than one radio frame); or

-	if the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-PC1dot5-MPE-FR1 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame); or

-	if the IE P-Max as defined in TS 38.331 [7] is provided and set to the maximum output power of the power class 2 or lower;

-	shall apply all requirements for power class 2 to the supported power class and set the configured transmitted power as specified in clause 6.2.4;

-	else shall apply all requirements for the supported power class and set the configured transmitted power as specified in clause 6.2.4.

A UE supporting [ue-SpecCBW-WithoutChRasterRestriction] for an NR operating band with a 100 kHz channel raster shall meet the minimum requirements for the UE maximum output power as indicated by ue-PowerClass for the operating band for supported UE-specific channel bandwidths configured by ServingCellConfig within the channel bandwidth corresponding to the carrierBandwidth indicated in uplinkConfigCommon or supplementaryUplink.

### 6.2.1I	Void

< text omitted >

## 6.5	Output RF spectrum emissions

### 6.5.0	General

A UE supporting [ue-SpecCBW-WithoutChRasterRestriction] shall meet the following additional requirements for supported UE-specific channel bandwidths configured by ServingCellConfig within the channel bandwidth corresponding to the carrierBandwidth indicated in uplinkConfigCommon or supplementaryUplink of an NR carrier within NR operating bands with 100 kHz channel raster:

-	the minimum requirements in sub-clause 6.5.1 for a BWP configured within the configured UE-specific channel bandwidth

-	the minimum requirements in sub-clauses 6.5.2 and 6.5.3 outside the channel bandwidth corresponding to the carrierBandwidth indicated in uplinkConfigCommon or supplementaryUplink of the NR carrier for a BWP configured within the UE-specific channel bandwidth.

### 6.5.1	Occupied bandwidth

< text omitted >

### 7.3.1	General

The reference sensitivity power level REFSENS is the minimum mean power applied to each one of the UE antenna ports for all UE categories, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel.

In later clauses of Clause 7 where the value of REFSENS is used as a reference to set the corresponding requirement:

in all bands, the UE shall be verified against those requirements by applying the REFSENS value in Table 7.3.2-1a,  Table 7.3.2-1b and Table 7.3.2-1c or Table 7.3.2-1d with 2 Rx antenna ports tested;

for bands where the UE is required to be equipped with 4 Rx antenna ports, the UE shall additionally be verified against those requirements by applying the resulting REFSENS value derived from the requirement in Table 7.3.2-2 with 4 Rx antenna ports tested.

A UE indicating [ue-SpecCBW-WithoutChRasterRestriction] for an NR operating band shall meet the following additional requirement for supported UE-specific channel bandwidths configured by ServingCellConfig within the channel bandwidth corresponding to the carrierBandwidth indicated in downlinkConfigCommon of the NR carrier within this band:

-	the minimum requirement in sub-clause 7.3.2 for the UE-specific channel bandwidth.

### 7.3.2	Reference sensitivity power level

< end of changes >