3GPP TSG-RAN WG4 Meeting #108bis		R4- 2315789

Xiamen, China, 9th – 13th October, 2023

Agenda item:	5.9.3.2

Source:	Intel Corporation

Title:	Remaining issues from inter-RAT measurement without gap

Document for:	Discussion

# Introduction

Up to this meeting, all agreed using scenarios for inter-RAT NR/LTE measurements without gap can summarized as:

- Inter-RAT NR measurements without gap in Rel-18 includes the scenario below.

  - Case a-1: UE performing the measurements without gap in NR carriers as there is vacant RF chains for UE measurements

- Inter-RAT LTE measurements without gap in Rel-18 includes the two scenarios below.

  - Case b-1: UE performing the measurements without gap in LTE carriers as there is vacant RF chains for UE measurements

  - Case b-2: LTE CRS are fully contained within UE’s active BWP

In this paper we provide our views on how to specify the requirements for each of the cases.

# Discussion

## RRM requirements

General

In the previous meetings, we have agreed on the list of RRM impacts from the cases included in the scope as agreed further before.

< Agreement >

| Using scenarios | Capability indications | New RRM requirements needed | Notes |
| --- | --- | --- | --- |
| Case a-1: Inter-RAT NR wo gap because of the vacant RF chain available | “gap” | No | The existing requirements in TS36.133 8.1.2.4.21&22 can be applied |
|  | “no gap but interruption allowed” | Yes | To be defined in TS36.133 |
|  | “no gap no interruption” | Yes | To be defined in TS36.133 |
| Case b-1: Inter-RAT LTE wo gap because of the vacant RF chain available | “gap” | No | The existing requirements in TS38.133 9.4.2&9.4.3 can be applied |
|  | “ncsg” (with interruption) | No | the existing requirements in TS38.133 9.4.2&9.4.3 can be reused. |
|  | “nogap-noncsg” (without interruption) | Yes | To be defined in TS38.133 |
| Case b-2: Inter-RAT LTE wo gap because the measurement reference signal can be contained within UE’s active BWP | “gap” | No | The existing requirements in TS38.133 9.4.2&9.4.3 can be applied |
|  | “no gap but interruption allowed” | No | Do not consider |
|  | “no gap” (without interruption) | Yes | To be defined in TS38.133 |

## Case a-1 requirements

Regarding the requirements of inter-RAT NR measurements without gap (case a-1), the frameworks of LTE TDD and FDD clauses can be reused while the requirements should be based on NR measurement periods. For example,

- The requirements shall be differentiated for TDD and FDD

- And the requirements can be

  - Tidentify_irat_without_index = (TPSS/SSS_sync_irat + TSSB_measurement_period_irat) ms

  - Tidentify_irat_with_index = (TPSS/SSS_sync_irat + TSSB_measurement_period_irat + TSSB_time_index_irat) ms

- Plus, we remove “MGRP” from the formulation of the measurement cycle.

Therefore, we propose that:

Proposal 1: For the inter-RAT NR measurements without gap (case a-1), the requirements can be defined as:

- The requirements shall be differentiated for TDD and FDD.

- And the requirements can be

  - Tidentify_irat_without_index = (TPSS/SSS_sync_irat + TSSB_measurement_period_irat) ms

  - Tidentify_irat_with_index = (TPSS/SSS_sync_irat + TSSB_measurement_period_irat + TSSB_time_index_irat) ms

- For the specific the requirement, e.g.

Table x-1: Time period for PSS/SSS detection (Frequency range FR1)

| Condition NOTE1,2 | TPSS/SSS_sync_irat |
| --- | --- |
| No DRX | Max(600ms, 8 SMTC period) Nfreq |
| DRX cycle ≤ 320ms | Max(600ms, Ceil(81.5) Max(SMTC period, DRX cycle)) Nfreq |
| DRX cycle > 320ms | 8 DRX cycle Nfreq |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in section 5. |  |

Table x-5: Measurement period for inter-RAT measurements (Frequency range FR1)

| Condition NOTE1,2 | TSSB_measurement_period_irat |
| --- | --- |
| No DRX | Max(200ms, 8 SMTC period) Nfreq |
| DRX cycle ≤ 320ms | Max(200ms, Ceil(8 1.5) Max(SMTC period, DRX cycle)) Nfreq |
| DRX cycle > 320ms | 8 DRX cycle Nfreq |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in section 5. |  |

## Case b requirements

Effective measurement window

Effective measurement window is agreed by the group and it is used when the UE indicates supports for measurement on inter-RAT LTE target frequency without gap. The UE is required to measure on the LTE CRS within the time duration of effective measurement windows so that the network is crystal clear about when exactly the UE measures on the target.

When UE indicates to the network about its supporting inter-RAT LTE measurement without gap, either it is with interruption or without interruption, the network configures effective measurement window configurations to the UE instead of measurement gap configurations. The configuration of effective measurement window includes at least the periodicity of the window occasions, the time duration of each window per period and the time offset for each window within the periodicity.

Having this defined, the UE will follow the configurations of effective measurement windows to measure the target LTE CRS only within the configured windows. If the ‘with interruption’ UE capability is indicated, there is interruptions caused within UE when the UE has to retune towards the target LTE frequency and retune back, which happen either at the beginning of the effective measurement window or at the end or both. In the requirements for inter-RAT LTE measurements without gaps, interruptions are allowed both at the beginning and at the end of each effective measurement window for the UE capable of ‘with interruption’ gapless measurements. However, interruption is not allowed for the UE-s indicating support of ‘without interruption’ gapless measurements.

During the effective window time durations, the network schedules the UE in the serving cell as usual. With this knowledge of UE measuring only within the window durations, the network is able to correctly arrange the measurements for the UE so that there is no collision between effective windows and SMTC/SSB/measurement gap. This is crucial and key benefit brought by effective measurement windows. The below figure depicts a typical use case for measurement arrangements by configurations of effective measurement window.

Figure 1 Measurement arrangement by configurations of effective measuremetn window

As shown in the above figure, by configuring the time offset of the effective measurement windows, measurements on the inter-RAT LTE CRS are not overlapping with any of the NR measurements. In this way system performance is optimized. No impact is observed to affect the existing requirements of any kind for NR measurements. There is only one effective measurement window configuration at a time for the UE so all the target LTE frequency layers share the occasions carried by the windows.

To achieve this optimal performance, the periodicity of the configured effective measurement windows must be equal to or larger than the periodicity of any one among SSB, SMTC and measurement gap configurations of any frequency layer configured as a measurement target to the UE. Meanwhile the time offset of the effective measurement window configuration must be different with that of any SMTC or measurement gap configured to the UE.

Measurement period requirements for case b-1

The agreement from the previous meetings about case b-1 is that we will reuse NeedForNCSG-InfoEUTRA-r17 to support Case b-1 and define new requirements for case when UE reports “nogap-noncsg” in NeedForNCSG-InfoEUTRA-r17 for indicating no-gap without interruption. For those UE indicating ‘ncsg’ and supporting only ‘with interruption’ gapless measurements, the existing requirements in TS38.133 9.4.2&9.4.3 can be applied.

For UE supporting inter-RAT EUTRA measurement without gap and without interruption and indicating NeedForGapNCSG-InfoEUTRA-r18 ‘nogap-noncsg’,

- An inter-RAT measurement is defined as measurement without gap if

  - the UE indicates ‘nogap-noncsg’ via NeedForGapNCSG-InfoEUTRA-r18 for the inter-RAT measurement

Parameter TInterEMW used in inter-RAT requirements without gap when effective measurement window is configured is specified according to the available measurement window total duration of each configuration of effective measurement window.

Table 1: Minimum available time for inter-RAT measurements when effective measurement window is configured

| EMW Pattern Id | EMW Length (EMWL, ms) | EMW Repetition Period (EMWRP, ms) | Minimum available time for inter-RAT measurements during 480 ms period (TinterEMW, ms) |
| --- | --- | --- | --- |
| 0 | 6 | 40 | 60 |
| 1 | 6 | 80 | 30 |
| 2 | 3 | 40 | 24Note 1 |
| 3 | 3 | 80 | 12Note 1 |
| 4 | 4 | 40 | 36 Note 1 |
| 5 | 4 | 80 | 18Note 1 |
| NOTE 1: When determining UE requirements using TinterEMW for pattern IDs 2, 3, 4, 5, TinterEMW = 60 for pattern IDs 2, 4, and TinterEMW = 30 for pattern IDs 3 and 5 shall be used. |  |  |  |

When the UE is configured with effective measurement window to identify and measure inter-RAT cells without gap and without interruption and the UE is capable of ‘nogap-noncsg’ in NeedForGapNCSG-infoEUTRA, the UE shall be able to identify a new detectable FDD or TDD cell within TIdentify, E-UTRAN FDD or TDD according to the following expression:

TIdentify, E-UTRAN FDD or TDD = 480 x TinterEMW/480 x Ceil(KEMW)

When no DRX is used and when the UE indicates ‘nogap-noncsg’ in NeedForGapNCSG-infoEUTRA capability of conducting such measurements without gaps and without any interruption, and when effective measurement windows are configured for inter-RAT EUTRA measurements without gap, the UE physical layer shall be capable of reporting RSRP, RSRQ, and RS-SINR measurements to higher layers with measurement period TMeasure, E-UTRAN FDD or TMeasure, E-UTRAN TDD given by the below table.

Table 2: Measurement period and measurement bandwidth when no DRX is configured

| Configuration | Physical Layer Measurement period: TMeasure, E-UTRAN FDD(or TDD) [ms] | Measurement bandwidth [RB] |
| --- | --- | --- |
| 0 | Max(480, TEMW) x Ceil(KEMW) | 6 |
| 1 (Note 1) | Max(240, TEMW) x Ceil(KEMW) | 50 |
| NOTE 1: This configuration is optional. NOTE 2: KEMW equals the number of total LTE frequency layer configured where the UE measures on them without gap and without interruption. |  |  |

Where,

TEMW is the configured periodicity of effective measurement window, and

KEMW equals the number of total LTE frequency layers configured where the UE measures on them without gap and without interruption.

When DRX is in use and when the UE indicates ‘nogap-noncsg’ in NeedForNCSG-infoEUTRA capability of conducting such measurements without gaps and without any interruption, and when effective measurement windows are configured for inter-RAT EUTRA measurements without gap, the UE physical layer shall be capable of the UE shall be able to identify a new detectable E-UTRAN FDD or TDD cell within TIdentify, E-UTRAN FDD or TDD specified in the below table.

Table 3: Requirement to identify a newly detectable E-UTRAN FDD or TDD cell when DRX is configured

| DRX cycle length (s) | TIdentify, E-UTRAN FDD or TDD (s) (DRX cycles) |  |
| --- | --- | --- |
|  | EMW period = 40 ms | EMW period = 80 ms |
| ≤0.16 | Non-DRX requirements apply | Non-DRX requirements apply |
| 0.256 | 5.12* Ceil(KEMW) (20* Ceil(KEMW)) | 7.68* Ceil(KEMW) (30* Ceil(KEMW)) |
| 0.32 | 6.4* Ceil(KEMW) (20* Ceil(KEMW)) | 7.68* Ceil(KEMW) (24* Ceil(KEMW)) |
| 0.32< DRX-cycle ≤10.24 | Note1 (20* Ceil(KEMW)) | Note1 (20* Ceil(KEMW)) |
| NOTE 1: The time depends on the DRX cycle length. |  |  |

When DRX is in use and when the UE indicates ‘nogap-noncsg’ in NeedForGapNCSG-infoEUTRA capability of conducting such measurements without gaps and without any interruption, and when effective measurement windows are configured for inter-RAT EUTRA measurements without gap, the UE physical layer shall be capable of reporting RSRP, RSRQ, and RS-SINR measurements to higher layers with measurement period TMeasure, E-UTRAN FDD or TMeasure, E-UTRAN TDD given by the below table.

Table 4: Requirement to measure E-UTRAN FDD or TDD cells when DRX is configured

| DRX cycle length (s) | Tmeasure, E-UTRAN FDD or TDD (s) (DRX cycles) |
| --- | --- |
| ≤0.08 | Non-DRX requirements apply |
| 0.08< DRX-cycle ≤10.24 | Note1 (5* Ceil(KEMW)) |
| NOTE 1: The time depends on the DRX cycle length. |  |

Where,

KEMW equals the number of total LTE frequency layers configured where the UE measures on them without gap and without interruption.

The UE does not apply any requirement when the configured effective measurement window periodicity is smaller than specified or it is smaller than any of the SSB/SMTC/measurement gap periodicities configured. In such cases the performance is not guaranteed since collision among measurements using EMW and other measurements are inevitable.

# Conclusion

Proposal 1: For the inter-RAT NR measurements without gap (case a-1), the requirements can be defined as:

- The requirements shall be differentiated for TDD and FDD.

- And the requirements can be

  - Tidentify_irat_without_index = (TPSS/SSS_sync_irat + TSSB_measurement_period_irat) ms

  - Tidentify_irat_with_index = (TPSS/SSS_sync_irat + TSSB_measurement_period_irat + TSSB_time_index_irat) ms

- For the specific the requirement, e.g.

Table x-1: Time period for PSS/SSS detection (Frequency range FR1)

| Condition NOTE1,2 | TPSS/SSS_sync_irat |
| --- | --- |
| No DRX | Max(600ms, 8 SMTC period) Nfreq |
| DRX cycle ≤ 320ms | Max(600ms, Ceil(81.5) Max(SMTC period, DRX cycle)) Nfreq |
| DRX cycle > 320ms | 8 DRX cycle Nfreq |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in section 5. |  |

Table x-5: Measurement period for inter-RAT measurements (Frequency range FR1)

| Condition NOTE1,2 | TSSB_measurement_period_irat |
| --- | --- |
| No DRX | Max(200ms, 8 SMTC period) Nfreq |
| DRX cycle ≤ 320ms | Max(200ms, Ceil(8 1.5) Max(SMTC period, DRX cycle)) Nfreq |
| DRX cycle > 320ms | 8 DRX cycle Nfreq |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in section 5. |  |

Proposal 2: For case b-1, minimum available time for inter-RAT measurements TinterEMW is specified in the table below.

Table 1: Minimum available time for inter-RAT measurements when effective measurement window is configured

| EMW Pattern Id | EMW Length (EMWL, ms) | EMW Repetition Period (EMWRP, ms) | Minimum available time for inter-RAT measurements during 480 ms period (TinterEMW, ms) |
| --- | --- | --- | --- |
| 0 | 6 | 40 | 60 |
| 1 | 6 | 80 | 30 |
| 2 | 3 | 40 | 24Note 1 |
| 3 | 3 | 80 | 12Note 1 |
| 4 | 4 | 40 | 36 Note 1 |
| 5 | 4 | 80 | 18Note 1 |
| NOTE 1: When determining UE requirements using TinterEMW for pattern IDs 2, 3, 4, 5, TinterEMW = 60 for pattern IDs 2, 4, and TinterEMW = 30 for pattern IDs 3 and 5 shall be used. |  |  |  |

Proposal 3: For case b-1, measurement period and measurement bandwidth when no DRX is configured is specified in the below table.

Table 2: Measurement period and measurement bandwidth when no DRX is configured

| Configuration | Physical Layer Measurement period: TMeasure, E-UTRAN FDD(or TDD) [ms] | Measurement bandwidth [RB] |
| --- | --- | --- |
| 0 | Max(480, TEMW) x Ceil(KEMW) | 6 |
| 1 (Note 1) | Max(240, TEMW) x Ceil(KEMW) | 50 |
| NOTE 1: This configuration is optional. NOTE 2: KEMW equals the number of total LTE frequency layer configured where the UE measures on them without gap and without interruption. |  |  |

Proposal 4: For case b-1, requirement to identify a newly detectable E-UTRAN FDD or TDD cell when DRX is configured is specified in the below table.

Table 3: Requirement to identify a newly detectable E-UTRAN FDD or TDD cell when DRX is configured

| DRX cycle length (s) | TIdentify, E-UTRAN FDD or TDD (s) (DRX cycles) |  |
| --- | --- | --- |
|  | EMW period = 40 ms | EMW period = 80 ms |
| ≤0.16 | Non-DRX requirements apply | Non-DRX requirements apply |
| 0.256 | 5.12* Ceil(KEMW) (20* Ceil(KEMW)) | 7.68* Ceil(KEMW) (30* Ceil(KEMW)) |
| 0.32 | 6.4* Ceil(KEMW) (20* Ceil(KEMW)) | 7.68* Ceil(KEMW) (24* Ceil(KEMW)) |
| 0.32< DRX-cycle ≤10.24 | Note1 (20* Ceil(KEMW)) | Note1 (20* Ceil(KEMW)) |
| NOTE 1: The time depends on the DRX cycle length. |  |  |

Proposal 5: For case b-1, requirement to measure E-UTRAN FDD or TDD cells when DRX is configured is specified in the below table.

Table 4: Requirement to measure E-UTRAN FDD or TDD cells when DRX is configured

| DRX cycle length (s) | Tmeasure, E-UTRAN FDD or TDD (s) (DRX cycles) |
| --- | --- |
| ≤0.08 | Non-DRX requirements apply |
| 0.08< DRX-cycle ≤10.24 | Note1 (5* Ceil(KEMW)) |
| NOTE 1: The time depends on the DRX cycle length. |  |

# References

- R4-2306331 WF on NR_MG_enh2 Part 2, Intel Corporation