3GPP TSG-RAN4 Meeting #108-bis	R4-2315819

# Xiamen, China, October 9 – October 13, 2023

Agenda item:			5.5.3.2

Source:	vivo

Title:	Discussion on Rx requirements for band combinations with 3Tx

Document for:	Discussion

- Introduction

In RAN4 #108 meeting, some agreements about general principles for MSD for 3Tx inter-band UL CA/EN-DC have been reached in[1], captured as follows. Based on these principles, the remaining CA combinations of PC2 and PC1.5 are analyzed in this contribution. And the related band combinations needed MSD studies are as follows:

<Way forward/Agreement>: - 23+27.8 is used as the power configuration in the 3Tx PC1.5 IMD calculation where band A is 23dBm and band B is 24.8dBm+24.8dBm with 2Tx. - Confirm that the conclusion of “use same MSD for the band combination with either 1Tx or 2Tx in TDD aggressor band UL with same power class for harmonic mixing and cross-band leakage” is also applicable to 3Tx with PC1.5 band combination. - For harmonic, harmonic mixing and cross band isolation. - Only add a text in subclause “7.3A.2.3 Reference sensitivity power level for Inter-band CA” to clarify that “the same requirements for REFSENS exceptions apply for UL aggressor configured with either 1Tx or 2Tx for UL MIMO or Tx diversity operation.

<Band combinations needs MSD studies>: PC2: Band comb; 3Tx MSD. CA_n26A-n78A; - FFS: 2Tx PC2 IMD4 MSD was not defined in spec - Keep as not defined: 2Tx PC2 harmonic mixing 4th order MSD. DC_40A_n78A; - FFS: 2Tx PC2 harmonic mixing 3rd order MSD was not defined in spec - FFS: 2Tx PC2 cross band leakage MSD was not defined in spec. PC1.5: Band combination; MSD status. CA_n71A-n41A; - Reuse: 2Tx PC3 MSD for n71 harmonic 4th order interfere n41 is defined in the spec, and can be reused to 3Tx - Keep as not defined: PC2 2Tx harmonic mixing 4th order MSD - FFS: 3T PC1.5 IMD4. Band combination; MSD status. CA_n25A -n77A (CA_n2A -n77A); - n25 UL harmonic 2 with PC3 - FFS: 3T PC1.5 harmonic mixing 2nd order - FFS: 3T PC1.5 IMD2/4 - FFS: 3T PC1.5 cross band isolation. CA_n5A -n77A; - n5 UL harmonic 4/5 with PC3 - Keep as not defined: 3T PC1.5 harmonic mixing 4th order - FFS: 3T PC1.5 IMD4.

- MSD analysis for PC2 band combination

## 2.1. CA_n26A-n78A

### 2.1.1. IMD

In current specification, there is no IMD MSD requirement for 2Tx PC2 CA_n26A-n78A. So, there are two kinds of MSD calculation schemes to be considered. Scheme 1: 2Tx requirement could be defined firstly and then reused to 3Tx architecture. Scheme 2: Directly calculate the IMD MSD for 3Tx of PC2. Below is the comparison of the two schemes. Scheme 1 uses architecture 1, scheme 2 uses architecture 2, and the calculation parameters are shown in table 1.

Figure 1. IMD4 product of CA_n26A-n78A

Figure2. architecture 1, 2Tx PC2 IMD4 for CA_n26A-n78A

Figure3. architecture 2, 2Tx PC2 IMD4 for CA_n26A-n78A

After calculation, it can be observed that there is almost no difference between 2TX and 3TX on IMD4 MSD. The MSD for IMD4 is 24.6dB, considering some margin the MSD requirement could be defined at 25dB.

Table 1. Parameters of IMD4 MSD evaluation

| Antenna isolation | 10 dB |
| --- | --- |
| Front end loss | 4 dB |
| Diplexer isolation | 15 dB |
| PCB isolation | 65 dB |
| n26 PA forward IP4 | 28 |
| n26 PA reverse IP4 | 33 |
| n78 PA forward IP4 | 28 |
| n78 PA reverse IP4 | 33 |
| n26 duplexer Tx rejection at n78 | 30 dB |
| n26 duplexer Tx-Rx isolation | 50 dB |
| n78 filter rejection at n26 Tx | 60 dB |
| n78 filter rejection at n26 Rx | 60 dB |
| REFSENS of n26 | -97.5 dBm/5MHz |

Proposal 1: 3Tx PC2 IMD4 MSD requirement for CA_n26A-n28A could be defined as the table below.

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n26-n78 | n26 | 836.5 | 5 | 25 | 881.5 | 25 | FDD | IMD4 |
|  | n78 | 3391 | 10 | 50 | 3391 | N/A | TDD | N/A |

## 2.2. DC_40A_n78A

### 2.2.1. Cross band leakage

The cross-band leakage is from UL of n78, and it will interfere DL of b40. The analysis of 2TX cross band leakage for PC2 uses the shared antenna architecture, as shown in figure 4. And the related calculation parameters are stated in table 2. 3TX MSD could reuse the 2TX requirement.

Figure 4. 2Tx PC2 Cross band leakage for DC_40A_n78A with shared antenna

Table 2. Parameters of cross band leakage MSD evaluation

| Antenna isolation | 10 dB |
| --- | --- |
| Diplexer isolation | 10 dB |
| Front end loss | 4 dB |
| PCB isolation | 60 dB |
| n78 PC3 Tx noise @ b40 | -125 dBm/Hz |
| b40 DL CBW | 5 MHz |
| n78 filter attenuation at b40 | 25 dB |
| REFSENS of b40 | -100 dBm/5MHz |

For 5MHz, the MSD for cross band leakage is 3.4 dB, considering some margin the MSD requirement could be defined at 5dB.

Proposal 2: 3Tx PC2 cross band leakage MSD requirement for DC_40A_n78A could be defined as the table below.

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n78 | 401 | 5 | 5 | 5 | 5 |  |  |  |  |  |  |  |  |  |
| NOTE 1: Applicable only when harmonic mixing MSD for this combination is not applied. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

- MSD analysis for PC1.5 band combination

In the last meeting it was agreed that, using same MSD for the band combination with either 1Tx or 2Tx in TDD aggressor band UL with same power class for harmonic mixing and cross-band leakage” is also applicable to 3Tx with PC1.5 band combination. So, the 3TX PC1.5 MSD requirement for 2nd order harmonic mixing and cross band isolation for band combinations of CA_n25A -n77A(CA_n2A -n77A) can reuse the current 2TX requirements. And the other IMD MSD requirements need to be calculated.

## 3.1. CA_n71A-n41A

### 3.1.1. IMD

The architecture used for IMD calculation for CA_n71A-n41A is shown in figure 6, and the calculation parameters are as table 3 stated. IMD4 product will interfere DL of n71, 23+27.8 is used as the power configuration in the 3Tx PC1.5 IMD calculation where band n71 is 23dBm and band n41 is 24.8dBm+24.8dBm with 2Tx.

Figure 5. IMD4 product of CA_n71A-n41A

Figure6 . 3Tx PC1.5 IMD4 MSD

Table 3. Parameters of IMD4 MSD evaluation

| Antenna isolation | 10 dB |
| --- | --- |
| Front end loss | 4 dB |
| Diplexer isolation | 15 dB |
| PCB isolation | 65 dB |
| n71 PA forward IP4 | 28 |
| n71 PA reverse IP4 | 33 |
| n41 PA forward IP4 | 28 |
| n41 PA reverse IP4 | 33 |
| n71 duplexer Tx rejection at n41 | 50 dB |
| n71 duplexer Tx-Rx isolation | 50 dB |
| n41 filter rejection at n71 Tx | 45 dB |
| n41 filter rejection at n71 Rx | 45 dB |
| REFSENS of n71 | -97.2 dBm/5MHz |

The calculation result of MSD for IMD4 is 27.3 dB, considering some margin the MSD requirement could be defined at 28 dB.

Proposal 3: 3Tx PC1.5 IMD4 MSD requirement for CA_n71A-n41A could be defined as the table below.

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n71A-n41A | n41 | 2614 | 5 | 25 | 2614 | N/A | TDD | N/A |
|  | n71 | 665 | 5 | 25 | 619 | 28 | FDD | IMD4 |

## 3.2. CA_n25A -n77A(CA_n2A -n77A)

In the current specification, 2TX PC1.5 harmonic mixing and cross band interference MSD requirements for CA_n25A-n77A and CA_n2A-n77A have been developed. So 3TX architecture can reuse them. Related analysis is stated below.

### 3.2.1. Harmonic mixing

3TX PC1.5 harmonic mixing MSD for CA_n25A-n77A and CA_n2A-n77A could reuse the requirements of Table 7.3A.4-4b in TS 38.101-1.

Proposal 4: 3Tx PC1.5 harmonic mixing MSD requirement for CA_n25A-n77A and CA_n2A-n77A could be defined as the table below.

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n77 | n2 | 10 | 15 | 25 (RBstart=0) | 5 | 11.8 | NOTE 4 | UL1/DL2 |
| n77 | n2 | 20 | 15 | 100 (RBstart=0) | 20 | 9.2 | NOTE 4 | UL1/DL2 |
| n77 | n25 | 10 | 15 | 25 (RBstart=0) | 5 | 11.9 | NOTE 4 | UL1/DL2 |
| n77 | n25 | 20 | 15 | 100 (RBstart=0) | 40 | 3.3 | NOTE 4 | UL1/DL2 |

### 3.2.2. Cross band interference

3TX PC1.5 cross band interference MSD for CA_n25A-n77A and CA_n2A-n77A could reuse the requirements of Table 7.3A.6-1b in TS 38.101-1.

Proposal 5: 3Tx PC1.5 cross band interference MSD requirement for CA_n25A-n77A and CA_n2A-n77A could be defined as the table below.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n77 | n2 | 3305 | 100 | 30 | 270 (RBstart=0) | 1987.5 | 5 | 1.8 | >ACLR2 |
| n77 | n25 | 3305 | 100 | 30 | 270 (RBstart=0) | 1992.5 | 5 | 1.8 | >ACLR2 |

### 3.2.3. IMD

The architecture used for IMD calculation for CA_n25A-n77A is shown in figure 8, and the calculation parameters are shown in table 4. IMD2 and IMD4 products will interfere DL of n25, 23+27.8 is used as the power configuration in the 3Tx PC1.5 IMD calculation where band n25 is 23dBm and band n77 is 24.8dBm+24.8dBm with 2Tx.

Figure 7. IMD product of CA_n25A-n77A

Figure 8. 3Tx PC1.5 IMD MSD

Table 4. Parameters of IMD4 MSD evaluation

| Antenna isolation | 10 dB |
| --- | --- |
| Front end loss | 4 dB |
| Diplexer isolation | 15 dB |
| PCB isolation | 65 dB |
| n25 PA forward IP2 | 27 |
| n25 PA reverse IP2 | 38 |
| n77 PA forward IP2 | 27 |
| n77 PA reverse IP2 | 38 |
| n25 PA forward IP4 | 28 |
| n25 PA reverse IP4 | 33 |
| n77 PA forward IP4 | 28 |
| n77 PA reverse IP4 | 33 |
| n25 duplexer Tx rejection at n77 | 28 dB |
| n25 duplexer Tx-Rx isolation | 50 dB |
| n77 filter rejection at b25 Tx | 40 dB |
| n77 filter rejection at n25 Rx | 40 dB |
| REFSENS of n25 | -96.5 dBm/5MHz |

The calculation result of MSD for IMD2 is 35.2 dB, considering some margin the MSD requirement could be defined at 36 dB. The calculation result of MSD for IMD4 is 26.6 dB, and the MSD requirement could be defined at 27 dB.

Proposal 6: 3Tx PC1.5 IMD4 MSD requirement for CA_n25A -n77A could be defined as the table below.

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n25-n77 | n25 | 1855 | 5 | 25 | 1935 | 36 | FDD | IMD2 |
|  | n77 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |
|  | n25 | 1900 | 5 | 25 | 1980 | 27 | FDD | IMD4 |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | TDD | N/A |

## 3.3. CA_n5A -n77A

### 3.3.1. IMD

The architecture used for IMD calculation for CA_n5A-n77A is shown in figure 10, and the calculation parameters are shown in table 5. IMD4 product will interfere DL of n5, 23+27.8 is used as the power configuration in the 3Tx PC1.5 IMD calculation where band n25 is 23dBm and band n77 is 24.8dBm+24.8dBm with 2Tx.

Figure9. IMD4 product of CA_n5A-n77A

Figure10. 3Tx PC1.5 IMD MSD

Table 5. Parameters of IMD4 MSD evaluation

| Antenna isolation | 10 dB |
| --- | --- |
| Front end loss | 4 dB |
| Diplexer isolation | 15 dB |
| PCB isolation | 65 dB |
| n5 duplexer Tx rejection at n77 | 35 dB |
| n5 duplexer Tx-Rx isolation | 45 dB |
| n77 filter rejection at n5 Tx | 35 dB |
| n77 filter rejection at n5 Rx | 35 dB |
| n5 PA forward IP4 | 28 |
| n5 PA reverse IP4 | 33 |
| n77 PA forward IP4 | 28 |
| n77 PA reverse IP4 | 33 |
| REFSENS of n5 | -98 dBm/5MHz |

The calculation result of MSD for IMD4 is 28.5 dB, considering some margin the MSD requirement could be defined at 29 dB.

Proposal 7: 3Tx PC1.5 IMD4 MSD requirement for CA_n5A -n77A could be defined as the table below.

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n5-n77 | 5 | 844 | 5 | 25 | 889 | 29 | FDD | IMD4 |
|  | n77 | 3421 | 10 | 50 | 3421 | N/A | TDD | N/A |

- Conclusion

Proposal 1: 3Tx PC2 IMD4 MSD requirement for CA_n26A-n28A could be defined as the table below.

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n26-n78 | n26 | 836.5 | 5 | 25 | 881.5 | 25 | FDD | IMD4 |
|  | n78 | 3391 | 10 | 50 | 3391 | N/A | TDD | N/A |

Proposal 2: 3Tx PC2 cross band leakage MSD requirement for DC_40A_n78A could be defined as the table below.

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n78 | 401 | 5 | 5 | 5 | 5 |  |  |  |  |  |  |  |  |  |
| NOTE 1: Applicable only when harmonic mixing MSD for this combination is not applied. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Proposal 3: 3Tx PC1.5 IMD4 MSD requirement for CA_n71A-n41A could be defined as the table below.

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n41-n71 | n41 | 2614 | 5 | 25 | 2614 | N/A | TDD | N/A |
|  | n71 | 665 | 5 | 25 | 619 | 28 | FDD | IMD4 |

Proposal 4: 3Tx PC1.5 harmonic mixing MSD requirement for CA_n25A-n77A and CA_n2A-n77A could be defined as the table below.

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n77 | n2 | 10 | 15 | 25 (RBstart=0) | 5 | 11.8 | NOTE 4 | UL1/DL2 |
| n77 | n2 | 20 | 15 | 100 (RBstart=0) | 20 | 9.2 | NOTE 4 | UL1/DL2 |
| n77 | n25 | 10 | 15 | 25 (RBstart=0) | 5 | 11.9 | NOTE 4 | UL1/DL2 |
| n77 | n25 | 20 | 15 | 100 (RBstart=0) | 40 | 3.3 | NOTE 4 | UL1/DL2 |

Proposal 5: 3Tx PC1.5 cross band interference MSD requirement for CA_n25A-n77A and CA_n2A-n77A could be defined as the table below.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n77 | n2 | 3305 | 100 | 30 | 270 (RBstart=0) | 1987.5 | 5 | 1.8 | >ACLR2 |
| n77 | n25 | 3305 | 100 | 30 | 270 (RBstart=0) | 1992.5 | 5 | 1.8 | >ACLR2 |

Proposal 6: 3Tx PC1.5 IMD4 MSD requirement for CA_n25A -n77A could be defined as the table below.

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n25-n77 | n25 | 1855 | 5 | 25 | 1935 | 36 | FDD | IMD2 |
|  | n77 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |
|  | n25 | 1900 | 5 | 25 | 1980 | 27 | FDD | IMD4 |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | TDD | N/A |

Proposal 7: 3Tx PC1.5 IMD4 MSD requirement for CA_n5A -n77A could be defined as the table below.

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n5-n77 | 5 | 844 | 5 | 25 | 889 | 29 | FDD | IMD4 |
|  | n77 | 3421 | 10 | 50 | 3421 | N/A | TDD | N/A |

- References

- R4-2314705 WF on 3Tx and 4Rx requirements, RAN4#108.