# 3GPP TSG-RAN WG4 Meeting #108bis	R4-2315822

# Xiamen, China, October 09 – October 13, 2023

Source: 	vivo

Title: 	Further discussion on CA_n5-n8

Agenda Item:	4.30.1.1

Document for:	Approval

Introduction

In the last meeting, we discussed the reply LS and further clarified the RRC configuration for non-simultaneous UL and DL from different two bands during UL CA and a reply LS [1] was sent to RAN2. We also discussed how to distinguish different UE implementations for CA_n5-n8 and all possible situations were captured in [2]. However, in RAN#101, there was an updated WID [3] for 700/800/900MHz band combinations and only n5 uplink is supported for CA_n5-n8. In this contribution, we further discussed issues for CA_n5-n8.

Discussion

In the updated WID [3], uplink is restricted to band n5 for CA_n5-n8 as shown in Table 1. It is so much easier for the down-selection of different UE implementations for CA_n5-n8.

Table 1: Band combinations studied in this WI

| Configuration | Uplink configuration | Supported operators |
| --- | --- | --- |
| CA_n5-n8 | - | China Telecom, Spark NZ, China Unicom |
| CA_n5-n28 | CA_n5-n28 | Spark NZ |
| CA_n8-n20-n28 | CA_n8-n20, CA_n8-n28, CA_n20-n28 | Vodafone, Telecom Italia, Orange, Deutsche Telekom, Telia Company |
| CA_n5-n105 | CA_n5-n105 | Spark NZ |
| CA_n28-n105 | - | Spark NZ |
| CA_n26-n28 | CA_n26-n28 | Telstra |
| CA_n5-n28-n105 | CA_n5-n105, CA_n5-n28 | Spark NZ |
| CA_n26(2A) | CA_n26(2A) | Telstra |
| Note 1: Uplink is only in n5 for CA_n5-n8. Note 2: CA_n26(2A) is not intended for use in the USA. |  |  |

In the discussion for UE implementations for CA_n5-n8, there were three options from the SI phase:

1)	Full band n5 and n8 RF filters implementation with option 1 and option2:

-	Option 1: Only support 1UL/2DL CA. Single UL in n5

-	Option 2: Support both 1UL/2DL and 2UL/2DL CA. Non-concurrent n5 DL and n8 UL

2)	Dedicated RF filters implementation with partial frequency range

-	Option 3: Support both 1UL/2DL and 2UL/2DL CA. Dedicated filter to allow simultaneous n5 DL and n8 UL

According to the updated information from [3], only Option 1 will be supported in Rel-18 and Option 2/3 will not be considered since they are out of scope of this work item.

Proposal 1：Adopt Option 1(Only support 1UL/2DL CA. Single UL in n5) for CA_n5-n8 in RAN4 according to updated WID.

As for the issue how to differentiate UE implementation for CA_n5-n8, we can go to Situation 1 as [2] suggested:

Situation 1: if Option 1 is supported in Rel-18, then

| Implementation | Applicable RRC configuration | How to distinguish UE implementation |
| --- | --- | --- |
| Full band n5 and n8 RF filters implementation: Option 1: Only support 1UL/2DL CA. Single UL in n5 | (UL: n5, DL: n5+n8) | There is no need to indicate additional UE capability. UE just reports the applicable CA configuration. |

In addition, one note can be added in RAN4 that band n8 can only be configured as DL Scell for band combinations including DL CA_n5-n8.

For the RF requirements for CA_n5-n8, only MSD value for 1U/2D will be defined.

- CA_n5-n8 MSD value for 1U/2D n5UL/n8DL

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n5 | n8 | 844 | 10 | 15 | 25 (RBstart=27) | 951.5 | 5 | 2.8 | >ACLR2 |

In this release, two uplink CA were discussed for CA_n5-n8 and different UE implementations were proposed to avoid n5 DL and n8 UL overlapping since SI phase. We also discussed different ways to differentiate these UE implementations for CA_n5-n8. Although 2UL/2DL was not pursued in this WID, at least all the output of study for 2UL/2DL CA_n5-n8 can be captured in the TR 38.872. It can be used as a reference for future release if operator requests two uplink CA configuration for CA_n5-n8.

Proposal 2: At least all the output of study about 2UL/2DL CA for CA_n5-n8 can be captured in the TR 38.872 for future reference.

Conclusion

This contribution discusses remaining issues for CA_n5-n8. The following proposals are made:

Proposal 1：Adopt Option 1(Only support 1UL/2DL CA. Single UL in n5) for CA_n5-n8 in RAN4 according to updated WID.

Proposal 2: At least all the output of study about 2UL/2DL CA for CA_n5-n8 can be captured in the TR 38.872 for future reference.

- References

[1] R4-2314656, Reply LS on non-simultaneous UL and DL from different two bands during UL CA, CATT, RAN4#108.

[2] R4-2314936, WF on CA_n5-n8, Huawei, RAN4#108.

[3] RP-232680, Revised WID on enhancement for 700/800/900MHz band combinations, CATT, RAN#101.