3GPP TSG-RAN WG4 Meeting #108bis	R4-2315824

# Xiamen, China, October 09 – October 13, 2023

Agenda Item:	5.19.2.4

Source:	vivo

Title: 	TP to TR 38.858 on UE RF impacts for full duplex operation

Document for:	Discussion

- Introduction

In RAN4#107, there was one agreement for UE aspects:

- Agreement:

  - FFS the impact on UE RF requirements pending on the co-existence study conclusion.

    - If no issues identified by co-existence study, existing UE RF requirements can be applied as default assumption for study phase conclusion. Detailed UE RF requirements if any should be discussed during WID phase.

For now, there was no critical issues identified for UE, thus no enhancement was needed. However, no final conclusion on UE RF requirements impacts before the co-existence study conclusion.

We’ve checked there was no proposal for UE RF requirements impact in the co-existence study in the last meeting. Since this meeting will be the last meeting for collecting co-existence simulation results, we suggest to sort it out if there were any proposals on UE RF requirements impact. If still there are no issues, we can conclude that existing UE RF requirements can be applied as default assumption for study phase conclusion.

In the last meeting, there were also two TPs [1][2] endorsed on feasibility of FR1/FR2-1 UE aspects for TR 38.858. In this contribution, we propose the wording for UE RF requirements impact for full duplex operation. The changes for the TP contents are made with revision marks.

- Text Proposal

<<Start of Change for TR 38.858 v1.0.0>>

- 10.2	Impact on UE RF requirements

Editor's note: This section captures the impact on UE RF requirements

In the whole discussion for UE part for full duplex, RAN4 studies UE-UE co-channel inter-subband CLI modeling and adjacent channel CLI modelling. There are in-channel RF requirements/metrics, IBE, subband selectivity and adjacent channel RF requirements/metrics ACLR/ACS discussed for UE Tx/Rx interference modelling, as shown in Table 10.2-1:

Table 10.2-1: UE RF requirements/metrics for interference modelling in full duplex operation

| In-channel RF requirements/metrics | FR1 | FR2-1 |
| --- | --- | --- |
| Tx Model: IBE | Existing UE IBE requirement in TS38.101-1 | Existing UE IBE requirement in TS38.101-1 |
| Rx Model: Subband Selectivity | 33dB for simulation usage purpose | 23 dB for simulation usage purpose |
| Adjacent channel RF requirements/metrics | FR1 | FR2-1 |
| Tx Model: ACLR | 30 dB at max power that improves 1dB/dB with backoff up to a maximum 10 dB of improvement | 24 dB at max power that improves 1dB/dB with backoff up to a maximum 10 dB of improvement |
| Rx Model: ACS | 33 dB under the assumption that the blocker from adjacent channel does not exceed the maximum input level (-25 dBm) | 23 dB under the assumption that the blocker from adjacent channel does not exceed the maximum input level (-25 dBm) |

For UE in-channel RF requirements/metrics, existing UE IBE requirements in TS 38.101-1/2 were assumed for the Tx model; For the Rx model, RAN4 defined subband selectivity for co-existence simulation purpose, the value 33dB for FR1 and 23dB for FR2-1 are derived from FFT selectivity of typical UE receiver. Currently, subband selectivity is not a UE RF requirement and it is only used in the co-existence study.

For UE adjacent channel RF requirements/metrics, the values of ACLR/ACS for FR1/FR2 are based on the existing UE RF requirements. Currently, the ACLR requirement was defined with the assumption of the maximum output power. In order to model typical UE performance, this model was revised considering UE transmits in power less than the maximum power. The ACS values for FR1 and FR2-1 were reused from TS 38.101-1/2.

In RAN4 study, existing UE RF requirements in TS 38.101-1 can be applied as default assumption for study phase conclusion if no issues identified by co-existence study. For now, there is no strong evidence for UE RF enhancement in the co-existence study. Thus, it can be concluded that there are no UE RF requirements impact for UE side in gNB SBFD operation.

<<End of Change>>

References

[1] R4-2313869, TP to TR 38.858 on Feasibility of FR1 UE aspects, MediaTek (Shenzhen) Inc., Ericsson, vivo, Nokia, Nokia, RAN4#108.

[2]R4-2313990, TP to TR 38.858 on Feasibility of FR2-1 UE aspects, Qualcomm, Ericsson, vivo, Nokia, Nokia Shanghai Bell, RAN4#108.