3GPP TSG-RAN WG4 Meeting #108bis	R4-2315871

Xiamen, Oct. 09 – 13, 2023

Source: 	Ericsson

Title:  	RRM performance requirements for high speed train scenario in frequency range 2

Agenda Item:	5.12.3

Document for:	Discussion

# Introduction

In this contribution, we herein present our views and proposals on the necessary test cases for high speed train scenario in FR2.

# Discussion

In principle a similar approach is to include additional requirements for carrier aggregation together with present HST NR FR1 test cases.

Firstly, The agreed core WI big CRs for HST FR1are in [1]. The corresponding added requirements are listed in the below:

- Measurements of inter-frequency NR cells

- One shot large UL timing adjustment with MAC-CE indication

- Maximum Receive Timing Difference under multi-panel simultaneous reception operation.

- TEvaluate_out_SSB and TEvaluate_in_SSB , with new scaling factor in case of bidirectional deployment and multi-panel simultaneous reception

- SCell activation time for the FR2 power class 6 UE with configured HST flag

- MAC-CE based TCI state switch delay with MAC-CE indication

- Intrafrequency measurements without measurement gaps for SCell

- Intrafrequency measurements with measurement gaps for SCell

- (Newly added) Inter-frequency measurement with measurement gaps for SCell

- (Newly added) Inter-frequency measurement without measurement gaps for SCell

- SSB based L1-RSRP Reporting, with new scaling factor in case of bidirectional deployment and multi-panel simultaneous reception

Based on this analysis we propose the following test case list that shall be added:

- Cell reselection to FR2 inter-frequency NR case, for UE supporting [measurementEnhancementCAInterFreqFR2-r18] when configured with [highSpeedMeasFlagFR2-r17] for FR2

- NR PCell FR2 HST active TCI state switch for a known TCI state, with MAC-CE indication (1 or 0)

- Inter-frequency measurement (SA event triggered reporting test without gap under non-DRX), for the UE supporting [measurementEnhancementCAInterFreqFR2-r18] when [highSpeedMeasFlagFR2] is configured

- Inter-frequency measurement without measurement gaps (SA event triggered reporting test with gap under non-DRX),  for the UE supporting [measurementEnhancementCAInterFreqFR2-r18] when [highSpeedMeasFlagFR2] is configured

- SSB based L1-RSRP measurement when DRX is used, for the UE configured bidirectional deployment and multi-panel simultaneous reception.

A newly introduced feature in Rel-18 is multi-panel simultaneous reception. Since only RX beam sweeping number is enhanced in the feature from RRM perspective, we suggest to only test SSB based L1-RSRP measurement with test configurations for multi-panel simultaneous reception. The update in the test configuration is setup of simultaneous downlink transmissions from two AoAs.

Proposal 2: Test SSB based L1-RSRP measurement with test configurations for multi-panel simultaneous reception, which set simultaneous downlink transmissions from two AoAs.

# Conclusion

Proposal 1: the following test case list that shall be added:

- Cell reselection to FR2 inter-frequency NR case, for UE supporting [measurementEnhancementCAInterFreqFR2-r18] when configured with [highSpeedMeasFlagFR2-r17] for FR2

- NR PCell FR2 HST active TCI state switch for a known TCI state, with MAC-CE indication

- Inter-frequency measurement (SA event triggered reporting test without gap under non-DRX), for the UE supporting [measurementEnhancementCAInterFreqFR2-r18] when [highSpeedMeasFlagFR2] is configured

- Inter-frequency measurement without measurement gaps (SA event triggered reporting test with gap under non-DRX),  , for the UE supporting [measurementEnhancementCAInterFreqFR2-r18] when [highSpeedMeasFlagFR2] is configured

- SSB based L1-RSRP measurement when DRX is used, for the UE configured bidirectional deployment and multi-panel simultaneous reception.

Proposal 2: Test SSB based L1-RSRP measurement with test configurations for multi-panel simultaneous reception, which set simultaneous downlink transmissions from two AoAs.

# References

- R4-2313541, Big CR to TS38.133 on enhanced NR support for FR2 HST