3GPP TSG-RAN WG4 Meeting #108bis 	R4-2315899

Xiamen, China, October 09 – October 13, 2023

Agenda Item:	5.7.2.1

Source: 	Ericsson

Title:	On general aspects

Document for:	Discussion

# Introduction

The agreements from RAN4#108 are summarized in [20] and [21] for the 1st and 2nd threads, respectively.

# Discussion

## Different numerology with the simultaneous RS reception

The related agreements/discussion from RAN4#106bis-e are:

Issue 1-4-3a: UE capability of simultaneousRxDataSSB-DiffNumerology-Inter-r16

<Agreement >:

- No need to discuss simultaneousRxDataSSB-DiffNumerology-Inter-r16 in R18 multi-Rx chains WI

Issue 1-4-3: UE capability of simultaneousRxDataSSB-DiffNumerology

<Way forward >:

- Proposals

  - Option 1:

    - The existing simultaneousRxDataSSB-DiffNumerology IE can be re-used in R18 multi-panel WI.

  - Option 2:

    - The existing UE capability simultaneousRxDataSSB-DiffNumerology is feasible if the supporting of concurrent SSB and data are consistent between FR1 and FR2-1. But it should be ultimately determined after the decision of single or multiple UE capabilities are necessary.

  - Option 3:

    - No need to specify new mix-numerology capability for multi-RX chain in FR2 when consider simultaneous SSB and data reception.

  - Option 4:

    - A new UE capability is needed to support simultaneous reception with mixed numerologies.

  - Option 5:

    - The existing UE capability to support mixed numerologies is not directly applicable for multi-RX chain operation with mixed numerologies, hence a new UE capability is needed to support simultaneous reception with mixed numerologies.

The existing relevant UE capability is below, which is, however, defined independently of the existing capability for simultaneous reception of data and RS with same numerology but with different QCL Type-D. Furthermore, the same principle is likely to apply for multi-rx, i.e., the general FR2 multi-rx capability will likely not cover the mixed numerologies case.

| simultaneousRxDataSSB-DiffNumerology Indicates whether the UE supports concurrent intra-frequency measurement on serving cell or neighbouring cell and PDCCH or PDSCH reception from the serving cell with a different numerology as defined in clause 8 and 9 of TS 38.133 [5]. | UE | No | No | Yes |
| --- | --- | --- | --- | --- |

However, two separate capabilities (the existing capability simultaneousRxDataSSB-DiffNumerology and the general FR2 multi-rx capability) do not necessarily imply that the UE will also support the mixed numerologies case under multi-rx operation. Hence, a new capability is needed to define the support for the mixed numerologies under multi-rx.

As per our understanding, in real deployments for FR2, RS and data may be using different numerology (e.g., CSI-RS/SSB using 120 kHz and CSI-RS/data using 60 kHz). Considering that this is most practical deployment, we think a new UE capability may have to be defined.

- Proposal 1 (mixed numerologies): Simultaneous reception with mixed numerologies should be supported.

- Proposal 2 (mixed numerologies): RAN4 agrees that the existing UE capability simultaneousRxDataSSB-DiffNumerology is not sufficient to cover the mixed numerologies support under multi-rx operation.

- Proposal 3 (mixed numerologies): A new UE capability is needed to support simultaneous reception with mixed numerologies.

Assuming new capability is defined, RAN4 may need to update the scheduling restrictions of the existing specification as the scheduling restrictions defined in current specification assumes that the UE can receive from only single beam direction. When UE can receive from different directions, at least in some cases, we may not need to specify scheduling restrictions.

- Proposal 4 (mixed numerologies): For UEs supporting simultaneous reception with mixed numerologies no scheduling restrictions are needed.

## On multi-rx enhancements for L3 measurements

In RAN4#106, it was agreed that simultaneous L3+L1 measurements are not supported in this release for multi-rx:

- Simultaneous L3 measurements and L1 measurements

  - Simultaneous L3 and L1 measurements is not supported in this release for multi-Rx.

  - Note: The agreement has no impact on scheduling restriction discussion for L3 measurements. Feasibility and benefit of scheduling restriction relaxation for L3 measurements is FFS.

However, it is still open whether simultaneous reception of L3+L3 are supported for multi-rx in Rel-18.

- Proposal 5 (L3+L3): Simultaneous reception for two L3 measurements on different panels/from different directions is not supported in R18 multi-Rx chain WI.

## Simultaneous RS reception with different QCL D’s configured together with QCL A/C

We think that QCL type D in combination with QCL type A/C should not be excluded in the requirements for simultaneous RS reception.

- Proposal 6 (QCL D+A/C): Scenarios where QCL type D is configured in combination with QCL type A/C are not precluded in the requirements for multi-rx operation.

## UE behaviour when some applicability condition is violated

RAN4 needs to discuss further the UE behaviour when some applicability condition is violated [20]:

#### Issue 1-3-5: UE behaviour when a condition  becomes violated during a measurement

- FFS if it is to define particular behavior when side conditions for multi-Rx operation are violated

For the conditions which are not static (unlike the UE multi-rx capability), RAN4 needs to also discuss what happens, when at least one of the necessary conditions are not met, particularly if this happens during an on-going measurement with multi-rx.

- Proposal 7 (violated applicability conditions): UE in multi-rx operation may need to stop and restart (in new conditions) L1 measurements or evaluations, when at least the following conditions for multi-rx operation become violated:

  - A completely new dual TCI configuration (e.g., dual TCI swapped to another dual TCI where both active states are new).

- Proposal 8 (violated applicability conditions): UE in multi-rx operation may need to continue L1 measurements or evaluations (using a single panel), but more relaxed requirements will apply for such measurements/evaluations, when any of the following conditions for multi-rx operation become violated:

  - Partially new dual TCI configuration (e.g., one active TCI state remains the same),

  - Different QCL type D condition is not valid, etc.

- Proposal 9 (violated applicability conditions): RAN4 to discuss whether there are scenarios when a UE in multi-rx operation may need to complete L1 measurements or evaluations in multi-rx operation prior to switching to single-rx operation, e.g.:

  - UE becomes configured with CA operation or with some other operation suggesting single-rx operation.

# Summary

The following have been proposed in this contribution.

- Proposal 1 (mixed numerologies): Simultaneous reception with mixed numerologies should be supported.

- Proposal 2 (mixed numerologies): RAN4 agrees that the existing UE capability simultaneousRxDataSSB-DiffNumerology is not sufficient to cover the mixed numerologies support under multi-rx operation.

- Proposal 3 (mixed numerologies): A new UE capability is needed to support simultaneous reception with mixed numerologies.

- Proposal 4 (mixed numerologies): For UEs supporting simultaneous reception with mixed numerologies no scheduling restrictions are needed.

- Proposal 5 (L3+L3): Simultaneous reception for two L3 measurements on different panels/from different directions is not supported in R18 multi-Rx chain WI.

- Proposal 6 (QCL D+A/C): Scenarios where QCL type D is configured in combination with QCL type A/C are not precluded in the requirements for multi-rx operation.

- Proposal 7 (violated applicability conditions): UE in multi-rx operation may need to stop and restart (in new conditions) L1 measurements or evaluations, when at least the following conditions for multi-rx operation become violated:

  - A completely new dual TCI configuration (e.g., dual TCI swapped to another dual TCI where both active states are new).

- Proposal 8 (violated applicability conditions): UE in multi-rx operation may need to continue L1 measurements or evaluations (using a single panel), but more relaxed requirements will apply for such measurements/evaluations, when any of the following conditions for multi-rx operation become violated:

  - Partially new dual TCI configuration (e.g., one active TCI state remains the same),

  - Different QCL type D condition is not valid, etc.

- Proposal 9 (violated applicability conditions): RAN4 to discuss whether there are scenarios when a UE in multi-rx operation may need to complete L1 measurements or evaluations in multi-rx operation prior to switching to single-rx operation, e.g.:

  - UE becomes configured with CA operation or with some other operation suggesting single-rx operation.

# References

[1] RP-221753, WI description, Requirement for NR frequency range 2 (FR2) multi-Rx chain DL reception, Qualcomm Inc., June 2022.

[2] R4-2214344, WF on FR2 multi-Rx RRM, vivo, Aug. 2022.

[3] R4-2217246, WF on RRM impacts and general aspects for multi-Rx, vivo, Oct. 2022.

[4] R4-2217587, WF on L1 measurements, beam sweeping factors and simultaneous reception, Qualcomm, Oct. 2022.

[5] R4-2217248, WF on TCI state switching for multi-RX chain DL reception, Ericsson, Oct. 2022.

[6] R4-2220742, WF on NR FR2 multi-Rx chain DL reception – Part 1, Nov. 2022.

[7] R4-2220743, WF on NR FR2 multi-Rx chain DL reception – Part 2, Nov. 2022.

[8] R4-2220402, WF on NR FR2 multi-Rx chain DL reception – Part 3, Nov. 2022.

[9] R4-2303193, Ad-hoc minutes for NR FR2 multi-Rx chain DL reception RRM, vivo, March 2023.

[10] R4-2303302, WF on NR FR2 multi-Rx chain DL reception RRM requirements (part 1), vivo, March 2023.

[11] R4-2303317, WF on NR FR2 multi-Rx chain DL reception RRM requirements(part2), Qualcomm, March 2023.

[12] R4-2306318, WF on NR FR2 multi-Rx chain DL reception RRM requirements (part 1), vivo, Apr. 2023.

[13] R4- 2306319, WF on FR2_multiRx_part2, Ericsson, Apr. 2023.

[14] R4-2306392, Topic summary for [106bis-e][202] FR2_multiRx_part1, vivo, Apr. 2023.

[15] R4-2306234, Topic summary for [106bis-e][203] FR2_multiRx_part2, Ericsson, Apr. 2023.

[16] R4-2310046, WF on NR FR2 multi-Rx chain DL reception RRM requirements (part 1), vivo, May 2023.

[17] R4-2310047, WF on FR2_multiRx_part2, Ericsson, May 2023.

[18] R4-2309952, Topic summary for [106][209] FR2_multiRx _part1, vivo, May 2023.

[19] R4-2309953, Topic summary for [107][208] FR2_multiRx_part2, Ericsson, May 2023.

[20] R4-2314477, WF on NR FR2 multi-RX operation RRM requirements (part 1), vivo, Aug. 2023.

[21] R4-2314283, WF on FR2 multi-Rx chain DL reception (part2), Ericsson, Aug. 2023.