3GPP TSG-RAN WG4 Meeting #108-bis 	R4-2315910

Xiamen, China, Oct 9 – Oct 13, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Advanced Receivers - Simulation results

Agenda item:	5.18.2.2

Document for:	Information

Introduction

This document details Nokia’s current simulation results for alignment on the WID for advanced receivers for MU-MIMO scenario. Updated agreements were made in RAN4#107 [1] for facilitating companies to provide simulation results for Phase I.

Simulation assumptions for Phase I

Table 1 provides an overview of the already agreed simulation assumptions for Phase I taken from the WF [1]:

Table 1: Agreed simulation assumptions for Phase I

| Reference receiver for phase II simulation | R-ML |
| --- | --- |
| Rank allocation for the target and co-scheduled UE, with 1 co-scheduled UE | 2Tx2Rx UE: 1+1 2Tx4Rx UE: 1+1 4Tx4Rx UE: 2+2 |
| Rank allocation for the target and co-scheduled UEs, with 2 co-scheduled UEs | 2Tx 2Rx: 1+1 (Frequency multiplexed co-UEs) 4Tx4Rx: 2+1+1 |
| DMRS port configurations for the target and co-scheduled UEs | - Use different CDM groups for: - rank 2 (DMRS port 0, 1) + 2 (DMRS port 2, 3) - Use the same CDM group for - rank 1 (DMRS port 0) + 1 (DMR port 1) |
| Antenna configuration | For rank 1+1: 2T2R, 2T4R For rank 2+2: 4Tx 4Rx For rank2+1+1: 4Tx4Rx |
| Channel model | Use TDLC300-100 when the rank of the target UE is 1 Use TDLA30-10 when the rank of the target UE is 2 |
| Antenna correlation | Rank 1+1: ULA medium Rank 2+2: ULA Low Rank 2+1+1: ULA Low |
| Precoder selection target and co-scheduled UEs | Single panel Type 1 For rank 1+1: Random PMI selection For rank 2+2: Orthogonal PMI selection For rank2+1+1: Random PMI selection |
| QCL assumptions | Assume all scheduled DMRS ports have same QCL assumptions |
| Evaluation metric | Reuse the Rel-17 MMSE-IRC phase I evaluation assumptions captured in TR38.833 as a start point. |
| The number of co-scheduled UEs | Only consider 1 and 2 co-scheduled UE |
| DMRS sequence for the co-scheduled UE | Assume the scrambling ID for DMRS sequence is the same for the target UE the co-scheduled UE(s) |
| MCS for the target UE | MCS 4 and MCS 13 |
| PDSCH resource allocation for the target UE | Maximum transmission bandwidth configuration. Full OFDM symbol allocation. |

Simulation Results

Based on the information in Table 1, we provide the simulation results on the cases of interest for this meeting, listed in Table 2.

Table 2 - Simulation Overview – study cases for genie-aided knowledge of all parameters + blind detection

| Case | Co- UE number | Rank target UE | Rank Co- UE | MCS target UE | Modulation order co- UE | MIMO | Channel model | Precoder Co- UE | FDRA Co-UE | R-ML genie | R-ML BD1 | IRC Rel17 | Gain of R-ML |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 1 | 1 | 4 | QPSK | 2Tx 2Rx ULA medium | TDLC300-100 | random | Full CHBW allocation (52PRBs) | 5.5 | 5.5 | 7 | 1.5 |  |
| 2 |  |  |  | 13 |  |  |  |  |  | 15 | 15 | 23.2 | 8.2 |  |
| 3 |  |  |  |  | 16QAM |  |  |  |  | 18.8 | 18.9 | 23.2 | 4.3 |  |
| 4 |  |  |  |  | 64QAM |  |  |  |  | 21.1 | 21.4 | 23.2 | 1.8 |  |
| 5 |  | 2 | 2 |  | QPSK | 4Tx 4Rx ULA medium | TDLA30-10 | orthogonal |  | 10.7 | 10.7 | 13.3 | 2.6 |  |
| 6 |  |  |  |  | 16QAM |  |  |  |  | 12.8 | 12.8 | 13.3 | 0.5 |  |
| 7 |  |  |  |  | 64QAM |  |  |  |  | 13.5 | 13.5 | 13.3 | -0.2 |  |
| 8 |  | 1 | 1 | 13 | QPSK | 2Tx 4Rx ULA medium | TDLC300-100 | random |  | 12.7 | 12.7 | TBA | TBA |  |
| 9 |  |  |  |  | 16QAM |  |  |  |  | 16.9 | 16.9 | TBA | TBA |  |
| 10 |  |  |  |  | 64QAM |  |  |  |  | 19.5 | 19.5 | TBA | TBA |  |
| 11 |  |  |  |  | QPSK | 2Tx 4Rx ULA low | TDLC300-100 | random |  | 8 | 8 | 10.8 | 2.8 |  |
| 12 |  |  |  |  | 16QAM |  |  |  |  | 9.7 | 9.7 | 10.8 | 1.1 |  |
| 13 |  |  |  |  | 64QAM |  |  |  |  | 9.8 | 9.8 | 10.7 | 0.9 |  |
| 14 |  | 1 | 1 | 13 | QPSK | 2Tx 2Rx ULA medium | TDLC300-100 | random | Partial CHBW allocation (0~25 PRBs) | 12.8 | 12.8 | 15.3 | 2.5 |  |
| 15 |  | 2 | 2 | 13 | 16QAM | 4Tx 4Rx ULA Low | TDLA30-10 | orthogonal |  | 10.5 | 10.5 | 10.8 | 0.3 |  |
| 16 |  | 1 | 1 | 13 | 16QAM | 2Tx 4Rx ULA medium | TDLC300-100 | random |  | 12.3 | 12.4 | TBA | TBA |  |
| 17 |  | 1 | 1 | 13 | QPSK | 2Tx4Rx ULA low | TDLC300-100 | Random |  | 6.5 | 7 | 8.7 | 1.7 |  |
| 18 | 2 | 1 | 1 for each Co-UE | 13 | Co-UE1: QPSK Co-UE2: 16QAM | 2Tx 2Rx ULA medium | TDLC300-100 | random | Co-UE1: 0~25 PRBs Co-UE2: 38~51 PRBs | TBA | TBA | TBA | TBA |  |
| 19 |  | 2 | 1 for each Co-UE | 13 | Co-UE1: QPSK Co-UE2: 16QAM | 4Tx 4Rx ULA low | TDLC300-100 | random | Full CHBW allocation (52PRBs) | 12.6 | 12.6 | 17.8 | 5.2 |  |
| Note 1: R-ML BD refers to blind detection of DMRS ports and FDRA of co-scheduled UEs with PRG granularity |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Following observations can be made based on these results:

- Results show that R-ML receivers has large gain over Rel 17 MMSE-IRC receiver when co-UE is scheduled with QPSK and reasonable gain when co-UE is scheduled with 16QAM

R-ML receiver has marginal or no gain over Rel 17 MMSE-IRC receiver when co-UE is scheduled with 64 QAM.

Blind detection of the co-scheduled UE’s FDRA and DMRS ports with PRG granularity has insignificant impact on performance of R-ML receiver as compared to genie receiver.

Conclusion

This paper presents Nokia's current simulation results on the WID for advanced receivers for MU-MIMO scenario. It includes the configurations agreed in RAN4#108 [1].

In the paper, the following Observations were made:

- Results show that R-ML receivers has large gain over Rel 17 MMSE-IRC receiver when co-UE is scheduled with QPSK and reasonable gain when co-UE is scheduled with 16QAM

R-ML receiver has marginal or no gain over Rel 17 MMSE-IRC receiver when co-UE is scheduled with 64 QAM.

Blind detection of the co-scheduled UE’s FDRA and DMRS ports with PRG granularity has insignificant impact on performance of R-ML receiver as compared to genie receiver.

- References

- R4-2313993 - WF for Advanced receiver to cancel inter-user interference for MU-MIMO