3GPP TSG-RAN WG4 Meeting #108-bis	R4-2315914

Xiamen, China, Oct 09 – 13, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	On MultiRx Demodulation performance and CSI requirements - PDSCH

Agenda item:	5.7.4.2

Document for:	Discussion

Introduction

During the RAN4#108 discussions, good progress was made on the topic of MultiRx demodulation performance and CSI requirements for PDSCH and related issues.

In the following, we would provide Nokia’s viewpoint on the remaining open issues and introduce new proposals wherever needed.

Discussion

Channel model

In RAN4#108 the channel model was discussed [3]:

Issue 2-1-1: Channel model <way forward> - Proposals - Option 1: Only use TDLA30-75 for 100 MHz/120 kHz - Option 2: Keep TDLA30-300 as FFS.

Initial alignment can be done using the already agreed channel mode of TDLA30-75. Extending further with additional channel model of TDLA30-300 must be decided in this meeting to have enough time for companies to provide simulation results. We also see cases of high Doppler to be less important for MultiRx scenarios.

Simulation alignment can be done using the agreed channel model (TDLA30-75). High Doppler is less important for MultiRx scenarios.

Use TDLA30-75 for 100 MHz/120 kHz as baseline for requirement definition.

PTRS Port allocation for sDCI schemes

In RAN4#108 it was agreed to further evaluate PT-RS port allocation for sDCI [3]:

Issue 2-1-2: PTRS Port for sDCI schemes Agreement: - Further evaluate both cases: one PTRS port across TRPs and one PTRS port for each TRP - Companies are encouraged to evaluate the performance impact and decide suitable parameters for each case. - Test applicable rules can be considered based on UE capability.

It could be beneficial, especially for scenarios with joint demodulation (i.e. sDCI), to transmit PT-RS on each TRP as it cannot be assumed that each such TRP will be received with the same phase difference. It will be fine to introduce test applicability rules to reduce the test overhead.

It cannot be assumed that each TRP will be received with the same phase difference, hence there is likely a need for transmitting PT-RS on each such TRP. However, the UE support of two PT-RS ports for sDCI is optional.

We are fine with defining test applicability rules based on UE capability.

Introduce requirements for both cases if feasible and decide on the suitable parameters for each case based on simulation results.

Time/frequency offset between TRPs

In RAN4#108 it was agreed to use time and frequency offsets between the two TRP transmission points for the respective sDCI and mDCI transmission configurations as stated below Error! Reference source not found..

Issue 2-1-5: Time/frequency offset between TRPs Agreements: - sDCI SDM: (-0.0625us, 600Hz) and (0.25us, 0Hz) - mDCI non-overlapping: (-0.0625us, 600Hz) - mDCI fully-overlapping: (-0.0625us, 600Hz) and (0.25us, 0Hz) - Interested companies can provide simulation results with (0.25us, 600Hz) for mDCI fully-overlapping scenario.

At this point, we do not see a reason to change the agreement from RAN4#108; hence we can proceed with the agreed time and frequency offsets stated above for initial simulations. After aligning the initial simulation results, it can be further decided on the feasibility of defining requirements for all the above combinations.

We are fine to continue with the agreed configurations from RAN4#108 with respect to the time/frequency offset between TRPs. If companies contribute results with time/frequency offset of (0.25us, 600Hz) for mDCI fully-overlapping scenario, we are open to discuss including the case of (0.25us, 600Hz) for mDCI fully-overlapping scenario.

PDSCH Configurations for demodulation requirements

The PDSCH demodulation configurations to evaluate performance were agreed in RAN4#108 as stated below.

Issue 2-1-6: PDSCH Configurations for demodulation requirements Agreements: Evaluate PDSCH demodulation performance with the following configurations. - Multi-DCI with fully-overlapping PDSCH, 1 layer per TRP - Multi-DCI with fully-overlapping PDSCH, 2 layers per TRP - Multi-DCI with non-overlapping PDSCH, 2 layers per TRP - Single-DCI SDM with 1+1 - Single-DCI SDM with 2+2

We have provided simulation results for Multi-DCI fully-overlapping PDSCH using separate processing with the updated values of gamma = 0.0625. We have not noticed any significant differences compared to the previous gamma = 0.125.

We do not see the need to add additional configurations, hence RAN4 can continue to evaluate the PDSCH demodulation performance with the agreed configurations.

Test cases and simulation parameters

Tentative test cases and simulation parameters for initial evaluations of sDCI SDM were agreed in RAN4#108. Additionally, proposals were discussed regarding simulation parameters for mDCI fully overlapping and mDCI non-overlapping scenarios [3].

Issue 2-1-7: Test cases and simulation parameters for sDCI SDM Agreements: Tentative simulation assumptions for initial evaluations: Test parameters for sDCI SDM Parameter; Unit; Value. TRxP #1(Note 1); TRxP #2(Note 1). Transmit TRxP of SSB; TRxP #1. PDCCH configuration; TCI state; TCI State #1. CORESETPoolIndex; 0. Duplex mode; TDD. Active DL BWP index; 1. PDSCH configuration; Mapping type; Type A. k0; 0. Starting symbol (S); 1. Length (L); Specific to each Reference channel as defined in A.3.2.2. PRB bundling type; Static. PRB bundling size; 2. Resource allocation type; Type 1. RBG size; Config2. VRB-to-PRB mapping type; Non-interleaved. VRB-to-PRB mapping interleaver bundle size; N/A. PDSCH DMRS configuration; Antenna port indexes; 1000 for 1+1 1000-1001 for 2+2; 1002 for 1+1 1002-1003 for 2+2. TCI state; TCI State #1; TCI State #2. DMRS Type; Type 1. Number of additional DMRS; 1. Maximum number of OFDM symbols for DL front loaded DMRS; 1. Resource allocation; Full-overlapping. Precoding configuration; SP Type I, independent precoding generation is applied for both TRxPs, random per slot with PRB bundling granularity.. Note 1: PDSCH transmission is done from both TRxPs (PDSCH Layer 0 is transmitted from TRxP #1 and PDSCH layer 1 is transmitted from TRxP #2) Note 2: PDSCH transmission is done from both TRxPs (PDSCH Layers 0 and 1 are transmitted from TRxP #1 and PDSCH layers 2 and 3 are transmitted from TRxP #2). - Other options are not precluded. Issue 2-1-8: Test cases and simulation parameters for mDCI fully overlapping.<way forward> All tables related to this issue can be found in R4-2314258. - Proposals - Option 1: Consider PDSCH test cases and simulation parameter for mDCI fully overlapping as presented in Tables 4-3 and 4-4. - Option 2: Use the simulation parameters for sDCI SDM (changes from RAN4#107 proposals highlighted) as presented in Tables 3-2. - Option 3: Consider PDSCH test cases and simulation parameter for mDCI fully overlapping scheme as presented in Tables 2-4 and 2-5. Issue 2-1-9: Test cases and simulation parameters for mDCI non-overlapping <way forward> All tables related to this issue can be found in R4-2314258. - Proposals - Option 1: Consider PDSCH test cases and simulation parameter for mDCI non-overlapping scheme as presented in Tables 4-5 and 4-6. - Option 2: Define simulation parameters where the mDCI non-overlapping case is configured by splitting the available RBs equally between the two TRPs. - Option 3: Consider PDSCH test cases and simulation parameter for mDCI non-overlapping scheme as presented in Tables 2-6 and 2-7.

Simulation parameters for mDCI fully overlapping.

Based on the existing proposal for mDCI fully overlapping (option 2 from RAN4#108 issue 2-1-8) including the agreements in RAN4#108, we propose the following simulation parameters for the mDCI fully overlapping case.

Use the following simulation parameters for mDCI fully overlapping:

Table 1: Test parameters for mDCI full-overlapping

| Parameter |  |  | Unit | Value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | TRxP #1(Note 1) | TRxP #2(Note 1) |
| Transmit TRxP of SSB |  |  |  | TRxP #1 |  |
| PDCCH configuration |  | TCI state |  | TCI State #1 | TCI State #2 |
|  |  | CORESETPoolIndex |  | 0,1 |  |
| Duplex mode |  |  |  | TDD |  |
| Active DL BWP index |  |  |  | 1 |  |
| PDSCH configuration | Mapping type |  |  | Type A |  |
|  | k0 |  |  | 0 |  |
|  | Starting symbol (S) |  |  | 1 |  |
|  | Length (L) |  |  | Specific to each Reference channel as defined in A.3.2.2 |  |
|  | PRB bundling type |  |  | Static |  |
|  | PRB bundling size |  |  | 2 |  |
|  | Resource allocation type |  |  | Type 1 |  |
|  | RBG size |  |  | Config2 |  |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |  |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |  |
| PDSCH DMRS configuration | Antenna port indexes |  |  | {1000,1001} | {1002,1003} |
|  | TCI state |  |  | TCI State #1 | TCI State #2 |
|  | DMRS Type |  |  | Type 1 |  |
|  | Number of additional DMRS |  |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  | 1 |  |
| Resource allocation |  |  |  | Full-overlapping |  |
| Timing offset of the second TRxP from the first TRxP |  |  | us | {-0.0625, 0.25} |  |
| Frequency offset of the second TRxP from the first TRxP |  |  | Hz | {0, 600} |  |
| Precoding configuration |  |  |  | SP Type I, independent precoding generation is applied for both TRxPs, random per slot with PRB bundling granularity. |  |
| Note 1: PDSCH transmission is done from both TRxPs. Transmission from TRxP #1 uses CORESETPoolIndex 0 and transmission from TRxP #2 uses CORESETPoolIndex 1 |  |  |  |  |  |

Simulation parameters for mDCI non-overlapping

Based on the existing proposal for mDCI fully overlapping (see [3]) including the agreements in RAN4#108, we propose the following simulation parameters for the mDCI non-overlapping case.

Use the following simulation parameters for mDCI non-overlapping scenario:

Table 2: Test parameters for mDCI non-overlapping

| Parameter |  |  | Unit | Value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | TRxP #1(Note 1) | TRxP #2(Note 1) |
| Transmit TRxP of SSB |  |  |  | TRxP #1 |  |
| PDCCH configuration |  | TCI state |  | TCI State #1 | TCI State #2 |
|  |  | CORESETPoolIndex |  | 0,1 |  |
| Duplex mode |  |  |  | TDD |  |
| Active DL BWP index |  |  |  | 1 |  |
| PDSCH configuration | Mapping type |  |  | Type A |  |
|  | k0 |  |  | 0 |  |
|  | Starting symbol (S) |  |  | 1 |  |
|  | Length (L) |  |  | Specific to each Reference channel as defined in A.3.2.2 |  |
|  | PRB bundling type |  |  | Static |  |
|  | PRB bundling size |  |  | 2 |  |
|  | Resource allocation type |  |  | Type 1 |  |
|  | RBG size |  |  | Config2 |  |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |  |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |  |
| PDSCH DMRS configuration | Antenna port indexes |  |  | {1000,1001} | {1002,1003} |
|  | TCI state |  |  | TCI State #1 | TCI State #2 |
|  | DMRS Type |  |  | Type 1 |  |
|  | Number of additional DMRS |  |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  | 1 |  |
| Resource allocation |  |  |  | non-overlapping |  |
| Timing offset of the second TRxP from the first TRxP |  |  | us | {-0.0625, 0.25} |  |
| Frequency offset of the second TRxP from the first TRxP |  |  | Hz | {0, 600} |  |
| Precoding configuration |  |  |  | SP Type I, independent precoding generation is applied for both TRxPs, random per slot with PRB bundling granularity. |  |
| Note 1: PDSCH transmission is done from both TRxPs. Transmission from TRxP #1 uses CORESETPoolIndex 0 and transmission from TRxP #2 uses CORESETPoolIndex 1 |  |  |  |  |  |

Scaling factor for transmitted signal in mDCI fully-overlapping case

In RAN4#108, it was brought up if a scaling factor should be considered for transmitted signals for the mDCI fully-overlapping case.

Issue 2-1-10: Whether to consider a scaling factor for transmitted signal in mDCI fully-overlapping case <way forward> - Proposals - Option 1: Yes, SNR corresponds to SNR of TRxP #1 and TRxP #2 with scaling factor as 1/sqrt(2) for transmitted signal from each TRxP.

For mDCI fully overlapping, the same time and frequency resources are used, as a result the power from each TRP will be combined, hence scaling will be required.

In addition, the existing requirements for FR1 with sDCI SDM scaling of the SNR of TRxP#1 and TRxP#2 have been done with a factor of 1/srqt(2).

As for mDCI fully overlapping the same time and frequency resources are used, scaling of the power of the TRPs will be required.

Requirements definition for mDCI fully overlapping shall be done by defining the SNR as the SNR of TRxP #1 and TRxP #2 with scaling factor as 1/sqrt(2) for transmitted signal from each TRxP (Option 1).

MCS and layer selection for sDCI

Issue 2-1-11: MCS and layer selection for sDCI. <way forward> - Proposals - Option 1: Select {MCS17, ρ = -6dB, rank 1+1} and/or {MCS13, ρ = -12dB, rank 2+2} for sDCI SDM cases. - Other options are not precluded.

We are fine with selecting only one factor of crosstalk factor each for rank 1+1 and rank 2+2 scenarios for sDCI.

Selecting only one factor of crosstalk factor each for rank 1+1 and rank 2+2 scenarios for sDCI should be sufficient.

Use {MCS17, ρ = -6dB, rank 1+1} and {MCS13, ρ = -12dB, rank 2+2} as baseline assumption. Decide on the final MCS and crosstalk values to define requirements for rank 1+1 and rank 2+2 scenarios (sDCI) after simulation alignment.

MCS and layer selection for mDCI.

Issue 2-1-12: MCS and layer selection for mDCI. <way forward> - Proposals - Option 1: Select {MCS17, ρ = -6dB, rank 1+1} and/or {MCS13, ρ = -12dB, rank 2+2} for mDCI fully-overlapping cases - Other options are not precluded.

We are fine with selecting only one factor of crosstalk factor each for rank 1+1 and rank 2+2 scenarios for mDCI.

We are fine with selecting only one crosstalk factor each for rank 1+1 and rank 2+2 scenarios (mDCI). However, the most suitable crosstalk factor (ρ) and the MCS for each scenario must be based on the simulation results.

Use {MCS17, ρ = -6dB, rank 1+1} and {MCS13, ρ = -12dB, rank 2+2} as baseline assumption. Decide on the final MCS and crosstalk values to define requirements for rank 1+1 and rank 2+2 scenarios (mDCI) after simulation alignment.

Conclusion

This paper presents Nokia's view on the open issues with relation definition of PDSCH requirements for MultiRx Demodulation performance.

In the paper, the following Observations and Proposals were made:

Channel model

- Simulation alignment can be done using the agreed channel model (TDLA30-75). High doppler is less important for MultiRx scenarios.

- Use TDLA30-75 for 100 MHz/120 kHz as baseline for requirement definition.

PTRS Port allocation for sDCI schemes

It cannot be assumed that each TRP will be received with the same phase difference, hence there is likely a need for transmitting PT-RS on each TRP. However, the UE support of two PT-RS ports for sDCI is optional.

We are fine with defining test applicability rules based on UE capability.

Introduce requirements for both cases if feasible and decide on the suitable parameters for each case based on simulation results.

Time/frequency offset between TRPs

We are fine to continue with the agreed configurations from RAN4#108 with respect to the time/frequency offset between TRPs. If companies contribute results with time/frequency offset of (0.25us, 600Hz) for mDCI fully-overlapping scenario, we are open to discuss including the case of (0.25us, 600Hz) for mDCI fully-overlapping scenario.

Test cases and simulation parameters

Use the following simulation parameters for mDCI fully overlapping:

Table 3: Test parameters for mDCI full-overlapping

| Parameter |  |  | Unit | Value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | TRxP #1(Note 1) | TRxP #2(Note 1) |
| Transmit TRxP of SSB |  |  |  | TRxP #1 |  |
| PDCCH configuration |  | TCI state |  | TCI State #1 | TCI State #2 |
|  |  | CORESETPoolIndex |  | 0,1 |  |
| Duplex mode |  |  |  | TDD |  |
| Active DL BWP index |  |  |  | 1 |  |
| PDSCH configuration | Mapping type |  |  | Type A |  |
|  | k0 |  |  | 0 |  |
|  | Starting symbol (S) |  |  | 1 |  |
|  | Length (L) |  |  | Specific to each Reference channel as defined in A.3.2.2 |  |
|  | PRB bundling type |  |  | Static |  |
|  | PRB bundling size |  |  | 2 |  |
|  | Resource allocation type |  |  | Type 1 |  |
|  | RBG size |  |  | Config2 |  |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |  |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |  |
| PDSCH DMRS configuration | Antenna port indexes |  |  | {1000,1001} | {1002,1003} |
|  | TCI state |  |  | TCI State #1 | TCI State #2 |
|  | DMRS Type |  |  | Type 1 |  |
|  | Number of additional DMRS |  |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  | 1 |  |
| Resource allocation |  |  |  | Full-overlapping |  |
| Timing offset of the second TRxP from the first TRxP |  |  | us | {-0.0625, 0.25} |  |
| Frequency offset of the second TRxP from the first TRxP |  |  | Hz | {0, 600} |  |
| Precoding configuration |  |  |  | SP Type I, independent precoding generation is applied for both TRxPs, random per slot with PRB bundling granularity. |  |
| Note 1: PDSCH transmission is done from both TRxPs. Transmission from TRxP #1 uses CORESETPoolIndex 0 and transmission from TRxP #2 uses CORESETPoolIndex 1 |  |  |  |  |  |

Use the following simulation parameters for mDCI non-overlapping scenario:

Table 4: Test parameters for mDCI non-overlapping

| Parameter |  |  | Unit | Value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | TRxP #1(Note 1) | TRxP #2(Note 1) |
| Transmit TRxP of SSB |  |  |  | TRxP #1 |  |
| PDCCH configuration |  | TCI state |  | TCI State #1 | TCI State #2 |
|  |  | CORESETPoolIndex |  | 0,1 |  |
| Duplex mode |  |  |  | TDD |  |
| Active DL BWP index |  |  |  | 1 |  |
| PDSCH configuration | Mapping type |  |  | Type A |  |
|  | k0 |  |  | 0 |  |
|  | Starting symbol (S) |  |  | 1 |  |
|  | Length (L) |  |  | Specific to each Reference channel as defined in A.3.2.2 |  |
|  | PRB bundling type |  |  | Static |  |
|  | PRB bundling size |  |  | 2 |  |
|  | Resource allocation type |  |  | Type 1 |  |
|  | RBG size |  |  | Config2 |  |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |  |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |  |
| PDSCH DMRS configuration | Antenna port indexes |  |  | {1000,1001} | {1002,1003} |
|  | TCI state |  |  | TCI State #1 | TCI State #2 |
|  | DMRS Type |  |  | Type 1 |  |
|  | Number of additional DMRS |  |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  | 1 |  |
| Resource allocation |  |  |  | non-overlapping |  |
| Timing offset of the second TRxP from the first TRxP |  |  | us | {-0.0625, 0.25} |  |
| Frequency offset of the second TRxP from the first TRxP |  |  | Hz | {0, 600} |  |
| Precoding configuration |  |  |  | SP Type I, independent precoding generation is applied for both TRxPs, random per slot with PRB bundling granularity. |  |
| Note 1: PDSCH transmission is done from both TRxPs. Transmission from TRxP #1 uses CORESETPoolIndex 0 and transmission from TRxP #2 uses CORESETPoolIndex 1 |  |  |  |  |  |

Scaling factor for transmitted signal in mDCI fully-overlapping case

As for mDCI fully overlapping the same time and frequency resources are used, scaling of the power of the TRPs will be required.

Requirements definition for mDCI fully overlapping shall be done by defining the SNR as the SNR of TRxP #1 and TRxP #2 with scaling factor as 1/sqrt(2) for transmitted signal from each TRxP (Option 1).

MCS and layer selection for sDCI

Selecting only one factor of crosstalk factor each for rank 1+1 and rank 2+2 scenarios for sDCI should be sufficient.

Use {MCS17, ρ = -6dB, rank 1+1} and {MCS13, ρ = -12dB, rank 2+2} as baseline assumption. Decide on the final MCS and crosstalk values to define requirements for rank 1+1 and rank 2+2 scenarios (sDCI) after simulation alignment.

MCS and layer selection for mDCI

We are fine with selecting only one crosstalk factor each for rank 1+1 and rank 2+2 scenarios (mDCI). However, the most suitable crosstalk factor (ρ) and the MCS for each scenario must be based on the simulation results.

Use {MCS17, ρ = -6dB, rank 1+1} and {MCS13, ρ = -12dB, rank 2+2} as baseline assumption. Decide on the final MCS and crosstalk values to define requirements for rank 1+1 and rank 2+2 scenarios (mDCI) after simulation alignment.

- References

- 38.101-4 User Equipment (UE) radio transmission and reception; Part 4: Performance requirements

(Release 18)

- R4-2307852 - Discussion on PDSCH requirements of FR2 Multi-Rx DL Demod (Mediatek)

- R4-2313977 - WF for Rel-18 Multi-Rx performance requirements

- R4-2314258 - Topic summary for NR_FR2_multiRX_DL_Demod

- R4-2313848 - Ad-hoc minutes for Multi-RX in FR2 requirements

- R4-2315915 - On MultiRx Demodulation performance and CSI requirements - Simulation Results