3GPP TSG-RAN WG4 Meeting #108-bis 	R4-2315932

Xiamen, China, October 09 – 13, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on RRM performance requirements for further mobility enhancements

Agenda item:	5.24.3

Document for:	Discussion

Introduction

The performance part of the Further mobility enhancements WI is starting in the RAN4#108bis meeting. The RAN4 core part for the WI has covered the following objectives, for which core part requirements have been agreed or are under discussion:

- Selective activation

- L1/L2 based inter-cell mobility - LTM

- Improvement on SCell/SCG setup delay

- Enhanced CHO configurations

- CHO with target PSCell

- CHO with candidate PSCell

In this contribution, we give our initial views on the test case need for these objectives.

Test cases

While for some objectives of the Further mobility enhancements WI, significant progress and agreements have been made by the last RAN4 meeting, for some objectives many issues are still open. Considering the performance part, based on the current status of each objective, it is possible to evaluate the test case need at least on high level. In the following we discuss each of the objectives, the agreed core part requirements and the corresponding test cases that RAN4 should define.

Selective activation

For subsequent-CPAC, the subsequent PSCell change delay has been agreed to not include the RRC configuration time related to the initial conditional configuration being received by the UE, as for the subsequent CPAC UE keeps the conditional configurations.

To be able to test a subsequent PSCell change, the test case may consist of UE first performing a PSCell change and then following this a subsequent PSCell change, where the UE has to fulfil the delay requirement for the subsequent PSCell change.

For subsequent-CPAC testing of PSCell change delay, introduce a new test case to test the delay requirement for PSCell change after a PSCell change.

LTM

Many details of the core part requirements for LTM are still under discussion in RAN4, but based on the current status of the work in RAN4, the following overview of the LTM procedure and related requirements can be concluded:

LTM candidate cell measurements

LTM candidate cell measurements may be:

- L1 measurements performed according to LTM candidate cell configuration (intra/inter-frequency measurements)

- [L3 measurements performed by the UE and reported in L1 measurement report]

Considering the L1 [and L3] measurements performed for LTM candidate cells, most of the details are still open in the core part. However, it is clear that test cases are needed for the measurements for intra- and inter-frequency cases. Test case details depend on how RAN4 defines the core part requirements, so the discussion about the exact implementation of the test cases is pending further agreements on the core part.

Define LTM L1 measurement test cases for intra- and inter-frequency measurements. The details are pending core part conclusion.

Define test cases for L3 measurement reporting in L1 report for intra- and inter-frequency measurements. The details are pending core part conclusion.

Early TCI state activation

For early TCI state activation, we have proposed to introduce delay requirements in our core part contribution. However, we think there does not need to be a separate test case for early TCI state activation. Instead, TCI state activation can be tested together with LTM cell switch.

No need to define a separate test case for early TCI state activation if it is tested in LTM cell switch test case.

Early TA acquisition based on PDCCH order

A test case for early TA acquisition based on PDCCH order (delay and interruption) may be defined independently and/or together with the LTM cell switch test case.

RAN4 to consider defining a test case for early TA acquisition based on PDCCH order separately from LTM cell switch and/or together with LTM cell switch. Details of the test case(s) are pending core part agreements.

LTM cell switch

LTM cell switch delay requirements are still under discussion in the core part of the WI. However, LTM cell switch may be considered to depend at least on:

- Whether the target cell is a current serving cell or a neighbouring cell

- Frequency range of source and target cell

- Whether the target TCI state is on the active TCI state list

- Whether early TA acquisition has been performed

Because of the large variation on the supported scenarios, RAN4 needs to discuss which of the scenarios need to be tested to have good coverage of testing the core requirements. RAN4 should also discuss how to take early TCI state activation and TA acquisition into account in the test cases for LTM cell switch delay. One way is to include all these procedures in the same test case at least for some test cases.

RAN4 to consider defining test cases for:

For LTM cell switch, RAN4 to consider defining test cases for

- RACH-based cell switch with TCI activation+indication at cell switch command

- RACH-based cell switch with early TCI state activation

- RACH-less cell switch with TCI state activation+indication at cell switch command

- RACH-less cell switch with early TCI state activation

RAN4 to define LTM cell switch test cases for FR1 and FR2 and target cell being a neighbour cell or a current serving cell.

Improvement on SCell/SCG setup delay

The requirements for the SCell/SCG setup delay are still under discussion in RAN4. However, it has been agreed that two solutions are discussed in parallel: solution with and without enhanced measurements.

When the UE reports measurements from idle/inactive mode, independent of whether the measurements are purely based on existing measurements or whether the UE has performed additional/enhanced measurements during/after connection setup, the reported measurement results need to fulfil the measurement accuracy requirements. Therefore, test cases for verifying measurement accuracy of the idle/inactive mode measurements reported by the UE are needed.

There is no need to verify how the UE measures or reports during idle/inactive mode, because in this WI, this is up to UE implementation. What needs to be verified is that when the UE indicates that it has idle mode measurements available (with or without enhanced measurements) for the configured carrier, the measurement accuracy of those reported measurements is fulfilled.

Reporting details during the test case depend on how RAN2 defines the reporting (e.g. whether the report can be sent on predefined time instances or whether the reporting is dynamic and/or based on response/request as in Rel-16.). Depending on the RAN2 defined reporting, there may be different test cases for e.g. the case when

- UE has idle/inactive mod measurements available at RRC setup complete (verifying the case without enhanced measurements)

- UE continues enhanced measurements during RRC setup and connected mode, and has measurements ready for reporting during connected mode (verifying the case with enhanced measurements)

For eEMR, define test cases for verifying measurement accuracy of UE reported idle/inactive mode measurements for the cases with and without enhanced measurements. The details of the measurements and reporting are depending on further RAN4 and RAN2 agreements.

CHO enhancements

For conditional handover enhancements, core requirements have been defined for:

- CHO with target PSCell

- CHO with candidate PSCell

Existing test cases for CHO (without PSCell) are defined in A.6.3.3 and A.7.3.3. Existing test cases for HO with PSCell are defined in A.4.3.3 (EN-DC FR1), A.5.3.3 (EN-DC FR2) and A.7.3.1.8 (NR-DC FR1+FR2), where the tests evaluate PRACH transmission timing and time to send the first CSI report after target PSCell is added. Similar test cases are needed for the enhanced CHO configurations with target/candidate PSCell.

For CHO enhancements, test cases for testing the delay and interruption for PCell and PSCell should be defined for:
- CHO with target PSCell (FR1+FR1 and FR1+FR2)
- CHO with candidate PSCell (FR1+FR1 and FR1+FR2)

Regarding CHO with candidate PSCell, two cases exist:

UE is not provided with CHO-only configuration: UE continues the evaluation of PCell and PSCell conditions until both are met.

UE is provided with CHO-only configuration: UE continues the evaluation of PCell and PSCell conditions until PCell condition is met. If PSCell condition is met at this point as well, UE proceeds with CHO+CPC, otherwise the UE proceeds with CHO without CPC.

From test case point of view, when the UE proceeds with CHO only when it is provided with CHO-only configuration and CHO condition is fulfilled without CPC condition being fulfilled, the existing test case for CHO applies. For the case when the UE proceeds with CHO+CPC, RAN4 should define a test case using the CHO test case as the baseline, but including both PCell and PSCell in the tests.

No need to define test case for CHO with candidate PSCell for the case when CPC condition is not met and the UE proceeds with CHO-only.

Conclusion

The following observations and proposals were made in this contribution:

Proposal 1: For subsequent-CPAC testing of PSCell change delay, introduce a new test case to test the delay requirement for PSCell change after a PSCell change.

Proposal 2: Define LTM L1 measurement test cases for intra- and inter-frequency measurements. The details are pending core part conclusion.

Proposal 3: Define test cases for L3 measurement reporting in L1 report for intra- and inter-frequency measurements. The details are pending core part conclusion.

Observation 1: No need to define a separate test case for early TCI state activation if it is tested in LTM cell switch test case.

Proposal 4: RAN4 to consider defining a test case for early TA acquisition based on PDCCH order separately from LTM cell switch and/or together with LTM cell switch. Details of the test case(s) are pending core part agreements.

Proposal 5: For LTM cell switch, RAN4 to consider defining test cases for

Proposal 6: RAN4 to define LTM cell switch test cases for FR1 and FR2 and target cell being a neighbour cell or a current serving cell.

Proposal 7: For eEMR, define test cases for verifying measurement accuracy of UE reported idle/inactive mode measurements for the cases with and without enhanced measurements. The details of the measurements and reporting are depending on further RAN4 and RAN2 agreements.

Proposal 8: For CHO enhancements, test cases for testing the delay and interruption for PCell and PSCell should be defined for: - CHO with target PSCell (FR1+FR1 and FR1+FR2) - CHO with candidate PSCell (FR1+FR1 and FR1+FR2)

Observation 2: No need to define test case for CHO with candidate PSCell for the case when CPC condition is not met and the UE proceeds with CHO-only.

- References

- R4-2310046, WF on NR FR2 Multi-Rx chain DL reception RRM requirements (part 1), Ericsson

- R4-2310047, WF on NR FR2 Multi-Rx chain DL reception RRM requirements (part 2), Ericsson