3GPP TSG-RAN WG4 # 108-bis	R4-2315940

Xiamen, China, October 9 – October 13, 2023

Agenda item:	5.11.4

Source:	MediaTek Inc.

Title:	Discussion on UE demodulation requirements for intra-band non-collocated NR-CA deployment scenario

Document for:	Discussion

# Introduction

In RAN4#108, the Way Forward document for demodulation requirements was approved [1]. In RAN4#107 it was agreed that RAN4 should discuss the need for demodulation requirements only for Type-2 UEs and preclude Type-3a/3b and Type-4a/4b UE from the UE demodulation performance requirements discussion. Furthermore, for Type-2 UE requirements are agreed to be defined in NR-CA scenario only. We provide discussion with proposals for the Rel-18 work in this contribution.

# Discussion

In this chapter we discuss agreements of the previous meeting and remaining open items.

In the previous meeting it was agreed to define new Type 2 UE demodulation requirement with NR-CA scenario only. In new requirement we must assume the power difference of 25dB and received time difference of 33us under the assumption that requirements introduced under static channel.

MCS pair and SNR test point pair

Way forward: Interested companies are encouraged to bring the simulation results in RANM4#108bis to decide the MCS index pair and corresponding SNR test point pair according to the agreed test setup and test metrics.

Note the required SNR difference between the carriers with lower power and higher power should not exceed 25dB.

|  | Tx antenna and rank | MCS table | Test metric |
| --- | --- | --- | --- |
| Carrier with lower power | Option 1: 2Tx, Rank 2 Option 2: 1Tx, Rank 1 | Option 1: MCS index 1 Option 2: MCS index 2 | 70% of the maximum throughput |
| Carrier with higher power | Option 1: 2Tx, Rank 2 Option 2: 1Tx, Rank 1 | Option 1: MCS index 1 Option 2: MCS index 2 | 70% of the maximum throughput |

In the previous meetings it was agreed for NR-CA requirement to measure both PCell and SCell at the same time using 70% of the maximum throughput as the test metric. The channel model agreed to be static channel. For Type 2 UEs both carriers are using 2 receiver antennas as defined for Type-2 UE. For simplicity we propose to use also 2 transmit antennas for both lower and higher power carriers.

Proposal #1: We propose to use 2 transmit antennas for both lower and higher power carriers.

We have simulated static channel 2x2 simulations with all fixed MCS options in MCS tables 1 and 2 with Rank 1 and Rank 2. These results are available in separate simulation results Tdoc [2]. This gives us 4 independent configuration options for PCell and SCell.

- Rank 1 with MCS table 1 (64-QAM table)

- Rank 1 with MCS table 2 (256-QAM table)

- Rank 2 with MCS table 1 (64-QAM table)

- Rank 2 with MCS table 2 (256-QAM table)

We propose to configure PCell with Rank 2 transmission using MCS table 2 and SCell with Rank 1 transmission using MCS table 1 to maximize SNR difference options. With these configurations we can find MCS that would give approximately 25dB SNR difference. If other companies would prefer aligned MCS table configuration between carriers, we are also fine to use MCS table 2 configuration also for SCell.

Proposal #2: We propose to use Rank 2 with MCS table 2 configuration for PCell.

Proposal #3: We propose to use Rank 1 with MCS table 1 configuration for SCell.

For Type 2 UE testing it was agreed to assume 25dB power difference between carriers. Therefore, we need to find suitable MCS pairs to achieve approximately 25dB difference. In Table 2 in separate simulation results Tdoc [2] possible MCS pairs are listed. We suggest excluding the lowest and highest MCS values. In Table 1 below there are MCS pairs listed and the lowest and highest MCS values are excluded. Also, SNR difference of these MCS pairs is less than 25dB.

| SCell MCS index | Rank 1 MCS Table 1 | PCell MCS index | Rank 2 MCS Table 2 | SNR difference |
| --- | --- | --- | --- | --- |
| MCS 0 | -6.5dB | MCS 19 | 18.9dB | 25.4dB |
| MCS 1 | -5.7dB | MCS 19 | 18.9dB | 24.6dB |
| MCS 2 | -5.0dB | MCS 20 | 20.1dB | 25.1dB |
| MCS 3 | -3.9dB | MCS 22 | 21.5dB | 25.4dB |
| MCS 4 | -3.0dB | MCS 22 | 21.5dB | 24.5dB |
| MCS 5 | -2.0dB | MCS 23 | 22.9dB | 24.9dB |
| MCS 6 | -1.1dB | MCS 24 | 23.7dB | 24.8dB |
| MCS 7 | -0.2dB | MCS 25 | 24.7dB | 24.9dB |
| MCS 8 | 0.7dB | MCS 26 | 25.7dB | 25.0dB |
| MCS 9 | 1.5dB | MCS 27 | 27.0dB | 25.5dB |
| MCS 10 | 2.1dB | MCS 27 | 27.0dB | 24.9dB |

Table 1: Fixed MCS mapping between Rank 1 MCS Table 1 to Rank 2 MCS Table 2.

From this table we get 4 proposals for test case configuration

- PCell: Rank 2, MCS Table 2, MCS 22 / SCell: Rank 1, MCS Table 1, MCS 4

- PCell: Rank 2, MCS Table 2, MCS 23 / SCell: Rank 1, MCS Table 1, MCS 5

- PCell: Rank 2, MCS Table 2, MCS 24 / SCell: Rank 1, MCS Table 1, MCS 6

- PCell: Rank 2, MCS Table 2, MCS 25 / SCell: Rank 1, MCS Table 1, MCS 7

Corresponding SNR requirements per carrier are also listed in Table 1 without impairments.

Proposal #4: We propose 4 following test case configurations as possible test candidates

- PCell: Rank 2, MCS Table 2, MCS 22 / SCell: Rank 1, MCS Table 1, MCS 4

- PCell: Rank 2, MCS Table 2, MCS 23 / SCell: Rank 1, MCS Table 1, MCS 5

- PCell: Rank 2, MCS Table 2, MCS 24 / SCell: Rank 1, MCS Table 1, MCS 6

- PCell: Rank 2, MCS Table 2, MCS 25 / SCell: Rank 1, MCS Table 1, MCS 7

# Conclusion

In this paper we provided the view on the UE demodulation requirements for intra-band non-collocated NR-CA deployment scenario. The following observations and proposals are made:

Proposal #1: We propose to use 2 transmit antennas for both lower and higher power carriers.

Proposal #2: We propose to use Rank 2 with MCS table 2 configuration for PCell.

Proposal #3: We propose to use Rank 1 with MCS table 1 configuration for SCell.

Proposal #4: We propose 4 following test case configurations as possible test candidates

- PCell: Rank 2, MCS Table 2, MCS 22 / SCell: Rank 1, MCS Table 1, MCS 4

- PCell: Rank 2, MCS Table 2, MCS 23 / SCell: Rank 1, MCS Table 1, MCS 5

- PCell: Rank 2, MCS Table 2, MCS 24 / SCell: Rank 1, MCS Table 1, MCS 6

- PCell: Rank 2, MCS Table 2, MCS 25 / SCell: Rank 1, MCS Table 1, MCS 7

# Reference

- R4-2313876, “WF for NonCol_intraB_ENDC_NR_CA_Demod”, Ericsson

- R4-2315941, “Simulation results of UE demodulation requirements for intra-band non-collocated NR-CA deployment scenario”, MediaTek Inc.