3GPP TSG-RAN WG4 Meeting # 108-bis	R4-2315992

# Xiamen, China, Oct.9th – Oct.13th, 2023

Title: 	Discussions on UE performance for Non-collocated CA

Source: 	Huawei, HiSilicon

Agenda item:	5.11.4

Document for:	Discussion

Background

A WF [1] on UE performance for Non-collocated CA was agreed in last meeting, many open issues are left for discussions. This contribution provides our views on these open issues.

Discussions

RAN4 agreed to reuse the test setup of existing PDSCH CA requirements, the remain open issues are antenna/rank configuration and MCS pairs. The candidate options are shown as below:

|  | Tx antenna and rank | MCS table | Test metric |
| --- | --- | --- | --- |
| Carrier with lower power | Option 1: 2Tx, Rank 2 Option 2: 1Tx, Rank 1 | Option 1: MCS index 1 Option 2: MCS index 2 | 70% of the maximum throughput |
| Carrier with higher power | Option 1: 2Tx, Rank 2 Option 2: 1Tx, Rank 1 | Option 1: MCS index 1 Option 2: MCS index 2 | 70% of the maximum throughput |

The simulation results for MCS table 1 and MCS table 2 with each candidate antenna/rank configuration are captured in Table 2-1 and Table 2-2. It is noted that we set EVM to 6% for 64QAM and 3% for 256QAM.

Table 2-1: Target SNR (SNR@70% of max TP) for each MCS in MCS table 1

| MCS | SNR@70% of maxTP |  |
| --- | --- | --- |
|  | 1T2R, Rank 1 | 2T2R, Rank 2 |
| 0 | -8.3 | -5.3 |
| 1 | -7.3 | -4.2 |
| 2 | -6.2 | -3.4 |
| 3 | -5.1 | -1.9 |
| 4 | -4.4 | -1.5 |
| 5 | -3.4 | -0.5 |
| 6 | -2.5 | 0.5 |
| 7 | -1.6 | 1.4 |
| 8 | -0.6 | 2.4 |
| 9 | 0.4 | 3.4 |
| 24 | 13.4 | 16.8 |
| 25 | 14.5 | 18.2 |
| 26 | 16.1 | 19.4 |
| 27 | 17.5 | 21.3 |
| 28 | 18.8 | 24.8 |

Table 2-2: Target SNR (SNR@70% of max TP) for each MCS in MCS table 2

| MCS | SNR@70% of maxTP |  |
| --- | --- | --- |
|  | 1T2R, Rank 1 | 2T2R, Rank 2 |
| 0 | -8.3 | -5.3 |
| 1 | -6.2 | -3.4 |
| 2 | -4.4 | -1.5 |
| 3 | -2.5 | 0.5 |
| 4 | -0.6 | 2.4 |
| 5 | 1.5 | 4.4 |
| 6 | 2.5 | 5.4 |
| 7 | 3.4 | 6.4 |
| 8 | 4.4 | 7.4 |
| 9 | 5.9 | 8.4 |
| 23 | 18.5 | 22.1 |
| 24 | 19.4 | 22.6 |
| 25 | 21.2 | 24.4 |
| 26 | 22.4 | 25.4 |
| 27 | 23.2 | 26.4 |

It can be observed that MCS Table 2 has large SNR dynamic range, so we prefer to select MCS Table 2. As for Rank configuration, we prefer 2T2R/Rank2 which has higher SNR to avoid radio link failure for some low MCSs. Regarding the MCS pair, we prefer to choose MCS 2 for CC with low power and MCS 24 for CC with higher power according to our simulation results.

Proposal 1: Select following configuration:

2T2R, Rank2

MCS Table 2

MCS2 for CC with low power and MCS24 for CC with high power

For applicability rules, we propose to select the CA combination with largest aggregated bandwidth combination among all supported CA combination with 2CCs.

- Proposal 2: Define following test applicability rules:

Select the CA combination with largest aggregated bandwidth combination among all supported CA combination with 2CCs.

Conclusion

In this paper we provide our overviews on UE performance for Non-collocated CA. The proposals are:

Proposal 1: Select following configuration:

2T2R, Rank2

MCS Table 2

MCS2 for CC with low power and MCS24 for CC with high power

- Proposal 2: Define following test applicability rules:

Select the CA combination with largest aggregated bandwidth combination among all supported CA combination with 2CCs.

Reference

[1]    R4-2313876  WF for NonCol_intraB_ENDC_NR_CA_Demod. Ericsson