3GPP TSG-RAN WG4 Meeting #108-bis	R4-2315997

# Xiamen, 9th - 13th Oct, 2023

Title: 	Discussion on BS demodulation requirements for further coverage enhancements

Source: 	Huawei, HiSilicon

Agenda item:	5.27.2

Document for:	Discussion

Background

During RAN#96 meeting, Revised WID [1] on Further NR coverage enhancements was approved. In this contribution, we share our views about BS demodulation requirements for further coverage enhancements.

Discussion

Test scope

In Rel-18, the uplink coverage is further enhanced including PRACH coverage enhancements, power domain enhancements and dynamic switching between DFT-S-OFDM and CP-OFDM. From our understanding, the demodulation performance requirements need to be defined for only PRACH coverage enhancements since the other two parts are not related to UE processing on demodulation.

Only consider PRACH coverage enhancements for demodulation performance requirements.

In the SI part, as per TS 38.830, only PRACH format B4 is identified as the potential bottleneck channel for both FR1 and FR2 28GHz. Considering the practicability, also to reduce the test effort and simulation effort, it is reasonable to only define PRACH requirements for PRACH format B4.

Only define PRACH requirements for PRACH format B4.

For the sequence length, 1151 and 571 are mainly used in unlicensed spectrum and are not considered for evaluation during the SI. Also considering that the high-speed scenarios have no coverage issue, we propose to only define PRACH requirements for normal mode and sequence length 139.

Only define PRACH requirements for normal mode and sequence length 139.

In the SI part, as per TS 38.830, only FR2 Urban 28GHz is evaluated. Considering the practicability, also to reduce the test effort and simulation effort, it is reasonable to consider 28GHz as higher priority for PRACH requirements.

Consider 28GHz as higher priority for PRACH requirements.

For the number of PRACH transmissions, RAN1 support {2, 4, 8} for the configurable value set. Considering the test coverage, it is reasonable to only define PRACH requirements for 2 PRACH transmissions that is the minimum requirements.

Only define PRACH requirements for 2 PRACH transmissions.

For the SCS, we propose to not consider 60kHz in FR2 for PRACH requirements, since that there is no any deployment in the real network.

Do not consider 60kHz in FR2 for PRACH requirements.

Simulation assumption

Based on above discussion, we propose to use the following simulation assumption for PRACH.

Use the following simulation assumption for PRACH.

| Parameter | Value for FR1 | Value for FR2 |
| --- | --- | --- |
| Number of PRACH transmissions | 2 | 2 |
| SCS | 15kHz, 30kHz | 120kHz |
| Antenna | 1x2, 1x4, 1x8 | 1x2 |
| Chanel | AWGN, TDLC300-100 Low with 400Hz FO | AWGN, TDLA30-300 Low with 4000Hz FO |
| PRACH format | B4 | B4 |
| Sequence length | 139 | 139 |
| Others | Same as legacy cases | Same as legacy cases |

Proposals

In this contribution, we discuss on BS demodulation requirements for further coverage enhancements. Our observations and proposals are:

- Only consider PRACH coverage enhancements for demodulation performance requirements.

Only define PRACH requirements for PRACH format B4.

Only define PRACH requirements for normal mode and sequence length 139.

Consider 28GHz as higher priority for PRACH requirements.

Only define PRACH requirements for 2 PRACH transmissions.

Do not consider 60kHz in FR2 for PRACH requirements.

Use the following simulation assumption for PRACH.

| Parameter | Value for FR1 | Value for FR2 |
| --- | --- | --- |
| Number of PRACH transmissions | 2 | 2 |
| SCS | 15kHz, 30kHz | 120kHz |
| Antenna | 1x2, 1x4, 1x8 | 1x2 |
| Chanel | AWGN, TDLC300-100 Low with 400Hz FO | AWGN, TDLA30-300 Low with 4000Hz FO |
| PRACH format | B4 | B4 |
| Sequence length | 139 | 139 |
| Others | Same as legacy cases | Same as legacy cases |

Reference

RP-221858, Revised WID on Further NR coverage enhancements, RAN#96, China Telecom