3GPP TSG-RAN WG4 Meeting #108-bis	R4-2315998

# Xiamen, 9th - 13th Oct, 2023

Title: 	Discussion on UE demodulation requirements for NR NTN enhancements

Source: 	Huawei, HiSilicon

Agenda item:	5.26.8.2

Document for:	Discussion

Background

During RAN#101 meeting, Revised WID [1] on NR NTN (Non-Terrestrial Networks) enhancements was approved. In this contribution, we share our views about UE demodulation requirements for NR NTN enhancements.

Discussion

Test scope

As per the latest WID, the UE demodulation performance requirements can be defined for above 10 GHz bands. For other parts, the detailed objectives still need to be further discussed in the RAN plenary meeting. From our understanding, the other parts are not related to the UE demodulation, so we propose to only consider to define UE demodulation performance requirements for above 10 GHz bands for VSAT UE.

The performance part objectives are applicable to the NR-NTN deployment in above 10 GHz bands objective. - Specify RRM performance requirements and test cases [RAN4] - Specify UE demodulation and CSI reporting requirements [RAN4] - Specify satellite access node demodulation requirements [RAN4] - Specify satellite access node conformance tests [RAN4] The RAN4 performance part for the remaining objectives needs to be further discussed once the scope is stabilized.

Only consider to define UE demodulation performance requirements for above 10 GHz bands for VSAT UE.

Above 10GHz for VSAT UE

PDSCH

In Rel-17, the demodulation performance requirements for NTN is defined only in FR1 NTN bands. For VSAT UE working on above 10GHz, RAN4 define new bands and new UE power class. It is reasonable to define new PDSCH demodulation performance requirements for FR2-NTN. In addition, all Rel-17 NTN features including 32 HARQ process and HARQ disabled can be considered for FR2-NTN.

Define new PDSCH demodulation performance requirements for FR2-NTN.

All Rel-17 NTN features including 32 HARQ process and HARQ disabled can be considered for FR2-NTN.

PDCCH

In Rel-17, PDCCH requirements are not defined in TS 38.101-5, considering that the NTN UE is assumed as handheld UE and has both capability on both NTN network an TN network, i.e. NTN UE must pass the TN PDCCH cases specified in TS 38.101-4. However, in Rel-18, VSAT UEs working on above 10GHz do not need to support TN network. To ensure the test coverage, the PDCCH demodulation performance requirements should be defined for FR2-NTN.

Define PDCCH demodulation performance requirements for FR2-NTN.

CSI reporting

In Rel-17, there is no CSI reporting requirements defined for NTN, considering that CSI may be outdated due to the large UE-Satellite RTT delay (order of tens of milliseconds delay for LEO-600 and LEO-1200 respectively at the same time order of hundreds of milliseconds delay for GEO). For the similar reason, we propose to not consider any CSI reporting requirements for FR2-NTN.

Do not consider any CSI reporting requirements for FR2-NTN.

  - Test setup

Channel model

Similar as FR1-NTN, both NLOS and LOS conditions are considered for FR2-NTN, i.e. NTN-TDLA and NTN-TDLC channel model. For the delay spread, we propose to consider 10ns as a proper value based on Rel-15/16 evaluation results during NTN SI stage. For the Doppler, 2000Hz can be selected that is the 0.1ppm of the maximum DL carrier frequency for the agreed bands for FR2-NTN.

For FR2-NTN, use NTN-TDLA and NTN-TDLC channel model.

For FR2-NTN, consider 10ns delay spread and 2000Hz Doppler.

Channel bandwidth & SCS

As per RF agreements, three new NTN satellite bands n512/n511/n510 is introduced with {50, 100, 200}MHz bandwidth configuration for 60kHz SCS and {50, 100, 200, 400}MHz bandwidth configuration for 120kHz SCS. Considering that 200MHz for 120kHz SCS is selected in RAN4 RF co-existence study, we propose to select the same value for demodulation performance requirements for FR2-NTN.

Select 200MHz for 120kHz SCS for demodulation performance requirements for FR2-NTN.

MCS & Rank for PDSCH

Similar as FR1-NTN, rank 1 with MCS4 (QPSK, 0.30) and MCS13 (16QAM, 0.48) can be considered for FR2-NTN as the starting point.

Rank 1 with MCS4 (QPSK, 0.30) and MCS13 (16QAM, 0.48) can be considered for FR2-NTN as the starting point.

PDCCH

For PDCCH, the existing legacy cases with some down selection can be considered with necessary modification, such as changing the channel model, and then run new simulation.

For PDCCH, the existing legacy cases with some down selection can be considered with necessary modification, such as changing the channel model, and then run new simulation.

Proposals

In this contribution, we discuss on UE demodulation requirements for NR NTN enhancements. Our observations and proposals are:

- Only consider to define UE demodulation performance requirements for above 10 GHz bands for VSAT UE.

Define new PDSCH demodulation performance requirements for FR2-NTN.

All Rel-17 NTN features including 32 HARQ process and HARQ disabled can be considered for FR2-NTN.

Define PDCCH demodulation performance requirements for FR2-NTN.

Do not consider any CSI reporting requirements for FR2-NTN.

For FR2-NTN, use NTN-TDLA and NTN-TDLC channel model.

For FR2-NTN, consider 10ns delay spread and 2000Hz Doppler.

Select 200MHz for 120kHz SCS for demodulation performance requirements for FR2-NTN.

Rank 1 with MCS4 (QPSK, 0.30) and MCS13 (16QAM, 0.48) can be considered for FR2-NTN as the starting point.

For PDCCH, the existing legacy cases with some down selection can be considered with necessary modification, such as changing the channel model, and then run new simulation.

Reference

RP-232669, Revised WID: NR NTN (Non-Terrestrial Networks) enhancements, RAN#101, Thales