3GPP TSG-RAN WG4 Meeting #108-bis	R4-2316004

# Xiamen, 9th - 13th Oct, 2023

Title: 	Discussion on demodulation performance requirements for NR network-controlled repeaters

Source: 	Huawei, HiSilicon

Agenda item:	5.28.6

Document for:	Discussion

Background

During RAN4#108 meeting, WF [1] on NCR_MT demodulation requirements was approved. In this contribution, we share our views about demodulation performance requirements for NR network-controlled repeaters.

Discussion

PDCCH

PDCCH requirements

- Agreements: - FFS whether new PDCCH requirements needed or not for DCI format 5_0/2_8 with large payload size.

RAN4 does not define performance requirements for all new DCI formats for both LTE and NR, but just focus on the UE baseband processing. From demodulation point of view, there is no any UE algorithm change for the new DCI format 5_0/2_8 of access link beam change indication. Therefore, to save the test effort and the simulation effort, we propose to reuse same DCI type from IAB-MT and legacy UE requirements for NCR PDCCH requirements, i.e. do not consider the new PDCCH requirements for DCI format 5_0/2_8 with large payload size.

Reuse same DCI type from IAB-MT and legacy UE requirements for NCR PDCCH requirements, i.e. do not consider the new PDCCH requirements for DCI format 5_0/2_8 with large payload size.

CSI

PMI requirements

- Agreements: - FFS for whether to define PMI requirements.

For the NCR-MT CSI reporting requirements, the only issue is that whether to consider PMI reporting cases. From our understanding, the NCR has the similar deployment scenario as the IAB. The PMI reporting cases for the IAB-MT are already defined in the specification TS 38.174, so we prefer to define PMI reporting cases for the NCR-MT with the test parameters reused from IAB. Since that there is applicability rule defined for the IAB that “Testing of performance requirements for RI and PMI reporting is optional”, we propose to reuse the same applicability for the NCR PMI reporting case to allow different BS implementation.

Define PMI reporting cases for the NCR-MT with the test parameters reused from IAB.

Define applicability rule that “Testing of performance requirements for PMI reporting is optional”.

Proposals

In this contribution, we discuss on demodulation performance requirements for NR network-controlled repeaters. Our observations and proposals are:

- Reuse same DCI type from IAB-MT and legacy UE requirements for NCR PDCCH requirements, i.e. do not consider the new PDCCH requirements for DCI format 5_0/2_8 with large payload size.

Define PMI reporting cases for the NCR-MT with the test parameters reused from IAB.

Define applicability rule that “Testing of performance requirements for PMI reporting is optional”.

Reference

R4-2313923, WF on NCR_MT demodulation requirements, RAN4#108, ZTE Corporation